3GPP TSG-SA3 Meeting #120	S3- 250941

Athens, Greece, 17-23 February 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | DRAFT CR | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Key Derivation for QUIC/TLS in ATSSS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | DUMMY |  |  |  |  |  | Date: |  |  | 2025-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In case of PSK based QUIC/TLS between UE and UPF, a key derivation and key delivery mechanism is required. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a sub-clause in Annex AA and details on how key derivation and key delivery mechanism is performed to PSK based QUIC/TLS for ATSSS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PSK based QUIC/TLS between UE and UPF is left unspecified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | X.X (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START OF CHANGES ***

Annex AA (normative): 
Security aspects of the Access Traffic Steering, Switching and Splitting (ATSSS)

# AA.1	General

This Annex specifies the Security aspects of the Access Traffic Steering, Switching and Splitting (ATSSS). The ATSSS feature is described in 3GPP TS 23.501 [2].

# AA.2	Server authentication for MPQUIC in ATSSS

When multipath QUIC (MPQUIC) [115, 116, 117] steering functionality is used for ATSSS, RFC 9001 [x2] mandates the use of TLS to secure QUIC.

NOTE: TLS server authentication based on X.509 certificates can be selected for authentication of the MPQUIC Proxy in the UPF, which is to be performed by the MPQUIC Client in the UE. Self-signed end entity certificates and hard-coded pre-shared keys need to be avoided. Establishing the trust anchor in the UE is left to deployment. If no trust anchor is provisioned in the UE, the entire certificate chain including the root CA certificate needs to be provided to the UE by the UPF during TLS handshake, and the root CA certificate is implicitly trusted.

# AA.X	PSK based QUIC/TLS between UE and UPF in ATSSS

The AMF derives the KUPF from the KAMF in a similar way as described in clause “A.9	KgNB, KWAGF, KTNGF, KTWIF and KN3IWF derivation function”. The AMF uses Access type distinguisher set to 3GPP access and sets the NAS COUNT to zero to avoid the derivation of the same key as the KgNB.

The KUPF is sent from the AMF via the SMF  to the UPF together with the SUPI.

Editor’s Note: details of the message from AMF to UPF are FFS

*** END OF CHANGES ***