3GPP TSG-SA3 Meeting #120	S3- 250942

Athens, Greece, 17-23 February 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | DRAFT CR | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Protecting XRM Media related information in UDP Option |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | DUMMY |  |  |  |  |  | Date: |  |  | 2025-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | For Extended Reality and Media service (XRM), data streams are end-to-end (e2e) encrypted when sent from an external Application Server (AS) to the UE via the UPF. To be able to select the correct QoS setting, the UPF needs access to Media related information sent together with the e2e encrypted data. The Media related information is encrypted between the AS and the UPF. The details on how this encryption is performed and what security context to use is not specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a clause and details on how MRI is encrypted and integrity protected betewen AS and UPF in case the MRI is transported in UDP Option. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Protection of Media related information in UDP Option is left unspecified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | X (New) , X.X (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START OF CHANGES ***

# X		Protection of XRM Media related information

## X.Y 	Protection of Media related information when using UDP Options

The transport of the MRI in UDP Options is specified in TS 23.501 [2], where the inner UDP datagram contains the DL UDP media packets, i.e. QUIC packet between AS proxy and UE and the outer UDP datagram between AS proxy and UPF contains UDP-Option carrying encrypted media related information.

In order to protect the MRI in the UDP Option, the API of the QUIC layer is reused and the content (i.e., media related information) to be carried by means of UDP Options is provided to the QUIC Layer as input for encryption and respective decryption with the selected cipher suite, including the respective security keys, in the same way as for the Connect-UDP packets. The content (i.e., MRI) ciphertext is transported in the UDP Option alongside the UDP datagram encapsulating the associated Connect-UDP packet. Further details are implementation specific.

*** END OF CHANGES ***