3GPP TSG-SA3 Meeting #120	S3-250951

# Athens, Greece, 16-21 February 2025

Source:	Nokia

Title:	NRF cert update notification and logging

Document for:	Approval

Agenda item:	4.11

Spec:	3GPP TS 33.501

Version:	19.1.0

Work Item:	TEI19, SBA

Comments

pCR against DRAFT CR (was S3-245334) for resolution of

Editor’s Note: It is FFS if a NF Service Producer should be able to subscribe for updates of NRF cert/pub keys, request status of a stored cert/pub key.

In addition, error cases and logging need to be addressed to address:

Editor’s notes:  Causes for validation failure can be several: stored certificate not valid anymore (expiry), stored certificate revoked, new cert / private key for signing used by a legitimate NRF, compromised NRF issuing a token. It is FFS how to handle validation failure

Editor’s Note: Multiple certificates of a NRF instance may be configured to a NF Service Producer, hence input parameters in new service operations are not sufficient if one wants to fetch a particular certificate. FFS if additional parameters are needed.

.

Proposed Changes

*** BEGIN OF CHANGES 4 ***

### 14.3.1	General

The following table illustrates the security related services for OAuth 2.0 that NRF provides. OAuth 2.0 based authorization is described in clause 13.4.1.

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnrf_AccessToken | Get | Request/Response | AMF, SMF, PCF, NEF, NSSF, SMSF, AUSF |
|  | RetrieveCertificate | Request/Response | AMF, SMF, PCF, NEF, NSSF, SMSF, AUSF, NRF |
|  | GetCert | Request/Response | AMF, SMF, PCF, NEF, NSSF, SMSF, AUSF, NRF |
|  | SubscribeCertStatus | Request/Response | AMF, SMF, PCF, NEF, NSSF, SMSF, AUSF, NRF |
|  | NotifyCertStatus | Request/Response | AMF, SMF, PCF, NEF, NSSF, SMSF, AUSF, NRF |
|  | ErrorReport | Request/Response | AMF, SMF, PCF, NEF, NSSF, SMSF, AUSF, NRF |
|  | Logging | Request/Response | AMF, SMF, PCF, NEF, NSSF, SMSF, AUSF, NRF |

The complete list of NRF services is defined in TS 23.501 [2], clause 7.2.6, and further refined in TS 23.502 [8], clause 5.2.7.

Editor’s Note: It is FFS, if AccessToken service is the correct service for adding a management operation.

Editor’s Note: It is FFS, which fields the OAuth 2.0 token should include, e.g. URIs or key ids, to allow for identification or location of the public key/cert of the access token issuer. To be considered, certificates may be managed by a different entity than the NRF.

Editor’s Note: it is FFS, whether a NF Service Producer has only one certificate/public key available, which can result in a failure and delay for fetching a new cert/pub key or if NF Service Producers should have at least two certificates of potential access token issuers. Also to support different NF services, the NRF may have several certificates (eg slice specific).

*** END OF CHANGES 4 ***

*** BEGIN OF CHANGES 6 ***

### 14.3.X	Nnrf_AccessToken_GetCert Service Operation

Service Operation name: Nnrf_AccessToken_GetCert.

Description: NF Service producer requests NRF to provide public key certificate to validate the signature of the Access Token.

Inputs, Required: the NF Instance Id of the token issuer NRF, the key identifier (Key ID from the Header Parameter of the JOSE header of the access token).

Inputs, Optional: UUID of NF Service Producer.

Outputs, Required: X.509 public key certificate(s) or certificate(s) chain used for signing the access token.

Outputs, Optional: None.

*** END OF CHANGES 6 ***

*** BEGIN OF CHANGES 7 ***

### 14.3.X	Nnrf_AccessToken_SubscribeCertStatus Service Operation

Service Operation name: Nnrf_AccessToken_SubscribeCertStatus.

Description: NF Service producer subscribes notification of certificate status change from NRF .

Inputs, Required: the NF Instance Id of the token issuer NRF, the key identifier ,  callback URI.

Inputs, Optional: UUID of the NF Service producer issuing the request,.

Outputs, Required: list of updated certificates, or issuer id and key id, which allows to use GetCert or RetrieveCert operation.

Outputs, Optional: None.

*** END OF CHANGES 7 ***

*** BEGIN OF CHANGES 8 ***

### 14.3.X	Nnrf_AccessToken_NotifyCertStatus Service Operation

Service Operation name: Nnrf_AccessToken_NotifyCertStatus.

Description: NRF sends notification of certificate status change to the NF producer subscribed to change of the certificate status.

Inputs, Required: original and new status of the certificate(s) with certificate id, e.g. issue id + kid.

Inputs, Optional: None.

Outputs, Required:  None.

Outputs, Optional: None.

*** END OF CHANGES 8 ***

*** BEGIN OF CHANGES 9 ***

### 14.3.X	Nnrf_AccessToken_ErrorReport Service Operation

Service Operation name: Nnrf_AccessToken_ErrorReport.

Description: NF Service Consumer requests NRF to take notice of the cause of error, when it is not possible to validate the signature of the Access Token.

Inputs, Required: the NF Instance Id of the token issuer NRF, the key identifier, UUID of the NF Service producer issuing the request, callback URI, error cause.

Inputs, Optional: None.

Outputs, Required: None.

Outputs, Optional: None.

*** END OF CHANGES 9 ***

*** BEGIN OF CHANGES 10 ***

### 14.3.X	Nnrf_AccessToken_Logging Service Operation

Service Operation name: Nnrf_AccessToken_Logging.

Description: NF Service Consumer povides NRF with any type of logging information related to the service operations.

Inputs, Required: the NF Instance Id of the token issuer NRF, the key identifier, UUID of the NF Service producer issuing the request, callback URI, logging information.

Inputs, Optional: None.

Outputs, Required: None.

Outputs, Optional: None.

*** END OF CHANGES 10 ***

*** BEGIN OF CHANGES 11 ***

### 14.3.X	Nnrf_AccessToken CertStatus Service Operation

Service Operation name: Nnrf_AccessToken_CertStatus.

Description: NF Service Producer requests NRF to provide status of a specific certificate or public key cached.

Inputs, Required: the NF Instance Id of the token issuer NRF, (list of) key identifier(s).

Inputs, Optional: UUID of NF Service Producer.

Outputs, Required: status information (active, revoked, expiry) for one or several certificates.

Outputs, Optional: None.

*** END OF CHANGES 11 ***