3GPP TSG-SA3 Meeting #120	S3-250984

# Athens, Greece, 17 - 21 February 2025

Source:	ZTE Corporation, Nokia, Huawei

Title:	Pseudo-CR on Hosting Party Authentication

Document for:	Approval

Agenda item:	4.16

Spec:	3GPP TS 33.545

Version:	0.0.0

Work Item:	5G_Femto_Sec

Comments

Update hosting party authentication enhancements of NR Femto according to TR conclusions.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[Y]	3GPP TS 33.320: "Security of Home Node B (HNB) / Home evolved Node B (HeNB)".

* * * Second Change * * * *

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

NR Femto hosting party: NR Femto hosting party is same as “H(e)NB Hosting Party” defined in TS 22.220 [X]. NR Femto hosting party also plays the role of CAG owner. In some cases, NR Femto hosting party can be a representative of NR Femto hosting party having contractual relationship with the operator.

* * * Third Change * * * *

5.2	NR Femto Hosting Party Authentication

5.2.1	General

Device Authentication of the NR Femto by the SeGW can be followed with a hosting party authentication. The security features for hosting party authentication specified in TS 33.320 [x] clause 5.3 and clause 7.3 can be derived with the following changes:

-	Replace H(e)NB with NR Femto;

-	Replace HSS with UDM;

-	EAP-AKA'-based method can be used for NR Femto hosting party authentication exchange.

5FMTF can act as the proxy for NR Femto authentication. 5FMTF can be co-located with SeGW or an independent function deployed in the 5GC.

NR Femto hosting party can plays the role of CAG owner to operate (add/delete/modify) on the allowed CAG list of the users after authenticated and authorized by the 5GC. The security aspects of NEF specified in clause 12 of TS 33.501[y] can be reused when NR Femto hosting party operating on the allowed list in the UDM. Refer to clause 5.5 of the present document for relevant access control mechanisms.

5.2.2	Combined device and hosting Party authentication procedure

5FMTF can act as the proxy for NR Femto authentication. 5FMTF can be co-located with SeGW or an independent function deployed in the 5GC. The procedure between the NR Femto, SeGW and EAP authentication server is shown in Figure z.

Figure z: NR Femto hosting party authentication using EAP-AKA' authentication method

1. The NR Femto establishes connection between the NR Femto and the SeGW.

2. The SeGW sends an EAP Identity/Request to the NR Femto.

3. The 5G NR Femto sends an EAP Response/Identity message. The NR Femto uses the SUCI in NAI format as its identity.

4. The EAP Response/Identity message is routed over SeGW towards the 5FMTF based on the realm part of the SUCI.

5. The 5FMTF sends the message Nausf_UEAuthentication_Authenticate Request with SUCI, Access Network Identity and 5FMT indicator towards the AUSF. 5FMT_indicator is used to indicate to the AUSF that the authentication request is for NR Femto purposes. The 5FMTF sets the Access Network Identity to "5G: 5FMT".

6. Based on the 5FMT_indicator, the AUSF (acting as the EAP authentication server) sends a Nudm_UEAuthentication_Get Request to the UDM, including SUCI and the Access Network Identity and 5FMT indicator.

7. Upon reception of the Nudm_UEAuthentication_Get Request, the UDM invokes SIDF. SIDF de-conceal SUCI to gain SUPI before UDM can process the request. Based on the 5FMT indicator and if 5FMT is allowed based on the subscription data, the UDM/ARPF selects the EAP-AKA' authentication method and generate an authentication vector using the Access Network Identity as the KDF input parameter. The UDM includes the EAP-AKA’ authentication vector (RAND, AUTN, XRES, CK´ and IK´) and may include SUPI to AUSF in a Nudm_UEAuthentication_Get Response message.

8. The AUSF stores XRES for future verification. The AUSF sends the EAP-Request/AKA'-Challenge message to the 5FMTF in a Nausf_UEAuthentication_Authenticate Response message.

9. The 5FMTF sends the EAP-Request/AKA'-Challenge message to the SeGW.

10. The SeGW forwards the EAP-Request/AKA'-Challenge message to the NR Femto.

11. At receipt of the RAND and AUTN in the EAP-Request/AKA'-Challenge message, the NR Femto derives CK' and IK' using the Access Network Identity as the KDF input parameter. The NR Femto may derive MSK from CK’ and IK’. When the NR Femto is performing 5FMT authentication, the KAUSF does not be generated by the 5G NR Femto.

12. The NR Femto sends the EAP-Response/AKA'-Challenge message to the SeGW.

13. The SeGW forwards the EAP-Response/AKA'-Challenge message to the 5FMTF.

14. The 5FMTF send the Nausf_UEAuthentication_Authenticate Request with EAP-Response/AKA'-Challenge message to AUSF.

15. The AUSF verifies if the received response RES matches the stored and expected response XRES. If the AUSF has successfully verified, it continues as follows to step 16, otherwise it returns an error to the 5FMTF. The AUSF derives the required MSK key from CK’ and IK’, based on the 5FMT indicator received in step 5. The AUSF does not generate the KAUSF.

16. The AUSF sends Nausf_UEAuthentication_Authenticate Response message with EAP-Success and MSK key to 5FMTF. The AUSF may optionally provide the SUPI to 5FMTF. The AUSF/UDM does not perform the linking increased home control to subsequent procedures (as stated in present document clause 6.1.4).

17. The 5FMTF sends the EAP-success and MSK to SeGW. The EAP-Success message is forwarded from SeGW to the NR Femto.

18. Upon receiving the EAP-Success message, the NR Femto derives the MSK, if it has not derived the MSK earlier. The NR Femto uses the first 256-bit of MSK as PMK to establish a secure connection with the SeGW.

* * * End of Changes * * * *