3GPP TSG-SA3 Meeting #120	S3-251001

Athens, Greece, 17 - 21 February 2025	merger of S3-250491, S3-250603, S3-250770, S3-250860, S3-250926

# Source:	Ericsson, China Telecom, Samsung, Xiaomi, Nokia

# Title:	Further conclusion for key issue #2

# Document for:	Approval

Agenda Item:	5.19

# 1	Decision/action requested

It is proposed to add the following update to the TR 33.700-22.

# 2	References

[1]	3GPP TR 33.700-22: "Study on security aspects of CAPIF Phase3".

[2]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs ".

# 3	Rationale

It is proposed to resolve the following EN in clause 7.3.2 of TR 33.700-22 [1], by considering the SA6 procedure specified in clause 8.25.3.9 in TS 23.222 [2].

# 4	Detailed proposal

It is proposed to approve the following change to TR 33.700-22 [1].

***START OF THE CHANGE***

## 7.2	Conclusion for KI #2: CAPIF interconnection security

### 7.2.0 	General

It is assumed that the API invoker onboards to CCF-A, which is referred as onboarded CCF.

It is assumed that the API invoker is onboarded to CCF-A and the target AEF is registered to a different CCF-B.

### 7.2.1 	Conclusion for CAPIF 6/6e security

It is concluded that for CAPIF-6 and CAPIF-6e reference points, same security mechanisms specified in clauses 6.6 and 6.10 of TS 33.122 [4] for CAPIF-3/4/5 and CAPIF-3e/4e/5e reference points will be used, respectively.

### 7.2.2 	Conclusion for security method negotiation

For security method negotiation procedure (as per requirement 2), clause 6.3.1.2 in TS 33.122 [4] will be used as baseline with the necessary enhancements (if any).

The details of how security method selection is done for the CAPIF 2/2e reference point based on the capabilities of the API Invoker and the AEF capabilities (that belongs to CCF-B) are up to normative work.

### 7.2.3	Conclusion for API invoker authentication and authorization mechanism

Authorization between API invoker (on-boarded to CCF-A) and the AEF (registered to CCF-B) is specified in TS 23.222 [2].

For mutual authentication between API invoker (on-boarded to CCF-A) and the AEF (registered to CCF-B), the procedures as defined in clause 6.5.2 of TS 33.122 [4] can be re-used with the following enhancement:

-	When using TLS-PSK or PKI:

-	On receiving the request from the AEF, CCF-B requests the security information (AEFPSK/root CA) from CCF-A (over CAPIF-6/6e reference point).

-	The AEF learns the authorization information from CCF-B.  Whether CCF-B can obtain access control policy from CCF-A to be decided during normative work

-	When using TLS with OAuth token:

-	On receiving the request from the AEF, CCF-B requests the security information (e.g., root CA) from CCF-A (over CAPIF-6/6e reference point).

-	CCF-A can send the access token request to CCF-B.

-	CCF-B can provide an access token to the API invoker via CCF-A as specified in clause 6.5.2.3 in TS 33.122 [4].

-	The AEF verifies the access token as described in 6.5.2.3 in TS 33.122 [4].

-	Whether CCF-A can issue an access token and whether CCF-B needs to verify CCF-A is authorized for the authorization decision are to be determined during normative work.

-	For the case of TLS-PSK or PKI, and TLS with OAuth token methods listed above, the specific details of how the CCF-B requests the security information of the API Invoker from the right CCF-A, i.e. where API invoker is onboarded, are up to normative work.

-	For the case of TLS with OAuth token methods, how CCF-A determines to request token from CCF-B is up to normative work.

-	Further details of the procedure are to be determined during normative work.

***END OF THE CHANGE***