3GPP TSG-SA3 Meeting #120	draft_S3-251035-r9

# Athens, Greece, 17 - 21 February 2025

Source:	Samsung, Interdigital, Philips International B.V.

Title:	pCR on Security for S&F operation on split MME architecture

Document for:	Approval

Agenda item:	4.19

Spec:	3GPP TS 33.401

Version:	18.2.0

Work Item:	5GSAT_Ph3_SEC

Comments

This pCR proposes to security procedure for S&F operation on split MME architecture.

* * * First Change * * * *

Annex X (normative):
Security for Store and Forward Satellite operation

# X.1	General

Editor’s Note: This clause provides general description for Annex X.

# X.2	Security aspects of Split MME architecture

## X.2.2	Protection against DoS attacks

### X.2.2.1 General

For the uplink control plane data (e.g. NAS message) the 3GPP systems on board the satellite are not able to verify message and data integrity. It is hard to detect whether the data is sent from a genuine UE or an attacker. All the uplink data needs to be stored during the feeder or ISL links’ period of unavailability. Hence, spoofed data can easily exhaust the storage capacity with the attack over the air. This issue is amplified by the inability to upgrade hardware (e.g., radios, memory) on board of satellite. As an example, in the case of the incomplete AKA procedure or control-plane data from unauthorized UE, the storage resource of the 3GPP system on board the satellite can be exhausted, resulting in the Denial of Service (DoS) attack on the satellite.

For the downlink data (e.g., NAS message before authentication), the UE cannot verify its integrity. It is hard to detect whether the data is sent from a legitimate network or an external attacker.

The security procedures specified in clause 6 and clause 7 do not address the DoS attacks described above. The procedure and possible implementation options to protect against DoS attacks before a security context is established are described in clause X.2.2.2.

### X.2.2.2	Procedure

Editor’s Note: This clause describes the procedures to protect against DoS attacks before a security context is established.

* * * Next Change * * * *

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AES	Advanced Encryption Standard

AK	Anonymity Key

AKA	Authentication and Key Agreement

AMF	Authentication Management Field

AN	Access Network

AS	Access Stratum

AUTN	Authentication token

AV	Authentication Vector

ASME	Access Security Management Entity

Cell-ID	Cell Identity as used in TS 36.331 [21]

CHO	Conditional Handover

CK	Cipher Key

CKSN	Cipher Key Sequence Number

C-RNTI	Cell RNTI as used in TS 36.331 [21]

CRL	Certificate Revocation List

DeNB	Donor eNB

DoS	Denial of Service

DSCP	Differentiated Services Code Point

EARFCN-DL	E-UTRA Absolute Radio Frequency Channel Number-Down Link

ECM	EPS Connection Management

EDT	Early Data Transmission

EEA	EPS Encryption Algorithm

EIA	EPS Integrity Algorithm

eKSI	Key Set Identifier in E-UTRAN

EMM	EPS Mobility Management

eNB	Evolved Node-B

EPC	Evolved Packet Core

EPS	Evolved Packet System

EPS-AV	EPS authentication vector

E-UTRAN	Evolved UTRAN

gNB	Next Generation Node-B

GERAN	GSM EDGE Radio Access Network

GUTI	Globally Unique Temporary Identity

HE	Home Environment

HFN	Hyper Frame Number

HO	Hand Over

HSS	Home Subscriber Server

IAB	Integrated Access and Backhaul

IK	Integrity Key

IKE	Internet Key Exchange

IMEI	International Mobile Station Equipment Identity

IMEISV	International Mobile Station Equipment Identity and Software Version number

IMSI	International Mobile Subscriber Identity

IOPS	Isolated E-UTRAN Operation for Public Safety

IRAT	Inter-Radio Access Technology

ISR	Idle Mode Signaling Reduction

KDF	Key Derivation Function

KSI	Key Set Identifier

LWIP	LTE WLAN RAN Level Integration using IPSec

LSB	Least Significant Bit

LSM	Limited Service Mode

LWA	LTE-WLAN Aggregation

MAC-I	Message Authentication Code for Integrity (terminology of TS36.323 [12])

MACT	Message Authentication Code T used in AES CMAC calculation

MeNB	Master eNB

ME	Mobile Equipment

MME	Mobility Management Entity

MME-RN	MME serving the RN

MS	Mobile Station

MSC	Mobile Switching Center

MSIN	Mobile Station Identification Number

NAS	Non Access Stratum

NAS-MAC	Message Authentication Code for NAS for Integrity (called MAC in TS24.301 [9])

NASDVM	Non Access Stratum - Data via MME

NCC	Next hop Chaining Counter

NH	Next Hop

OCSP	Online Certificate Status Protocol

OTA	Over-The-Air (update of UICCs)

PCI	Physical Cell Identity as used in TS 36.331 [21]

PDCP	Packet Data Convergence Protocol

PLMN	Public Land Mobile Network

PRNG	Pseudo Random Number Generator

PSK	Pre-shared Key

P-TMSI	Packet- Temporary Mobile Subscriber Identity

RAND	RANDom number

RAU	Routing Area Update

RLOS	Restricted Local Operator Services

RN	Relay Node

RRC	Radio Resource Control

SCG	Secondary Cell Group

SEG	Security Gateway

SGSN	Serving GPRS Support Node

SIM	Subscriber Identity Module

SMC	Security Mode Command

SeNB	Secondary eNB

SgNB	Secondary gNB

SN	Serving Network

SN id	Serving Network identity

SQN	Sequence Number

SRB	Source Route Bridge

SRVCC	Single Radio Voice Call Continuity

S-TMSI	S-Temporary Mobile Subscriber Identity

S&F	Store and Forward

TAI	Tracking Area Identity

TAU	Tracking Area Update

UE	User Equipment

UEA	UMTS Encryption Algorithm

UIA	UMTS Integrity Algorithm

UICC	Universal Integrated Circuit Card

UMTS	Universal Mobile Telecommunication System

UP	User Plane

USIM	Universal Subscriber Identity Module

UTRAN	Universal Terrestrial Radio Access Network

WT	WLAN Termination as used in TS 36.300 [30]

XRES	Expected Response

* * * End of Changes * * * *