3GPP TSG-SA3 Meeting #122	S3-251895

# Fukuoka, Japan, 19 - 23 May 2025

Source:	Sony, OPPO

Title:	KI#3, New Merged Solution

Document for:	Approval

Agenda item:	5.9

Spec:	3GPP TR 33.713

Version:	0.8.0

Work Item:	FS_AIOT_Sec

Comments

Based on the discussion during the CC between SA3#121 and SA3#122 the following new merged solution is proposed.

Proposed Changes

* * * First Change * * * *

## 6.xx	Solution #XX: Temp ID protection for Ambient IoT device identifier

### 6.xx.1	Introduction

This solution addresses key issue #3 on privacy by protecting AIoT device identifiers. The identified use cases are:

-	Group Inventory, this can be stand-alone event or an event that precedes dedicated Command procedure as specified in TS 23.369.

-	Individual Inventory only, this allows the network to check for one specific device

-	Individual Inventory + Command, this allows the network to page one device and deliver a command to that device.

The details how the AIoT Device identities are protected are documented in the procedures. The following high level principles applies:

- For Group Inventory, no protection of the Target Device Information in the Inventory Request is added. It is assumed that e.g. the filtering information in the Target Device Information provides enough protection. Each AIoT Device that responds to the Inventory Request message will protect its identity by using a Temp ID derived using a freshness parameter in the inventory request. The network can verify the Temp ID and MAC in the inventory response.

- For Individual Inventory only, the network adds protection to the Target Device Information (i.e., the Permanent ID) in the Inventory Request. The protection is based on the use of Temp ID in the Inventory Request message. The AIoT Device responds to the Inventory Request message with the Temp ID and MAC that the network can verify.

- The Device can match the Target Device Information or Temp ID directly without performing any calculation using the freshness parameter in the Inventory Request. Allowing the AIoT Device to spend minimum energy when parsing inventory requests not intended for the AIoT Device.

- Group Inventory with ALL, shall not be specified or allowed as such group inventory procedure will force the ADM to derive Temp IDs for all AIoT Device entries in the data base. The size of the group shall be considered.

- When the Inventory and Command procedure is used, the Inventory procedure part of the Inventory and Command procedure is simplified since there is no need for the device to generate or update its own Temp ID. The network generated Temp ID is sent securely to the AIoT device in the Command procedure.

- Temp ID synchronization and re-synchronization are described in separate clause 6.xx.2.4.

### 6.xx.2	Solution details

#### 6.xx.2.1 Group Inventory Procedure

This procedure details how the Device Identity is protected when the network performs a group inventory.

Figure 6.xx.2.1-1: Procedure for Device ID protection during group inventory

Procedure:

1-3. Details of step 1-3 are specified in TS 23.369. No changes are needed to protect the AIoT Device Permanent ID.

NOTE 1:	The Target Device Information provided by the AF addresses a group of devices.

4. The AIOTF generate a NONCE_nw. In this step the AIOTF may exchange the Target Device Information and NONCE_nw with the ADM, but this can also be done in step 11.

5-6. Details of step 1-3 are specified in TS 23.369. No changes are needed to protect the AIoT Device Permanent ID.

7. The AIOTF sends a Inventory Request to the NG-RAN. The Inventory requests include Target device Information, Correlation ID as specified TS 23.369 and the NONCE_nw. The Correlation ID is used to link all responses related this Inventory Request.

NOTE 2:	The AIOTF does not add any protection to the Target device Information.

8. Details of this step is specified in TS 23.369. No changes are needed to protect the AIoT Device ID.

9a. The NG-RAN Reader sends the paging message which includes the Target device Information (e.g., filter) and the NONCE_nw in the Container as specified by RAN2. If an AIoT Device matches its identify to the Target device Information  (e.g., filter) it uses the NONCE_nw to derive a Temp ID, NONCE_d using an HMAC.

Figure 6.xx.2.1-2: Derivation of Temp ID and NONCE_d

9b-c. The NG-RAN and AIoT device performs the Ambient IoT random access procedure as specified by RAN2.

9d. The AIoT Device sends the D2R NAS message which includes the Temp ID, NONCE_d and MAC.

Figure 6.xx.2.1-3: Derivation of MAC

NOTE 3:	The NONCE_d and MAC is not needed for the AIoT Device Permanent ID protection, but used by the network to Authenticate the response message and protect against replay attacks

10. The NG-RAN forwards the NAS message to the AIOTF

11a. The AIOTF request the ADM to check the Temp ID(s) in the responses received linked to the Correlation ID. The Request includes the Target Device Information, Nonce_nw and for each device response the Temp ID, Nonce_d, MAC. The ADM generates the Temp IDs of all AIoT Devices in the group of devices that matches to the Target device Information, in the same way as the AIoT Device did in step 9. The ADM authenticates the device response by verifying the MAC.

11b. The ADM reports the AIoT Device Permanent IDs to the AIOTF for each of the device that was verified during the check.

12-14. Details of step 12-14 are specified in TS 23.369. No changes are needed to protect the AIoT Device Permanent ID

#### 6.xx.2.2 Individual Inventory Procedure

This procedure details how the Device Identity is protected when the network performs an Individual inventory.

Figure 6.xx.2.2-1: Procedure for Device ID protection during Individual inventory

Procedure:

0. The AIoT Device has a pre-configured Random number a.k.a. a SEED stored in the device. The Network (the ADM) also have the SEED stored together with the device Permanent ID, pre-shared key in the AIoT Device profile. Both the AIoT Device and ADM use the SEED to derive the Initial Temp ID_0 and store it locally.

Figure 6.xx.2.2-2: Derivation of initial Temp ID

Or the AIoT Device has a pre-configured initial Temp ID, the Network (the ADM) also have the initial Temp ID stored together with the device Permanent ID, pre-shared key in the AIoT Device profile.

1-3. Details of step 1-3 are specified in TS 23.369. No changes are needed to protect the AIoT Device Permanent ID.

NOTE 1:	The Target Device Information provided by the AF addresses a specific device i.e., its AIoT Device Permanent ID.

4. The AIOTF generate a NONCE_nw. The AIOTF fetches the current Temp ID_n from the AIoT Device Profile in the ADM or from local device context in the AIOTF.

5-6. Details of step 1-3 are specified in TS 23.369. No changes are needed to protect the AIoT Device Permanent ID.

7. The AIOTF sends an Inventory Request to the NG-RAN. The Inventory requests include Temp ID, Correlation ID as specified TS 23.369 and the NONCE_nw. The Correlation ID is used to link response related this Inventory Request.

8. Details of this step is specified in TS 23.369. No changes are needed to protect the AIoT Device Permanent ID.

9. The NG-RAN Reader sends the paging message which includes the Temp ID_n and the NONCE_nw in the Container as specified by RAN2. If a AIoT Device matches its identity to the Temp ID_n the NG-RAN and AIoT device performs the Ambient IoT random access procedure as specified by RAN2. The AIoT Device generates a new Temp_ID_n+1 and NONCE_d and stores the new Temp ID_n+1.

Figure 6.xx.2.2-3: Derivation of Temp ID_n+1 and NONCE_d

The AIoT Device sends the Inventory Response NAS message which includes the Temp ID_n, NONCE_d and MAC.

Figure 6.xx.2.2-4: Derivation of MAC

NOTE 3:	The NONCE_d and MAC is not needed for the AIoT Device Permanent ID protection, but used by the network to Authenticate the response message and protect against replay attacks

10. The NG-RAN forwards the NAS message to the AIOTF

11. The AIOTF request the ADM to verify the Inventory Response by deriving the XMAC and check if the XMAC=MAC. The ADM reports the Permanent Device ID to the AIOTF with the result of the message verification. If the message was authentic, the ADM generates the next Temp ID_n+1 in the same way as the AIoT Device did in step 9 and stores Temp ID_n+1 and stores it in the AIoT Device Profile.

Figure 6.xx.2.1-3: Derivation of XMAC

12-14. Details of step 12-14 are specified in TS 23.369. No changes are needed to protect the AIoT Device Permanent ID

#### 6.xx.2.3 Individual Inventory + Command Procedure

Figure 6.xx.2.3-1: Procedure for Device ID protection during Individual + Command procedure

0. The AIoT Device has a pre-configured Random number a.k.a. a SEED stored in the device. The Network (the ADM) also have the SEED stored together with the device Permanent ID, pre-shared key in the AIoT Device profile. Both the AIoT Device and ADM use the SEED to derive the Initial Temp ID_0 and store it locally.

Figure 6.xx.2.3-2: Derivation of initial Temp ID

Or the AIoT Device has a pre-configured initial Temp ID, the Network (the ADM) also have the initial Temp ID stored together with the device Permanent ID, pre-shared key in the AIoT Device profile.

1-6. Details of step 1-6 are specified in TS 23.369. No changes are needed to protect the AIoT Device Permanent ID.

7a-7b. AIOTF checks the Permanent ID with ADM. The ADM and to retrieves the stored Temp ID_n and generate a session key and NONCE_nw. ADM responds with the Temp ID_n, a session key and a NONCE_nw that was used to derive the session key.

8. The AIoTF sends an Inventory Request to the NG-RAN. The Inventory requests include Temp ID_n, Correlation ID, follow-on command indicator and NONCE_nw). The Correlation ID is used to link all responses related this Inventory Request.

9. The NG-RAN sends an Inventory Response to the AIOTF with the Correlation ID indicating that the Inventory Request is received successfully and will perform the service operation accordingly.

10. The NG-RAN Reader sends the paging message which includes the Temp ID_n, Command Indicator, and NONCE_nw in the Container as specified by RAN2. If a AIoT Device matches its identity to the Temp ID_n, the NG-RAN and AIoT device performs the Ambient IoT random access procedure as specified by RAN2. The AIoT Device calculates the session key using the NONCE_nw in the same way as ADM, generates a NONCE_d and derives a MAC.

Figure 6.xx.2.3-3: Derivation of MAC

The AIoT Device replies to the paging request with D2R with the Temp ID, NONCE_d and MAC. .

11. The NG-RAN forwards the NAS message to the AIOTF in the Inventory Report message.

12. The AIOTF verifies the Inventory Response by deriving the XMAC and check if the XMAC=MAC. The AIOTF requests the ADM to generate a new Temp ID_n+1. The ADM generates the next Temp ID_n+1 and stores it in the AIoT Device Profile.

Figure 6.xx.2.1-3: Derivation of XMAC

13. AIOTF encrypts NAS message, (as concluded in KI#4) using the session key, the message includes both the command and new Temp ID_n+1. The AIOTF also derives a MAC. The AIOTF sends a Command Request to the NG-RAN..

Figure 6.xx.2.1-4: Derivation of MAC

14. The NG-RAN Reader sends the R2D message which includes the encrypted NAS message (Command and new Temp ID_n+1) and the MAC in the Container as specified by RAN2.

15. AIoT device verifies the MAC and decrypts the NAS message and performs the command and stores the new Temp ID_n+1 in NVM.

16. The AIoT Device send a D2R message that includes encrypted NAS message and a MAC.

17. The NG-RAN sends a Command response that includes encrypted NAS message and a MAC to the AIOTF. The AIOTF verifies and decrypts the NAS message.

18-19. Details of step 18-19 are specified in TS 23.369.

#### 6.xx.2.4 Temp_ID Out-of-Synch detection and Resynchronization

In case the network does not receive an Inventory Response from a AIoT Device after an Individual Inventory Request as described in clause 6.xx.2.2 or 6.xx.2.3, it can indicate that the AIoT Device and network is out-of-synch with the Temp IDs. This can happen if:

- The Inventory Response from the Device was lost during transmission due to radio issues e.g. interference, range, etc. in that case the AIoT Device would generate the Temp ID_n+1, but the ADM would not generate the Temp ID_n+1 as it did not get any response.

- Something when wrong during the Inventory procedure e.g. the AIoT Device managed to write to the NVM but not send the inventory response or the AIoT Device sent the inventory response but was not able to write to the NVM.

This means that the ADM either has a Temp ID that is older or newer than the Temp ID in the AIoT Device. They can never be more than one off.

Temp ID sequence recovery is possible if the network performs Individual Inventory with both Temp ID_n-1 or Temp ID_n+1. When the device responds to the network, the network adjusts the sequence, and both are in synch again.

The network can also verify the out-of-synch problem by performing a group inventory with a group that includes the specific device. If the device response to that group Inventory, the network knows that there is an out-of-synch issue. If the AIoT Device didn’t respond, the network knows that the device is not reachable and there may not be an out-of-synch issue.

An optional method to re-synchronize is to perform a group inventory with an indicator that indicates that the AIoT Device shall use the received NONCE_nw as a new SEED and restart the Temp ID sequence using this SEED.

NOTE:	All devices that receive and respond to the group inventory, will restart their Temp ID sequence. This optional method must be carefully used if specified.

### 6.xx.3	Evaluation

This hybrid solution has the following properties:

- Group Inventory will not require device to have an NVM or writing to the NVM. This will minimize the frequency of writes to the NVM as requested by RAN1.

- Individual Inventory will trigger the AIoT Device to write a new Temp ID to the NVM. That is aligned with RAN1 as they state that the device will be able to write at some point in time, meaning that the write may happen once enough energy has been harvested.

- A Temp_ID is only used in one pair of messages (request + response).

- The Target Device Information in the group inventory is not protected by network layer security but assumed to be hard to parse if intercepted.

- There is no threat that the device Temp ID gets out-of-synch due to attacker broadcasting group inventory (e.g. replay of intercepted group inventory request). It will load the radio interface.

Editor’s Note: Further evaluation is FFS.

* * * End of Changes * * * *