3GPP TSG-SA3 Meeting #122	S3-251897

# Fukuoka, Japan, 19 - 23 May 2025

Source:	Sony, OPPO

Title:	Procedure for Protection of AIoT device identifier privacy

Document for:	Approval

Agenda item:	4.26

Spec:	3GPP TS 33.369

Version:	0.1.0

Work Item:	AIOT_Sec

Comments

Based on the conclusion in TR 33.713 the following text is proposed.

Proposed Changes

* * * First Change * * * *

## 5.4	Protection of AIoT device identifier privacy

### 5.4.1	Introduction

The protection of AIoT Device identifiers applies to the following use cases:

-	Group Inventory, this can be stand-alone event or an event that precedes dedicated Command procedure as specified in TS 23.369 [2].

-	Individual Inventory only, this allows the network to check for one specific AIoT Device.

-	Individual Inventory + Command, this allows the network to page one AIoT Device and deliver a command to that AIoT Device.

The details how the AIoT Device identities are protected are documented in the procedures. The following high-level principles applies:

- For Group Inventory, no protection of the Target Device Information in the paging message is added. It is assumed that e.g. the filtering information in the Target Device Information provides enough protection and privacy. Each AIoT Device that responds to the paging message will protect its identity by using a Temp ID derived locally using a freshness parameter.

- For Individual Inventory only, the network provides AIoT Device privacy by use of Temp ID in the paging message. The AIoT Device responds to the paging message with the Temp ID and integrity protection as described in clause 5.x. After completion of the Inventory procedure a new Temp ID is locally derived by both the AIoT Device and the network. The new Temp ID is used the next time the AIoT Device is individually paged.

- When the Inventory + Command procedure is performed, the network generates a new Temp ID and send it securely to the AIoT device in the following Command message. It allows the AIoT Device to save some power of locally deriving a new Temp ID. The new Temp ID is used the next time the AIoT Device is individually paged.

- Temp ID synchronization and re-synchronization are described in separate clause 5.4.3.

### 5.4.2	Procedures for AIoT Device identifier protection

#### 5.4.2.1 Group Inventory Procedure

This procedure details how the AIoT Device Identity is protected when the network performs a group inventory.

Figure 5.4.2.1-1: Procedure for Device ID protection during group inventory

Procedure:

1-3. Details of step 1-3 are specified in TS 23.369 [2]. No changes are needed to protect the AIoT Device Permanent ID.

NOTE 1:	The Target Device Information provided by the AF addresses a group of devices.

4. The AIOTF generate a NONCE_nw. In this step the AIOTF may exchange the Target Device Information and NONCE_nw with the ADM, but this can also be done in step 11.

5-6. Details of step 1-3 are specified in TS 23.369 [2]. No changes are needed to protect the AIoT Device Permanent ID.

7. The AIOTF sends an Inventory Request to the NG-RAN. The Inventory requests include Target device Information, Correlation ID as specified TS 23.369 [2] and the NONCE_nw. The Correlation ID is used to link all AIoT Device responses related this Inventory Request.

NOTE 2:	The AIOTF does not add any protection to the Target device Information.

8. Details of this step is specified in TS 23.369 [2]. No changes are needed to protect the AIoT Device ID.

9a. The NG-RAN Reader sends the paging message which includes the Target device Information (e.g., filter) and the NONCE_nw in the Container as specified by RAN2. If an AIoT Device matches its identify to the Target device Information (e.g., filter) it uses the NONCE_nw to derive a Temp ID, NONCE_d using an HMAC.

NOTE 3: The NONCE_d is not used to protect the AIoT Device Identifier. It is used to provide integrity protection of the response as described in clause 5.3.

Figure 5.4.2.1-2: Derivation of Temp ID

9b-c. The NG-RAN and AIoT device performs the Ambient IoT random access procedure as specified by RAN2.

9d. The AIoT Device sends the D2R NAS message which includes the Temp ID.

10. The NG-RAN forwards the NAS message to the AIOTF

11a. The AIOTF request the ADM to check the Temp ID(s) in the responses received linked to the Correlation ID. The Request includes the Target Device Information, NONCE_nw and the Temp ID for each AIoT Device response. ADM generates the Temp IDs of all AIoT Devices in the group of devices that matches to the Target device Information, in the same way as the AIoT Device did in step 9.

11b. The ADM reports the AIoT Device Permanent IDs to the AIOTF for each of the device that was verified during the check.

12-14. Details of step 12-14 are specified in TS 23.369 [2]. No changes are needed to protect the AIoT Device Permanent ID

#### 5.4.2.2 Individual Inventory Only Procedure

This procedure details how the Device Identity is protected when the network performs an Individual inventory.

Figure 5.4.2.2-1: Procedure for Device ID protection during Individual inventory only

Procedure:

0. The AIoT Device has a pre-configured Random number a.k.a. a SEED stored in the device. The Network (the ADM) also have the SEED stored together with the device Permanent ID, pre-shared key in the AIoT Device profile. Both the AIoT Device and ADM use the SEED to derive the Initial Temp ID_0 and store it locally.

Figure 5.4.2.2-2: Derivation of initial Temp ID

Or the AIoT Device has a pre-configured initial Temp ID, the Network (the ADM) also have the initial Temp ID stored together with the device Permanent ID, pre-shared key in the AIoT Device profile.

1-3. Details of step 1-3 are specified in TS 23.369 [2]. No changes are needed to protect the AIoT Device Permanent ID.

NOTE 1:	The Target Device Information provided by the AF addresses a specific device i.e., its AIoT Device Permanent ID.

4. The AIOTF generate a NONCE_nw. The AIOTF fetches the current Temp ID_n from the AIoT Device Profile in the ADM or from local device context in the AIOTF.

5-6. Details of step 1-3 are specified in TS 23.369 [2]. No changes are needed to protect the AIoT Device Permanent ID.

7. The AIOTF sends an Inventory Request to the NG-RAN. The Inventory requests include Temp ID_n, Correlation ID as specified TS 23.369 [2] and the NONCE_nw. The Correlation ID is used to link response related this Inventory Request.

8. Details of this step is specified in TS 23.369 [2]. No changes are needed to protect the AIoT Device Permanent ID.

9. The NG-RAN Reader sends the paging message which includes the Temp ID_n and the NONCE_nw in the Container as specified by RAN2. If a AIoT Device matches its identity to the Temp ID_n, the NG-RAN and AIoT device performs the Ambient IoT random access procedure as specified by RAN2. The AIoT Device generates a new Temp_ID_n+1 and NONCE_d and stores the new Temp ID_n+1.

NOTE 2: The NONCE_d is not used to protect the AIoT Device Identifier. It is used to provide integrity protection of the response as described in clause 5.3.

Figure 5.4.2.2-3: Derivation of Temp ID_n+1 and NONCE_d

The AIoT Device sends the Inventory Response NAS message which includes the Temp ID_n.

10. The NG-RAN forwards the NAS message to the AIOTF

11. The AIOTF request the ADM to verify the Inventory Response. The ADM reports the result of the message verification and optionally the AIoT Device Permanent ID. If the message was authentic, the ADM generates the next Temp ID_n+1 in the same way as the AIoT Device did in step 9 and stores Temp ID_n+1 and stores it in the AIoT Device Profile.

NOTE 3: The message verification is specified in clause 5.3.

12-14. Details of step 12-14 are specified in TS 23.369 [2]. No changes are needed to protect the AIoT Device Permanent ID

#### 5.4.2.3 Individual Inventory + Command Procedure

Figure 5.4.2.3-1: Procedure for Device ID protection during Individual + Command procedure

0. The AIoT Device has a pre-configured Random number a.k.a. a SEED stored in the device. The Network (the ADM) also have the SEED stored together with the device Permanent ID, pre-shared key in the AIoT Device profile. Both the AIoT Device and ADM use the SEED to derive the Initial Temp ID_0 and store it locally.

Figure 5.4.2.3-2: Derivation of initial Temp ID

Or the AIoT Device has a pre-configured initial Temp ID_0, the Network (the ADM) also have the initial Temp ID_0 stored together with the device Permanent ID, pre-shared key in the AIoT Device profile.

1-6. Details of step 1-6 are specified in TS 23.369 [2]. No changes are needed to protect the AIoT Device Permanent ID.

7a-7b. AIOTF checks the Permanent ID with ADM. The ADM retrieves the stored Temp ID_n and generate a session key and NONCE_nw. ADM responds with the Temp ID_n, the session key and NONCE_nw that was used to derive the session key.

8. The AIoTF sends an Inventory Request to the NG-RAN. The Inventory requests include Temp ID_n, Correlation ID, follow-on command indicator and NONCE_nw). The Correlation ID is used to link all responses related this Inventory Request.

9. The NG-RAN sends an Inventory Response to the AIOTF with the Correlation ID indicating that the Inventory Request is received successfully and will perform the service operation accordingly.

10. The NG-RAN Reader sends the paging message which includes the Temp ID_n, Command Indicator, and NONCE_nw in the Container as specified by RAN2. If a AIoT Device matches its identity to the Temp ID_n, the NG-RAN and AIoT device performs the Ambient IoT random access procedure as specified by RAN2. The AIoT Device calculates the session key using the NONCE_nw in the same way as ADM and integrity protects the Inventory response message as specified in clause 5.3.

The AIoT Device replies to the paging message with D2R with the Temp ID_n.

11. The NG-RAN forwards the NAS message to the AIOTF in the Inventory Report message.

12. The AIOTF verifies the Inventory Response message. The AIOTF requests the ADM to generate a new Temp ID_n+1. The ADM generates the next Temp ID_n+1 and stores it in the AIoT Device Profile.

13. AIOTF encrypts NAS message as specified in clause 5.3 using the session key, the NAS message includes both the command and new Temp ID_n+1.

14. The NG-RAN Reader sends the R2D message which includes the encrypted NAS message (Command and new Temp ID_n+1) in the Container as specified by RAN2.

15. AIoT device verifies and decrypts the NAS message and performs the command and stores the new Temp ID_n+1.

16. The AIoT Device send a D2R message that includes encrypted NAS message.

17. The NG-RAN sends a Command response that includes encrypted NAS message to the AIOTF. The AIOTF verifies and decrypts the NAS message.

18-19. Details of step 18-19 are specified in TS 23.369 [2].

#### 5.4.3 Out-of-Synch detection and Resynchronization of Temp_ID

In case the network does not receive an Inventory Response from a AIoT Device after an Individual Inventory Request as described in clause 5.4.2.2 and 5.4.2.3, it can indicate that the AIoT Device and network is out-of-synch with the Temp IDs. This can happen if e.g.,:

- The Inventory Response or Command Response from the Device was lost during transmission due to radio link issues e.g. interference, range, etc. in that case the AIoT Device would generate the Temp ID_n+1, but the ADM would not generate the Temp ID_n+1 or know that the device has received the Temp ID_n+1 as it did not get any response.

- Something went wrong during the Inventory procedure e.g. the AIoT Device managed to write to the NVM but not send the inventory response or command response or the AIoT Device sent the inventory response or command response but was not able to write to the NVM.

This means that the ADM either has a Temp ID that is older or newer than the Temp ID in the AIoT Device. They can never be more than one off.

Temp ID sequence recovery is possible if the network performs Individual Inventory with both Temp ID_n-1 or Temp ID_n+1. When the device responds to the network, the network adjusts the sequence, and both are in synch again.

The network can verify the out-of-synch problem by performing a group inventory with a group that includes the specific device. If the device response to that group Inventory, the network knows that there is an out-of-synch issue. If the AIoT Device didn’t respond, the network knows that the device is not reachable and there may not be an out-of-synch issue.

Optional method to re-synchronize is to perform a group inventory with an indicator that indicates that the AIoT Device shall use the received NONCE_nw as a new SEED and restart the Temp ID sequence using this SEED.

Editor’s Note: All devices that receive and respond to the group inventory, will restart their Temp ID sequence. This optional method must be carefully used if specified.