3GPP TSG-SA3 Meeting #122	S3-251901

# Fukuoka, Japan, 19 - 23 May 225

Source:	Cisco Systems

Title:	IANA registrations

Document for:	Approval

Agenda item:	4.18

Spec:	3GPP TS 33.310

Version:	19.3.1

Work Item:	ACME_SBA

Comments

This pCR provides the information for the IANA registrations for the Annex YY (informative): Certificate management for 5GC NFs using ACME. In doing so, it addresses the Editor's note in clause YY.3.3.2 of draft-CR S3-251863.

The IANA registration information is provided in a new clause. The work item can be completed, TS 33.310 can be updated accordingly, and this clause can be updated once the IANA registrations are complete.

Proposed Changes

* * * First Change * * * *

#### YY.3.3.2	"NfInstanceId" identifier type

A new ACME identifier type, "NfInstanceId", is defined in this clause. A NF uses its NF Instance ID as the value of the “NfInstanceId". The format of the value of the "NfInstanceId" is that of the NfInstanceId, as defined in TS 29.571 [57]:

-	NfInstanceId: string: String uniquely identifying a NF instance. The format of the NF Instance ID shall be a Universally Unique Identifier (UUID) version 4, as described in RFC 4122 [64]. The hexadecimal letters should be formatted as lower-case characters by the sender, and they shall be handled as case-insensitive by the receiver.

-	Example: "4ace9d34-2c69-4f99-92d5-a73a3fe8e23b"

An example of an ACME order object "identifiers" field containing a "NfInstanceId" is as follows:

- "identifiers": [{"type":"NfInstanceId","value":"4ace9d34-2c69-4f99-92d5-a73a3fe8e23b"}]

In NF certificates, both client and server, the subjectAltName extension contains the NfInstanceId as a "uniformResourceIdentifier" formatted as a URN as described in clause 5.3.2 of TS 29.571 [57]. For example, "urn:uuid:4ace9d34-2c69-4f99-92d5-a73a3fe8e23b" is the string representation of the NF Instance ID "4ace9d34-2c69-4f99-92d5-a73a3fe8e23b" as a URN.

When processing a certificate order containing an identifier of type "NfInstanceId", a CA uses the Authority Token challenge type of "tkauth-01" with a "tkauth-type" of "atc", as defined in RFC 9447 [XB], to verify that the requesting ACME client has authenticated and authorized control over the requested resources represented by the "NfInstanceId" value as well as any other NF profile parameters included in the certificate order.

The NF's ACME client responds to the challenge by posting the Authority Token, as received from the OAM system, to the challenge URL identified in the returned ACME authorization object, an example of which follows:

POST /acme/chall/prV_B7yEyA4 HTTP/1.1

Host: boulder.example.com

Content-Type: application/jose+json

{

"protected": base64url({

"alg": "ES256",

"kid": "https://example.com/acme/acct/evOfKhNU60wg",

"nonce": "Q_s3MWoqT05TrdkM2MTDcw",

"url": "https://boulder.example.com/acme/authz/asdf/0"

}),

"payload": base64url({

"tkauth": "DGyRejmCefe7v4N...vb29HhjjLPSggwiE"

}),

"signature": "9cbg5JO1Gf5YLjjz...SpkUfcdPai9uVYYQ"

}

The "tkauth" field is, as defined in RFC 9448 [XE], a field in the challenge object specific to the tkauth-01 challenge type that contains an Authority Token as defined in the next clause.

* * * Next Change (all new text) * * * *

## YY.X	IANA registrations

### YY.X.1	IANA registration information for a new ACME Identifier Type

The following value can be registered as a new ACME Identifier Type in the "ACME Identifier Types" registry, as defined in section 9.7.7 of RFC 8555 [XX].

-	Label:	NfInstanceId

-	Reference:	3GPP TS 33.310

### YY.X.2	IANA registration information for a new ACME Validation Method

The following entry can be registered as a new ACME Validation Method in the "ACME Validation Methods" registry, as defined in section 9.7.8 of RFC 8555 [XX].

-	Label:	tkauth-01

-	Identifier Type:	NfInstanceId

-	ACME:	Y

-	Reference:	3GPP TS 33.310

* * * End of Changes * * * *