3GPP TSG-SA3 Meeting #122	S3-251902

# Fukuoka, Japan, 19 - 23 May 2025

Source:	Cisco Systems

Title:	Acquiring an authority token

Document for:	Approval

Agenda item:	4.18

Spec:	3GPP TS 33.310

Version:	19.3.1

Work Item:	ACME_SBA

Comments

This pCR clarifies the secure process used by the NF to acquire a NF Certificate Authority Token from the OAM. Clause YY.2.1 includes the following, "It is assumed that the 5G NF has an authenticated channel to the OAM (e.g., established using an OAM-issued TLS client certificate or platform specific method)." This authenticated channel is used by the NF to secure the HTTP POST transaction used to acquire a NF Certificate Authority Token.

Proposed Changes

* * * First Change * * * *

#### YY.3.3.3	NF Certificate Authority Token

A new Authority Token profile, NF Certificate Authority Token, is defined in this clause. The NF Certificate Authority Token is a profile instance of the ACME Authority Token defined in RFC 9447 [XB].

The NF Certificate Authority Token protected header meets the requirements for "Request Authentication", as specified in RFC 8555 [XX], section 6.2.

The NF Certificate Authority Token payload includes the mandatory claims "exp", "jti", and "atc":

-	"exp" claim, defined in RFC 7519 [XC], section 4.1.4, is included and contains the DateTime value of the date and time that the NF Certificate Authority Token expires.

-	"jti" claim, defined in RFC 7519 [XC], section 4.1.7, is included and contains a unique identifier for this NF Certificate Authority Token transaction.

-	"atc" claim, defined in RFC 9447 [XB], is included and contains a JSON object with the following mandatory elements:

-	"tktype" key with a string value equal to "NfInstanceId" to identify this as a NF Instance ID claim.

-	"tkvalue" key with a string value equal to value of the "NfInstanceId".

-	"fingerprint" key constructed as defined in RFC 8555 [XX], section 8.1, corresponding to the computation of the "Thumbprint" step using the ACME account key credentials.

Additional elements for additional NF profile parameters can optionally be included, per RFC 9447 [XB], section 4. These include the following:

-	"nftype" key with a string value equal to an NF Type as defined in RFC 9310 [XF],

-	"sans" key with value of an array of identifiers, as defined in clause 6.1.3c, to be included as SANs in addition to the NF Instance ID.

An example of the NF Certificate Authority Token is as follows:

{

"protected": base64url({

"typ":"JWT",

"alg":"ES256",

"x5u":"https://authority.example.org/cert"

}),

"payload": base64url({

"exp":1640995200,

"jti":"id6098364921",

"atc":{"tktype":"NfInstanceId",

"tkvalue":"4ace9d34-2c69-4f99-92d5-a73a3fe8e23b",

"fingerprint":"SHA256 56:3E:CF:AE:83:CA:4D:15:B0:29:FF:1B:71:

D3:BA:B9:19:81:F8:50:9B:DF:4A:D4:39:72:E2:B1:F0:B9:38:E3",

"nftype" : "AMF",

"sans" : ["amf1234.mcc.mnc.3ggp.org"]}

}),

"signature": "9cbg5JO1Gf5YLjjz...SpkUfcdPai9uVYYQ"

}

The Authority Token is acquired by the NF using a RESTful HTTP POST transaction within the NF's authenticated channel to the OAM as follows:

POST /at/account/:id/token HTTP/1.1

Host: authority.example.org

Content-Type: application/json

The request includes the account identifier as a string in the request parameter "id". This string is managed as an identifier specific to the Token Authority's relationship with an operator CA.

The body of the POST request contains a JSON object with key value pairs corresponding to values that are requested as the content of the claims in the issued token. An example is as follows:

{

"tktype":"NfInstanceId",

"tkvalue":"4ace9d34-2c69-4f99-92d5-a73a3fe8e23b",

"fingerprint":"SHA256 56:3E:CF:AE:83:CA:4D:15:B0:29:FF:1B:71:D3

:BA:B9:19:81:F8:50:9B:DF:4A:D4:39:72:E2:B1:F0:B9:38:E3",

"nftype" : "AMF",

"sans" : ["amf1234.mcc.mnc.3ggp.org"]}

}

If successful, the response to the POST request returns a 200 (OK) with a JSON body that contains, at a minimum, the NF Certificate Authority Token as a JSON object with a key of "token" and the base64url-encoded string representing the atc token. An example of a successful response is as follows:

HTTP/1.1 200 OK

Content-Type: application/json

{"token": "DGyRejmCefe7v4N...vb29HhjjLPSggwiE"}

If the request is not successful, the response indicates the error condition. Specifically, for the case that the account identifier provided does not exist, the response code 403 (Forbidden) is returned. Other 4xx and 5xx responses follow standard HTTP error condition conventions, as described in RFC 9110 [65].

When creating the NF Certificate Authority Token, the Token Authority validates that the information contained in the token accurately represents the NF Instance ID and additional NF profile parameters the requesting party is authorized to represent based on their pre-established, verified, and secure relationship. Note that the fingerprint in the token request is not meant to be verified by the Token Authority but rather is meant to be signed as part of the token so that the party that requests the token can, as part of the challenge response, allow the ACME server to validate that the token requested and used came from the same party that controls the ACME client.

* * * End of Changes * * * *