3GPP TSG-SA3 Meeting #122	S3-251940

# Fukuoka, Japan,  19 - 23 May 2025

Source:	ZTE

Title:	Resolving EN for Authentication and authorization for digital asset services

Document for:	Approval

Agenda item:	4.21

Spec:	TS 33.434

Version:	18.2.0

Work Item:	Security aspects of 5G Mobile Metaverse services

Comments

This pCR proposes to resolve the ENs in clause 6.Y according to the progress in CAPIF_Ph3_Sec.

Proposed Changes

* * * First Change * * * *

6.Y	Authentication and authorization for digital asset services

6.y.1 Authentication and authorization for digital asset services when CAPIF is used

When CAPIF is used as specified in TS 23.434 [2], the security mechanism for CAPIF specified in TS 33.122 [29] is referred to protect interfaces between digital asset client and digital asset server specified in TS 23.438 [y], between VAL server and digital asset server. The digital asset (DA) requestor (e.g. digital asset client or VAL server) takes the role of API Invoker in CAPIF. Digital asset server takes role of AEF in CAPIF.

Authentication and authorization for RNAA specified in clause 6.5.3 of TS 33.122 [29] shall be followed for the DA server to authorize the DA requestor. In addition to verifying the access token, the DA server shall further check the access control list in the DA profile.

Figure 6.Y.1-1: Procedure for DA requestor authorization to access DA services with token

1. 	The DA requestor sends the DA service request (e.g. DA create/retrieve/update/delete request) to the DA server. The access token received from the CCF is sent along with the DA service request. The UE subscriber identity, e.g. GPSI, is included in the access token, as per the procedure in TS 33.122 [29] clause 6.5.3.

2. 	Upon receiving the request with an access token, the DA server validates the integrity of the token using the CCF’s certificate. If validation of the token is successful, the DA server further verifies the DA service request against the authorization claims in the token.

In case the access token does not contain sufficient claims for authorizing the DA service API invocation, the DA server further checks its local DA profile for finer level authorization if needed. For example, if the DA profile contains the information of owner or allowed user or allowed application or allowed operation for resource level authorization, the DA server checks if the UE subscriber matches the owner or allowed user.

3.	If the DA service API invocation is successfully authorized by the DA server, it executes the service logic for the invoked DA service API and returns the DA service response as a result of the DA service API invocation.

NOTE x: The assumption is that the user behind the digital asset client is the UE subscriber.

NOTE y: How CAPIF RNAA supports service operation level and resource level authorization is specified in TS 33.122 [29].

* * * End of Changes * * * *