# 3GPP TSG-SA3 Meeting #122																			S3-251950

# Fukuoka, Japan, 19 – 23 May 2025

Source:	ZTE

Title:	Update the clause 5.2

Document for:	Approval

Agenda item:	4.26

Spec:	3GPP TS 33.369

Version:	0.1.0

Work Item:	AmbientIoT-SEC

Comments

The authentication procedure needs to be added.

Proposed Changes

* * * First Change * * * *

5.2	Authentication procedure

Editor’s Note: This clause contains the security procedures on the authentication.

The authentication procedure for inventory only is described as follows.

0.	Credentials are securely stored in the ADM on the network side and AIoT device.

1-5.	These steps refer to the step 1-6 in the TS 23.369[2], the AIoTF gets the session key and the counter from the ADM if the AIoTF does not have the session key and counter related to the AIoT device.

6.	The AIOTF sends the Inventory Request message include the AIoT Device Identification informationto the selected AIoT RAN.

7.	The AIoT RAN sends an Inventory Response to the AIoTF indicating that the Inventory Request is received successfully and will perform the service operation accordingly.

8.	Upon reception of the Inventory Request message from the AIOTF, the AIoT Reader(s) will execute the inventory operation. The AIoT Reader(s) sends an Inventory Response to the AIoT device.

9. If an AIoT device matches the AIoT Device Identification information in the paging message, the AIoT Device generates the session key Ks based on the long term key and calculates the MAC. MAC=HMAC(device ID, counter), the input key is Ks. Then the AIoT device increase the counter by one.

10. The AIoT device responds to the paging message and sends an AIOT NAS message that includes its AIoT Device ID and MAC.

11.	AIoT RAN sends one or more Inventory Report messages to the AIOTF including  the AIOT NAS message(s) from the AIoT Device(s).

12.	The AIOTF calculate a XMAC and validates the MAC received. If successful, the AIoTF increase the counter by one.

13.	The AIOTF reports the progress of the AIoT inventory request to the NEF by sending the AIoT_Notify message including a list of AIoT Device ID(s).

14.	The NEF informs the AF of the outcome of the AIoT_Inventory request by sending the AIoT_Notify message including the AIoT Device ID(s).

* * * End of Changes * * * *