# 3GPP TSG-SA3 Meeting #122																			S3-251953

Fukuoka, Japan, 19 – 23 May 2025

Source:	ZTE

Title:	update solution #6

Document for:	Approval

Agenda item:	5.9

Spec:	3GPP TR 33.713

Version:	0.8.0

Work Item:	FS_AIOT_Sec

Comments

- Add step 0 to the Figure 6.6.2-1.

- Delete the EN base on the conclusion in clause 7.3.

- Update some typo in clause 6.6.1 and 6.6.2.

Proposed Changes

* * * First Change * * * *

6.6	Solution #6: AIoT device authentication

6.6.1	Introduction

This solution addresses KI#5.

This solution propose an AIoT device Security Management and an AIoT controller. The AIoT device Security Management is a core network function, has the following security capabilities:
1. Stores the initial Kaiot together with the AIoT device. Derives the Ks and this key is used for MAC calculation.

2. Sends the Ks and counter to AIoT controller to calculate the network MAC.

3. Synchronizes the counter with the AIoT device.

4. Stores the mapping relationship between the device ID and reader ID.

AIoT controller has capability about calculating the network MAC and verifying the device MAC.

AIoT device has capability about calculating the device MAC and verifying the network MAC.

The counter is maintained by the AIoT controller and the AIoT device.

6.6.2	Solution details

The following figure shows the call flow for AIoT device authentication.

Figure 6.6.2-1: AIoT device authentication

0.	AIoT device and AIoT device Security management both store the Kaiot and initial counter. AIoT device stores Device ID. The AIoT device Security management stores the mapping relationship between the Device ID and reader ID.

1.	AF sends Inventory(or command) request to AIoT controller, which includes Reader ID and Device ID. The Reader ID indicates the reader requested for Inventory(or command). Multiple Reader IDs can be included. The Device ID indicates the AIoT device for Inventory(or command). Multiple Device IDs can be included.

2.	AIoT controller sends Device information request to AIoT device Security management, which includes Device ID received in step 1. If multiple Device IDs are received in step 1, AIoT controller can repeat this step per Device ID for multiple times, or AIoT controller can send the Device ID list to AIoT device Security management in the same message.

3.	The AIoT device Security management derives the Ks base on the device ID, counter and Kaiot. for example, Ks= KDF (device ID,  counter, Kaiot), the input key is Kaiot, where Kaiot is the Kaiot stored on the AIoT device Security management. If  multiple Device IDs in one group are received in step 1, the device ID may be the prefix ID of the group.

4.	AIoT device Security management sends Device information response to AIoT controller, which includes the reader ID, counter and Ks, if the network decides to synchronize the counter with the AIoT device, the start indication is included and the counter is the initial value (e.g., 0). If Multiple Device IDs are received in step 2, this message may include all device information corresponding to the Device IDs.

5.  The AIoT controller calculates the network MAC based on the Ks and counter, for example, MACn=HMAC(device ID, counter), the input key is Ks. If  multiple Device IDs in one group are received in step 1, the device ID may the prefix ID of the group.

6.	The AIoT controller sends the Inventory(or command) Request to the reader(s) base on reader ID in step 3, which includes Device ID and MACn. If the counter is initial value, which includes start indication.

Note 1: How to protect privacy of device identifier will be addressed in Key issue 3.

7.	The Reader(s) send Inventory(or command) Request to AIoT device, which includes Device ID and MACn, and which may includes start indication.

8. The AIoT device derives the Ks like the AIoT device Security management, verifies the MACn, that is, the AIoT device calculates the local MACn', for example, MACn’=HMAC(device ID, counter), the input key is Kaiot.  The counter is the counter stored in the AIoT device. If start indication is valid, the the counter is initial counter value. If MACn and MACn' are the same, the verification succeeds. The AIoT device increases the counter by one, and store the new counter. The AIoT device can optionally calculate the device MAC, for example, MACu= HMAC (device ID, new counter), the input key is Ks, where Ks is derived by the AIoT device base on the Ks and new counter. If  multiple Device IDs in one group are received in step 1, the device ID may be the prefix ID of the group.

9.	The AIoT device sends Inventory(or command) Response to the Reader with its Device ID, which may includes MACu. If the new counter is initial value (e,g, due to wrap around), start indication is included.

10.	The Reader sends Inventory(or command) Response to the AIoT controller with the Device ID and Reader ID, which may includes MACu.

11. The AIoT controller increases the counter by one and calculates the MACu' like the AIoT device, and verifies the MACu'. If the start indication is received, the AIoT controller check whether the new counter is the initial value first. If the verification is successful, the Inventory(or command) is successful. AIoT controller stores the new counter.

12.	AIoT controller stores the Device ID and the Reader ID, if multiple Inventory(or command) Responses are received from different Readers, the AIoT controller stores the Device ID and multiple Reader IDs. AIoT controller sends the Inventory(or command) Response to AF with the Device ID and Reader ID(s).

13.  The AIoT controller sends the new counter to the AIoT device Security management, and the AIoT device Security management stores the new counter.

Note 2: The Kaiot protection in the device is left to implementation.

6.6.3	Evaluation

This solution addresses the requirement of Key Issue #5.

This solution assumes that the AIoT device Security Management stores key authentication credential. The AIoT controller has capability about calculating the network MAC and verifying the device MAC, AIoT device has capability about calculating the device MAC and verifying the network MAC.  A counter is used for the MAC calculation, the AIoT device and the AIoT device Security Management need to store the counter. AIoT device Security Management needs to synchronize the counter with the AIoT device by a start indication. The AIoT controller needs to notify the new counter to the AIoT device Security management.

The Kaiot protection in the AIoT device is left to implementation.

AIoT device Security Management needs to store the mapping relationship between the device ID and reader ID.

* * * End of Changes * * * *