3GPP TSG-SA3 Meeting #122	S3-251957

# Fukuoka, Japan,  19 - 23 May 2025

# Source:	ZTE Corporation

# Title:	Discussion paper on MAC layer security

# Document for:	Discussion

Agenda Item:	6.2

1	Decision/action requested

This contribution proposes to consider the issue as discussed in this document.

2	References

[1]		S3-251124 Reply LS on security handling for inter-CU LTM in non-DC cases

[2]		TS 38.321

3	Rationale

3.1	Security analysis of  LTM cell switch command MAC CE

LS S3-251124 [1] Considering the RAN2’s request on avoiding RRC configuration between cell switches for subsequent inter-CU LTM in solution selection, SA3 reached a compromise that the NCC can be transmitted to the UE in plaintext via MAC CE message, even though the unprotected MAC leads to increased attack scenarios and probability of handover failure. In addition, SA3 will not consider the protection of MAC CE messages in this release, but may consider studying MAC CE protection in the future.

Observation 1: SA3 has agreed that NCC can be transmitted via MAC CE message, and the unprotected MAC leads to increased attack scenarios and probability of handover failure. SA3 may consider studying MAC CE protection in the future.

3.2	Security analysis of MAC CEs with recommended security protection

The MAC CEs listed below represent a subset of those that would benefit from security protection.

3.2.1 Timing Advance Command MAC CE and Absolute Timing Advance Command MAC CE

TS 38.321[2] 6.1.3.4 Timing Advance Command MAC CE The Timing Advance Command MAC CE is identified by MAC subheader with LCID as specified in Table 6.2.1-1. It has a fixed size and consists of a single octet defined as follows (Figure 6.1.3.4-1): - TAG Identity (TAG ID): This field indicates the TAG Identity of the addressed TAG. The TAG with the Identity 0 contains the SpCell. The length of the field is 2 bits; - Timing Advance Command: This field indicates the index value TA (0, 1, 2… 63) used to control the amount of timing adjustment that MAC entity has to apply (as specified in TS 38.213 [6]). The length of the field is 6 bits. Figure 6.1.3.4-1: Timing Advance Command MAC CE 6.1.3.4a Absolute Timing Advance Command MAC CE The Absolute Timing Advance Command MAC CE is identified by MAC subheader with eLCID as specified in Table 6.2.1-1b. It has a fixed size and consists of two octets defined as follows (Figure 6.1.3.4a-1): - Timing Advance Command: This field indicates the index value TA used to control the amount of timing adjustment that the MAC entity has to apply in TS 38.213 [6]. The size of the field is 12 bits; - TI: If two TAGs are configured for the SpCell, this field indicates one of the two TAGs to which the Timing Advance Command is applied. If tag2-flag is set to true by upper layers, the field set to 0 indicates the tag2-Id and the field set to 1 indicates the tag-Id of the SpCell, otherwise the field set to 0 indicates the tag-Id and the field set to 1 indicates the tag2-Id of the SpCell. If the SpCell is not configured with two TAGs, the R bit is present instead; - R: Reserved bit, set to 0. Figure 6.1.3.4a-1: Absolute Timing Advance Command MAC CE

Observation 2: Timing Advance Command MAC CE and Absolute Timing Advance Command MAC CE include timing advance information, which enables UE to adjust its uplink transmission timing. Without integrity protection, an attacker may manipulate the timing advance information hence causing UL interference to the whole cell. Without confidentiality protection, an attacker may eavesdrop the timing advance information, estimate the distance between UE and BS, and further estimate the UE’s location and movement patterns by monitoring timing advance information updates over time.

3.2.2 SCell Activation/Deactivation MAC CEs

TS 38.321[2] 6.1.3.10 SCell Activation/Deactivation MAC CEs The SCell Activation/Deactivation MAC CE of one octet is identified by a MAC subheader with LCID as specified in Table 6.2.1-1. It has a fixed size and consists of a single octet containing seven C-fields and one R-field. The SCell Activation/Deactivation MAC CE with one octet is defined as follows (Figure 6.1.3.10-1). The SCell Activation/Deactivation MAC CE of four octets is identified by a MAC subheader with LCID as specified in Table 6.2.1-1. It has a fixed size and consists of four octets containing 31 C-fields and one R-field. The SCell Activation/Deactivation MAC CE of four octets is defined as follows (Figure 6.1.3.10-2). - Ci: If there is an SCell configured for the MAC entity with SCellIndex i as specified in TS 38.331 [5], this field indicates the activation/deactivation status of the SCell with SCellIndex i, else the MAC entity shall ignore the Ci field. The Ci field is set to 1 to indicate that the SCell with SCellIndex i shall be activated. The Ci field is set to 0 to indicate that the SCell with SCellIndex i shall be deactivated; - R: Reserved bit, set to 0. Figure 6.1.3.10-1: SCell Activation/Deactivation MAC CE of one octet Figure 6.1.3.10-2: SCell Activation/Deactivation MAC CE of four octets NOTE: If UE receives the SCell Activation/Deactivation MAC CE for an SCell configured with TRS for fast activation of the SCell, such TRS is not used for the corresponding SCell.

Observation 3: SCell Activation/Deactivation MAC CE is used to activate or deactivate data transmission for SCell. Without integrity protection, an attacker can activate/deactivate cells randomly causing link interruption and UE to communicate on wrong cells even.

3.2.3 SP CSI-RS/CSI-IM Resource Set Activation/Deactivation MAC CE

TS 38.321[2] 6.1.3.12 SP CSI-RS/CSI-IM Resource Set Activation/Deactivation MAC CE The SP CSI-RS/CSI-IM Resource Set Activation/Deactivation MAC CE is identified by a MAC subheader with LCID as specified in Table 6.2.1-1. It has a variable size and consists of the following fields: - A/D: This field indicates whether to activate or deactivate indicated SP CSI-RS and CSI-IM resource set(s). The field is set to 1 to indicate activation, otherwise it indicates deactivation; - Serving Cell ID: This field indicates the identity of the Serving Cell for which the MAC CE applies. The length of the field is 5 bits; - BWP ID: This field indicates a DL BWP for which the MAC CE applies as the codepoint of the DCI bandwidth part indicator field as specified in TS 38.212 [9]. The length of the BWP ID field is 2 bits; - SP CSI-RS resource set ID: This field contains an index of NZP-CSI-RS-ResourceSet containing Semi Persistent NZP CSI-RS resources, as specified in TS 38.331 [5], indicating the Semi Persistent NZP CSI-RS resource set, which shall be activated or deactivated. The length of the field is 6 bits; - IM: This field indicates the presence of the octet containing SP CSI-IM resource set ID field. If the IM field is set to 1, the octet containing SP CSI-IM resource set ID field is present. If IM field is set to 0, the octet containing SP CSI-IM resource set ID field is not present; - SP CSI-IM resource set ID: This field contains an index of CSI-IM-ResourceSet containing Semi Persistent CSI-IM resources, as specified in TS 38.331 [5], indicating the Semi Persistent CSI-IM resource set, which shall be activated or deactivated. The length of the field is 6 bits; - TCI State IDi: This field contains TCI-StateId, as specified in TS 38.331 [5], of a TCI State, which is used as QCL source for the resource within the Semi Persistent NZP CSI-RS resource set indicated by SP CSI-RS resource set ID field. TCI State ID0 indicates TCI State for the first resource within the set, TCI State ID1 for the second one and so on. The length of the field is 7 bits. If the A/D field is set to 0, the octets containing TCI State ID field(s) are not present; - R: Reserved bit, set to 0. Figure 6.1.3.12-1: SP CSI-RS/CSI-IM Resource Set Activation/Deactivation MAC CE

Observation 4: SP CSI-RS/CSI-IM Resource Set Activation/Deactivation MAC CE includes A/D, Serving Cell ID, BWP ID, etc. Without integrity protection, an attacker may manipulate the MAC CE, resulting in wrong configuration/activation and deactivation and hence causing link interruption.

3.3	Security analysis of MAC CEs with optional security protection

The MAC CEs listed below represent a subset of those where security protection is not strictly required but may provide additional benefits.

3.3.1 Duplication Activation/Deactivation MAC CE

TS 38.321[2] 6.1.3.11 Duplication Activation/Deactivation MAC CE The Duplication Activation/Deactivation MAC CE of one octet is identified by a MAC subheader with LCID as specified in Table 6.2.1-1. It has a fixed size and consists of a single octet containing eight D-fields. The Duplication Activation/Deactivation MAC CE is defined, for a MAC entity, as follows (Figure 6.1.3.11-1). - Di: This field indicates the activation/deactivation status of the PDCP duplication of DRB i where i is the ascending order of the DRB ID among the DRBs configured with PDCP duplication and with RLC entity(ies) associated with this MAC entity. The Di field is set to 1 to indicate that the PDCP duplication of DRB i shall be activated. The Di field is set to 0 to indicate that the PDCP duplication of DRB i shall be deactivated. Figure 6.1.3.11-1: Duplication Activation/Deactivation MAC CE NOTE: The Duplication Activation/Deactivation MAC CE is not used if a DRB is configured with more than two RLC entities, i.e. with moreThanTwoRLC-DRB.

Observation 5: Duplication Activation/Deactivation MAC CE is used to inform UE to activate/deactivate PDCP packet duplication. It has a fixed size and consists of a single octet. Without integrity protection, an attack may manipulate wrong configuration/activation and deactivation, e.g., deactivate duplication in scenarios requiring high reliability, causing poor link performance. For this MAC CE, considering its minimal size, integrity protection is not mandatory, while enabling it can prevent unintended misconfigurations and enhance link performance.

3.3.2 Recommended bit rate MAC CE

6.1.3.20 Recommended bit rate MAC CE The Recommended bit rate MAC CE is identified by a MAC subheader with LCID as specified in Tables 6.2.1-1 and 6.2.1-2 for bit rate recommendation message from the gNB to the UE and bit rate recommendation query message from the UE to the gNB, respectively. It has a fixed size and consists of two octets defined as follows (Figure 6.1.3.20-1): - LCID: This field indicates the identity of the logical channel for which the recommended bit rate or the recommended bit rate query is applicable. The length of the field is 6 bits; - Uplink/Downlink (UL/DL): This field indicates whether the recommended bit rate or the recommended bit rate query applies to uplink or downlink. The length of the field is 1 bit. The UL/DL field set to 0 indicates downlink. The UL/DL field set to 1 indicates uplink; - Bit Rate: This field indicates an index to Table 6.1.3.20-1. The length of the field is 6 bits. For bit rate recommendation the value indicates the recommended bit rate. For bit rate recommendation query the value indicates the desired bit rate; - X: Bit rate multiplier. For UEs supporting recommended bit rate multiplier, when bitRateMultiplier is configured for the logical channel indicated by LCID field, X field set to 1 indicates the actual value of bit rate is the value corresponding to the index indicated by the Bit Rate field multiplied by bitRateMultiplier as specified in TS 38.331 [5]. - R: reserved bit, set to 0. Figure 6.1.3.20-1: Recommended bit rate MAC CE

Observation 6: Recommended bit rate MAC CE is used for dynamically adjust transmission rate. It has a fixed size and consists of two octets. Without integrity protection, an attacker may manipulate wrong recommendation bit rate, causing the UE to use wrong bit rates for certain applications. For this MAC CE, considering its minimal size, integrity protection is not mandatory, while enabling it can prevent unintended misconfigurations and enhance transmission performance.

Based on the observation above, in addition to LTM cell switch command MAC CE, for some MAC CEs, security protection is recommended to avoid the risk of eavesdropping and/or wrong configuration causing link interruption. For others, the absence of protection does not pose critical security threats, although they may be beneficial to have security protection.

4	Detailed proposal

Based on the observations:

Observation 1: SA3 has agreed that NCC can be transmitted via MAC CE message, and the unprotected MAC leads to increased attack scenarios and probability of handover failure. SA3 may consider studying MAC CE protection in the future.

Observation 2: Timing Advance Command MAC CE and Absolute Timing Advance Command MAC CE include timing advance information, which enables UE to adjust its uplink transmission timing. Without integrity protection, an attacker may manipulate the timing advance information hence causing UL interference to the whole cell. Without confidentiality protection, an attacker may eavesdrop the timing advance information, estimate the distance between UE and BS, and further estimate the UE’s location and movement patterns by monitoring timing advance information updates over time.

Observation 3: SCell Activation/Deactivation MAC CE is used to activate or deactivate data transmission for SCell. Without integrity protection, an attacker can activate/deactivate cells randomly causing link interruption and UE to communicate on wrong cells even.

Observation 4: SP CSI-RS/CSI-IM Resource Set Activation/Deactivation MAC CE includes A/D, Serving Cell ID, BWP ID, etc. Without integrity protection, an attacker may manipulate the MAC CE, resulting in wrong configuration/activation and deactivation and hence causing link interruption.

Observation 5: Duplication Activation/Deactivation MAC CE is used to inform UE to activate/deactivate PDCP packet duplication. It has a fixed size and consists of a single octet. Without integrity protection, an attack may manipulate wrong configuration/activation and deactivation, e.g., deactivate duplication in scenarios requiring high reliability, causing poor link performance. For this MAC CE, considering its minimal size, integrity protection is not mandatory, while enabling it can prevent unintended misconfigurations and enhance link performance.

Observation 6: Recommended bit rate MAC CE is used for dynamically adjust transmission rate. It has a fixed size and consists of two octets. Without integrity protection, an attacker may manipulate wrong recommendation bit rate, causing the UE to use wrong bit rates for certain applications. For this MAC CE, considering its minimal size, integrity protection is not mandatory, while enabling it can prevent unintended misconfigurations and enhance transmission performance.

It is proposed:

Proposal 1: It is proposed to identify MAC CE messages that need to be protected in 5G-A.

Proposal 2: It is proposed to study MAC layer security under R20 6G topic.