3GPP TSG-SA3 Meeting #122	S3-251995

# Fukuoka, Japan, 19 - 23 May 2025

Source:	Huawei, HiSilicon

Title:	removing the editor's note in solution 4

Document for:	Approval

Agenda item:	5.9

Spec:	3GPP TR 33.713

Version:	0.8.0

Work Item:	FS_Ambient_IoT_Sec

Comments

Considering information flow in solution 4 aligns with the command operation procedure in SA2, this contribution provides removing the following editor's note:

Editor’s Note: The information flow will be updated based on the conclusion regarding command operation procedure in SA2.

* * * First Change * * * *

### 6.4.2.2	Protection for inventory and command procedure

Figure 6.4.2-2: Information Flow for protection of information transfer for AIoT services

Compared with the inventory-only procedure in clause 6.4.2.1, the protection for command is enhanced with the following modification.

10. In addition to the XAuth_token calculation, the session key Ks is derived if receiving additional indication from AIoT Function. The Nonce1 and Nonce2 is used to form the input S to the KDF and the pre-shared key is used as input key to derive the session key Ks. The Ks is different with XAuth_token as different input (e.g., FC value).

11. Session key is also included. If the device security capability (e.g., AES-128 algorithm) is stored in AIoT_ UDM, it will also be sent to AMF/AIoTMF.

12. The command message is protected with session key. For example, the MAC1 will be calculated to protect the integrity of the message. The algorithm will be selected based on device security capability.

13-14. The AMF/AIoTMF sends the protected AIoT Command message to the AIoT Device, which includes encrypted data and MAC1. No AIoT device ID is required to be included in step 14, as other ID will be used between device and reader, which is discussed and decided in RAN groups. It is assumed that the device receives the command message while energy is available in energy storage.

15-16. The device will receive the message in step 14. Integrity verification and decryption will be performed. After successful verification, the device will follow the command operation. The uplink message will also be sent from device to network if needed.  For example, if the command is “read” operation, data2 to be reported will be included after encryption. The message will also be integrity protected.

17-18. The AMF/AIoTMF reports the result of the AIoT service operation request to the NEF by sending the AIoT_ notification message.

* * * End of Changes * * * *