3GPP TSG-SA3 Meeting #122	S3-251997

Fukuoka, Japan, 19 - 23 May 2025

# Source:	Huawei, HiSilicon

# Title:	Discussion on security aspects for QUIC or TLS

# Document for:	Discussion

Agenda Item:	6

1	Decision/action requested

It is proposed to discuss the security aspect for QUIC usage.

2	References

[1]	3GPP TS 23.501: "System Architecture for the 5G System".

[2]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[3]	S3-244876: "Discussion paper for using MPQUIC in ATSSS scenario".

3	Rationale

3.1	MPQUIC usage in ATSSS scenario status

In Release 18, SA2 specified MPQUIC in clause 5.32.6.2.2 of TS 23.501 [1] for steering, switching, and splitting of UDP traffic between the UE and UPF. In Release 19, SA2 specified MPQUIC in clause 5.32.6.2.2 of TS 23.501 [1] for steering, switching, and splitting of non-UDP traffic between the UE and UPF. MPQUIC uses QUIC, which further makes TLS handshake protocol mandatory.

During the SA3#117 meeting, it was decided to add an annex to describe certificate-based authentication for MPQUIC in ATSSS. Therefore, the current version of Annex AA of TS 33.501 [2] only allows the UE to use X.509 certificate in MPQUIC protocol. Neither the management of certificates nor other types of authentication mechanisms (e.g. pre-shared key mode) are specified.

3.2	Potential issues for security aspects of MPQUIC

For certificate-based authentication, the root certificate of the UPF has to be configured in the UE in order to complete the authentication procedure. In our view, the profile of the root certificate has to be considered in 3GPP specification for completeness. In addition, in some scenarios, stronger security may be required, and mutual authentication may be required. Therefore, mutual authentication is also worth studying.

For pre-shared-key (PSK) mode, as we observed in S3-244876 [3], it can save on computation and communication. Furthermore, it can be flexibly deployed and would enable Early Data (0-RTT) in QUIC protocol. This is a major advantage of such mode.

In 3GPP, the PSK between the UE and the UPF can be obtained in multiple manners, for example, through pre-configuration or derivation from the security context between the UE and the network. When and how to obtain the PSK should be discussed. And the complexity of different approaches may be compared in order to provide further reference for specific deployments.

4	Detailed proposal

Based on the rationale above, a study about the security aspects of MPQUIC with the following objectives maybe necessary for 5G-Advanced security work.

1. Study whether and how to support of mutual authentication for QUIC/TLS.

2. Study the certificate profiling and management of QUIC/TLS for UE and UPF.

3. Study the support of PSK mode, includingthe corresponding mechanism to derive or configure such keys for MPQUIC/TLS between UE and UPF.