3GPP TSG-SA3 Meeting #122																			S3-252014

Fukuoka, Japan, 19-23 May 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.535 | CR | 0233 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update service disabling procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | AKMA_Ph2 |  |  |  |  |  | Date: |  |  | 2025-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The AF may or may not running AKMA services when receiving the service disable notification. If AF is running AKMA services, AF may stop AKMA services or stop encryption. If AF is not running AKMA services, AF may not be allowed to initiate encrypted AKMA services or AF may initiate unencrypted AKMA services. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Re-word the AF behavior after AF receiving the service disable notification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.8.1, 6.8.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

6.8.1	Notification to internal AF about AKMA service disabling

This procedure is used when the AKMA sessions have already been started (before roaming was detected), and as soon as PLMN change is detected at the AAnF, the AAnF may execute this procedure based on the roaming policy.

Figure 6.8.1-1: AAnF notification to internal AF about AKMA service disable

1.	 UE registers with a (H)PLMN.

2.	 UE is accessing the AF and key material is provided to AF as described in 6.2.1. While accessing the AAnF, AF may also provide the Notification URI.

3. UE is getting registered in a VPLMN and AAnF detects the PLMN change via the Nudm_EventExposure_Notification received from UDM.

4. AAnF determines if AF(s) have subscribed to receive notifications for AKMA service disabling and roaming policy is configured and restrict the AKMA access in the VPLMN; if yes, steps 5 and 6 are executed. Otherwise, steps 5 and 6 are skipped.

5. If AF(s) are determined at step 4, the AAnF shall send notifications to the subscribed AF(s) about AKMA roaming via Naanf_AKMA_ServiceDisableNotification. The A-KID is the transmitted A-KID for the corresponding AF, which is kept track of in step 8 in clause 6.2.1.

NOTE 1:	The re-authentication of the UE might imply that the current A-KID at the AAnF is not identical to the original A-KID by which the AF previously requested the AKMA key. Therefore, the AAnF retrieves and uses the original A-KID in this message to the AF.

6. The AF shall send the response and based on the notification and internal policy, the AF may stop the UE AKMA serviceor stop the encryption when there is AKMA service running for the UE in the AF. If there is no AKMA service running for the UE in the AF, the AF may not be allowed to initiate encrypted AKMA service or AF may initiate unencrypted AKMA service for the UE.

NOTE 2:	By stopping the encryption (e.g., TLS 1.2 NULL cypher negotiation), LI interception could work in the VPLMN.

* * * Second Change * * * *

6.8.2	Notification to external AF about AKMA service disabling

Figure 6.8.2-1: AAnF notification to external AF about AKMA service disable via NEF

This procedure is used when AAnF notifies AF about AKMA service disabling via NEF.

1.	UE registers with a (H)PLMN.

2.	UE is accessing the AF and key material is provided to AF via NEF as described in 6.3. While accessing the AAnF, AF may also provide the Notification URI.

3.	For details about step 3, see step 3 in Figure 6.8.2-1.

4.	AAnF determines if AF(s) have subscribed to receive notifications for AKMA service disabling and roaming policy is configured and restrict the AKMA access in the VPLMN; if yes, steps 5-8 are executed. Otherwise, steps 5-8 are skipped.

5.	If AF(s) are determined at step 5, the AAnF sends notifications to the subscribed AF(s) via NEF about AKMA roaming. The AAnF sends Naanf_AKMA_ServiceDisableNotification to NEF. The notification includes the A-KID and the AF_ID. The A-KID is the transmitted A-KID for the corresponding AF, which is kept track of in step 4 in clause 6.3.

NOTE 1:	The re-authentication of the UE might imply that the current A-KID at the AAnF is not identical to the original A-KID by which the AF previously requested the AKMA key. Therefore, the AAnF retrieves and uses the original A-KID in this message to the AF.

6.	Once receiving the notification from AAnF, NEF shall send Nnef_AKMA_ServiceDisableNotification to AF.

7.	The AF sends Nnef_AKMA_ServiceDisableNotification response to NEF and based on the notification and internal policy, the AF may stop the UE serviceor stop the encryption when there is AKMA service running for the UE in the AF. If there is no AKMA service running for the UE in the AF, the AF may not be allowed to initiate encrypted AKMA service or AF may initiate unencrypted AKMA service for the UE.

8.	Once receiving the response from AF, the NEF sends Naanf_AKMA_ServiceDisableNotification response to AAnF.

NOTE 2: 	By stopping the encryption (e.g., TLS 1.2 NULL cypher negotiation), LI interception could work in the VPLMN.

* * * End of Changes * * * *