# 3GPP TSG-SA3 Meeting #122																			S3-252039

# Fukuoka, Japan, 19 – 23 May 2025

Source:	Samsung

Title:	pCR for authentication and authorization for RNAA in CAPIF interconnection scenario

Document for:	Approval

Agenda item:	4.22

Comments

This pCR proposes to update clause 6.X.3.4 in baseline pCR with procedure for authentication and authorization for RNAA in CAPIF interconnection scenario.

Authentication and authorization for RNAA as specified in clause 6.5.3, in TS 33.122 can be re-used.

Following the same concept as for method 3 in clause 6.X.3.3, CCF-A (AEF registered CCF) should be the authorization server for RNAA in interconnection scenario i.e. CCF-A has the role of OAuth 2.0 server (providing the access token used for RNAA).

In case of an external AF (i.e., not the application on the UE) being the API invoker, for mutual authentication of API invoker AF and API exposing function, the authentication methods for interconnection as specified in clause 6.X.3.1 and clause 6.X.3.2 can be reused.

For authorization, the OAuth 2.0 flows as specified in clause 6.5.3.1 is applicable and can be re-used.

For selecting authorization method, the procedure defined in clause 6.X.2 can be followed. The API invoker includes the supported RNAA authorization flows in the Security Method Request. The CCF-B determines the RNAA authorization flow in case has the supported authorization flows based on the RNAA capabilities of the  CCF-A, AEF, and API invoker or contacts CCF-A in order to get the selected authorization flow.

With the above understanding it is proposed to address the EN under clause 6.X.3.4.

* * * First Change * * * *

#### 6.X.3.4		Authentication and Authorization for RNAA

Authentication and authorization for RNAA as specified in clause 6.5.3, in TS 33.122 shall be re-used. The CCF-A (AEF registered CCF) has the role of the OAuth 2.0 authorization server, i.e., providing the access token used for RNAA.

In case of an external AF (i.e., not the application on the UE) being the API invoker, for mutual authentication of API invoker AF and API exposing function, the authentication methods for interconnection as specified in clause 6.X.3.1 and clause 6.X.3.2 can be reused.

For authorization, the OAuth 2.0 flows as specified in clause 6.5.3.1 is applicable and shall be re-used.

For selecting authorization method, the procedure defined in clause 6.X.2 shall be followed. The API invoker shall include the supported RNAA authorization flows in the Security Method Request. In case where CCF-B is not in possession of the RNAA authorization flow, CCF-B sends the request to CCF-A to either get the supported list of RNAA authorization flows or to get a selected RNAA authorization flows. The CCF-A shall provide either the list of supported RNAA authorization flows or the RNAA authorization flows to the API invoker via CCF-B. The CCF-A/B shall determine the RNAA authorization flow based on the RNAA capabilities of the CCF-A, AEF, and API invoker. The API invoker shall use the determined RNAA authorization flow in the subsequent communication with the CCF-A and AEF.

For revocation, the CCF-A shall initiate the revocation and procedure as defined in clause 6.5.3.4 can be re-used. The CCF-A via CCF-B notifies the the API invoker after token revocation.

* * * End of Changes * * * *