# 3GPP TSG-SA3 Meeting #122																			S3-252043

# Fukuoka, Japan, 19 – 23 May 2025

Source:	Samsung, Johns Hopkins University Applied Physics Lab, Philips, Intel, Interdigital

Title:	DoS attack remediation in S&F operation

Document for:	Approval

Agenda item:	4.19

Work Item:	5GSAT_Ph3_SEC

Comments

This contribution proposes to give implementation options to address DoS attacks specific for S&F operation in a split MME architecture.

* * * First Change * * * *

# X.2	Security aspects of Split MME architecture

Mutual authentication between the UE and the network in a split MME architecture can involve more than one satellite (i.e., more than one MME-onboard), in which case the ground segment of the network is responsible for the selection and provisioning of MME-onboard the same, or another satellite, with the necessary information (e.g., Authentication Vector) to perform or finish an authentication procedure. The MME on-board obtains the EPS authentication vectors when the feeder link is available, and stores the authentication vectors when the service link is unavailable.

The NAS security shall be terminated on the MME-onboard. The ground segment of the network shall ensure that the latest NAS security context of the UE, or an Authentication Vector, is available at the MME-onboard.

NOTE: 	The distribution and synchronization of the latest NAS security context between the MME-ground and MME-onboard is out of the scope of 3GPP.

## X.2.1	(D)DoS attacks and remediation options

### X.2.1.1 Attack Description

In the initial uplink control plane phase (e.g., during an Attach Request prior to authentication), 3GPP systems operating onboard satellites are unable to verify the authenticity or integrity of transmitted data due to the Store and Forward (S&F) operational mode. During periods of feeder link unavailability, uplink data is stored onboard the satellite, which can enable storage exhaustion attacks via over-the-air malicious data transmissions

For instance, multiple authorized and/or unauthorized UEs can transmit spurious Attach Requests, resulting in the depletion of the satellite's storage resources and leading to a Denial-of-Service (DoS) condition within the MME-onboard.

Furthermore, a single attacker or a coordinated group can conduct a Distributed Denial-of-Service (DDoS) attack targeting individual satellites or entire constellations.

During the pre-authentication phase, downlink Non-Access Stratum (NAS) signalling messages are not integrity-protected, thus limiting UE’s ability to verify the authenticity of received messages, and potentially rendering them vulnerable to DoS attacks through the acceptance of illegitimate network instructions.

Clause X.2.1.2 specifies implementation options intended to mitigate the aforementioned DoS threats.

### X.2.1.2 Implementation options for (D)DoS Attack Mitigation

Several potential mitigation approaches are identified for addressing (D)DoS attacks during periods of feeder link unavailability. Suppliers and operators can optionally implement one or a combination of the following categories in accordance with their deployment scenarios, risk tolerances, and operational policies:

### X.2.1.2.1 Compliance-based Mitigation Solution

In this approach, the UEs are assumed to be following the standardized procedures. The examples of compliance-based mitigation include a back-off timer and a wait timer as specified in TS 23.401 clause 4.13.

### X.2.1.2.2 Pre-Authentication Mitigation Solution

In this approach, UEs and satellites either use a key derived from the permanent key or are provisioned with additional credentials to authenticate initial Attach Request.

The key or credentials include one of the followings:

- Credentials for Elliptic Curve Certificate-less Signatures for Identity-based encryption (ECCSI), as defined in RFC 6507 [xx].

- Shared key derived from the long-term key

During operation in S&F mode, when the feeder link is unavailable, the satellite provides an authentication parameter alongside the S&F operation indication.

- The UE transmits an Attach Request message with an authentication parameter.

- Upon successful verification of the Attach Request message, the satellite stores elements of the request message. Additionally, the satellite transmits an authenticatable Attach Reject message.

### X.2.1.2.3 Rate-Limiting Mitigation Solution

This type of solution slows the rate of attach request messages from all UEs. However, in case that attackers are sending a higher-rate of attach request messages as compared to legitimate UEs, this type of solution would disproportionately slow attackers compared to legitimate UEs. Note that the full effect of this solution depends on attacker UE compliance to puzzle solving requests. Compromized attacker UEs can ignore the puzzle solving requests and thereby proceed with higher-rate attach messages. Compliant satellite eNBs can reject unsolved puzzle responses, thereby protecting some of its memory resources, but are still subject to receiving high-rates of false puzzle responses. While this does not entirely prevent (D)DoS attacks on satellites or UEs, it reduces the attack potential without requiring additional credential derivation or provisioning. Instead, it relies on method-specific parameters to be available, provisioned, and used in UEs. Use of cryptographic puzzles when the feeder link is unavailable to slow down the rate of the (D)DOS attack is an example of a rate-limiting approach. For such an approach to work, the satellite creates a cryptographic puzzle for each UE requesting the service and sends it to the UE. The UE, in turn, solves the puzzle and sends the evidence of successful puzzle solving to the eNodeB on board the satellite. Upon successful matching of the expected evidence with the received evidence, the satellite allows the Attach request to proceed to the MME-onboard.

NOTE: Not every UE is assumed to be capable or provisioned with method-specific parameters; in these situations, the operator’s policy determines whether to grant limited service access, enforce additional checks, or deny service to non-compliant UEs.

* * * End of Changes * * * *