# 3GPP TSG-SA3 Meeting #122																			S3-252071

# Fukuoka, Japan, 19 – 23 May 2025

Source:	Xiaomi

Title:	Security Handling for inter-CU LTM

Document for:	Approval

Agenda item:	4.12

Spec:	3GPP TS 33.501

Version:	19.2.0

Work Item:	NR_Mob_Ph4_Sec

Comments

Based on the conclusion in S3-245164 and the changes in R3-252517 and R3-252520 agreed by RAN3, this pCR proposes the normative text to the draftCR for security handling for inter-CU LTM.

* * * First Change * * * *

6.X	Security handling for inter-CU LTM

6.X.1	When DC is not configured

For the case where CU is acting as MN and DC is not configured as specified in TS 38.300 [52], the UE security context is configured on the involved gNBs via LTM Configuration Update procedure, during which the {KNG-RAN*, NCC} pair, UE’s 5G security capabilities and UE’s UP security policy are sent by the initial/serving gNB to the candidate gNBs in LTM CONFIGURATION UPDATE message, and the selection AS security algorithms and UE’s UP security activation status determined by the candidate gNBs are returned to the initial/serving gNB in LTM CONFIGURATION UPDATE ACKNOWLEDGE message.

The selected AS security algorithms and UE’s UP security activation status determined by each of the involved gNBs are sent together to the UE in the RRCReconfiguration message via the initial/serving gNB.

During LTM execution phase, the source gNB-DU includes the NCC used for the derivation of KNG-RAN* in the Cell Switch Command MAC CE to the UE to avoid any de-synchronization issue due to NCC mismatch.

If there is any mismatch between the UE security context received from the source gNB and the UE security context received in the PATH SWITCH REQUEST ACKNOWLEDGE message from the AMF, the target gNB shall initiate an intra-gNB-CU handover to change the current KgNB or reconfigure the reselected algorithms or reconfigure the DRBs to activate or de-activate the UP integrity/confidentiality as per the received policy from SMF, via the RRC Connection Reconfiguration procedure.

Meanwhile, the target gNB shall update the UE’s 5G security capabilities and/or UE’s UP security policy for the active PDU session to all candidate gNBs via LTM Configuration Update procedure. The reselected AS security algorithms and/or updated UE’s UP security activation status determined by each of the candidate gNBs are sent together to the UE in the RRCReconfiguration message via the target gNB.

6.X.2	When DC is configured

In the case where CU is acting as SN and MN is unchanged as specified in TS 37.340 [51], the security handling as specified in clause 6.10.2.4 of this specification for SCPAC shall be applied.

* * * End of Change * * * *