# 3GPP TSG-SA3 Meeting #122																			S3-252072

# Fukuoka, Japan, 19 – 23 May 2025

Source:	Xiaomi

Title:	Editor’s Note Resolution for Privacy Protection

Document for:	Approval

Agenda item:	4.21

Spec:	3GPP TS 33.434

Version:	18.2.0

Work Item:	Metaverse_Sec

Comments

In the current living document for Metavers_Sec, there is an editor’s note for privacy protection for user information exposure as follows:

Editor’s Note: whether CAPIF RNAA supports the case where the resource owner is different from the user of the application invoker is to be aligned with CAPIF_Ph3-Sec.

Based on the progress on CAPIF_Ph3-Sec, what can be authenticated in RNAA procedure are the API invoker and the resource owner. Hence what are included in the RNAA access token are API invoker ID and owner ID. The access token is used by the exposure function to authorize whether the resource/information of the owner can be exposed to the API invoker or not.

However, the user behind the API invoker is not considered in CAPIF_Ph3-Sec. It means that there is no defined method for CAPIF RNAA to authenticate the user behind the API invoker. Hence the identifier of the user behind the API invoker is not included in the RNAA access token. In lack of user information in the access token, the exposure function is not able to authorize a user different from the owner for user/owner information exposure.

Therefore, it is proposed to delete the editor’s note and keep the existing note on the applicability restriction of the procedures.

* * * First Change * * * *

6.Z	Privacy protection for user information exposure

When CAPIF is used as specified in TS 23.434 [2], during exposure of user specific information (e.g. user identity, user location) in localized mobile metaverse services through the application enabler layer, the RNAA framework defined in TS 33.122 [29] is used for for privacy protection of user information. The user information is only exposed if CCF obtains permission from the resource owner as specified in TS 33.122 [29].

When CAPIF is not used, the user information is only exposed if SIM-S obtains authorization decision from the VAL user after the SIM-S authenticates the VAL user based on OpenID connection (OIDC) procedure referred in TS 33.434 [4].

NOTE X: the above procedures are only applicable to the case where the resource owner is same to the VAL user.

Editor’s Note: whether CAPIF RNAA supports service operation level and resource level granularity in RNAA is to be aligned with CAPIF_Ph3-Sec.

* * * End of Change * * * *