3GPP TSG-SA3 Meeting #122	S3-252083

# Fukuoka, Japan, 19 – 23 May 2025

Source:	Xiaomi

Title:	Authentication procedure for AIoT service

Document for:	Approval

Agenda item:	4.26

Spec:	3GPP TS 33.369

Version:	0.1.0

Work Item:	AmbientIoT-SEC

Comments

Based on the conclusion in TR 33.713, it is proposed to define the authentication procedure for operator-managed AIoT device.

* * * First Change * * * *

5.2	Authentication procedure

The authentication procedure may be performed in the Inventory-only procedure for one-way authentication (i.e., the 5G network authenticates the operator-managed AIoT device), and in the Inventory-and-Command procedure for mutual authentication. The authentication procedure for operator-managed AIoT device is described below in Figure 5.2-1.

Figure 5.2-1: Authentication procedure for operator-managed AIoT device

For the Inventory-only procedure:

- Based on the Service request received from AF, the AIOTF performs the Reader selection and constructs the Inventory request. For the operator-managed AIoT device, the AIoTF may generate a Nonce 1 and include it in the Inventory request. The Reader responds to the AIOTF whether the Inventory operation can be performed.

- The Reader executes the Inventory operation by sending the paging message.

- If an AIoT device matches the AIoT Device Identification information in the paging message and the Nonce 1 is included in the Inventory request, the AIoT device generates a Nonce 2 and calculates the Auth_Value and session key. The Auth_Value is calculated by using the authentication function with the input of Nonce 1, Nonce 2, and the AIoT device credential. The session key is derived by using KDF with the input of Nonce 1, Nonce 2, and the AIoT device credential. The session key is temporarily stored in the AIoT device. The AIoT device may include AIoT device ID, Nonce 2, and Auth_Value in the AIoT Device NAS message and respond to the paging message.

- The Reader sends the Inventory report to the AIOTF, including the aggregated AIoT Device NAS message(s).

- For the received Inventory report message, the AIOTF may send the Authentication and Key request to the ADM, which manages the AIoT device credential(s). The Authentication request includes the Nonce 1 generated by AIOTF, one or multiple sets of <AIoT device ID, Nonce 2> obtained from the AIoT Device NAS message(s).

- For each received AIoT device ID, the ADM retrieves the corresponding credential and calculates the Auth_XValue and session key in the same manner as the AIoT device. The ADM may return one or multiple sets of <AIoT device ID, Auth_XValue, session key> to the AIOTF.

- For each discovered AIoT device, the AIOTF compares Auth_Value and Auth_XValue and determines whether its authentication is successful.

For the Inventory-and-Command procedure:

- If the authentication is successful, the AIOTF may protect the NAS command (e.g., read, write, or disable) by using the session key associated with the AIoT device ID. If the integrity protection needs to be activated, the AIOTF calculates the MAC for the NAS command. The AIOTF sends the Command request to the Reader directly or indirectly. If the NAS command is for a group of AIoT devices (e.g., disable), the AIOTF aggregates the protected NAS command request in the Command request to the Reader.

- The Reader sends the Command request to the selected AIoT device.

- If the MAC is included in the received Command request, the AIoT device verifies the integrity of NAS command by using the stored session key. If the verification is passed, the AIoT device determines that the authentication for network is successful.

* * * End of Changes * * * *