3GPP TSG-SA3 Meeting #122	S3-252085

# Fukuoka, Japan, 19 – 23 May 2025

Source:	Xiaomi

Title:	Remove Editor’s Notes in solution 16 of TR 33.713

Document for:	Approval

Agenda item:	5.9

Spec:	3GPP TS 33.713

Version:	0.8.0

Work Item:	FS_Ambient_IoT_Sec

Comments

It is proposed to remove Editor’s Notes in solution 16 of TR 33.713.

Editor’s Note:	Whether the Disabling acknowledgement message is needed is FFS.

Based on clause 6.2.3 of TS 23.369, the command response message from AIoT device to Reader is mandatory and the command response message from Reader to AIoTF is optional. It is proposed to remove this Editor’s Notes and add further clarification in step 6.

Editor’s Note: 	Alignment with conclusion from TR 23.700-13 [4] is FFS.

It is proposed to change this Editor’s Note to NOTE: Alignment with conclusion from TR 23.700-13 [4] is not addressed in the present document.

Editor’s Note: 	Further evaluation is FFS.

No further evaluation is expected. Therefore, it is proposed to remove this Editor’s Note.

* * * First Change * * * *

6.16	Solution #16: Disabling operation procedure for Ambient IoT services

6.16.1	Introduction

This solution is proposed to address Key Issue #1, which provides a method for permanently disabling the Ambient IoT device(s). This solution applies to Topology 1 and Topology 2.

Considering different roles are involved in the Ambient IoT services, it is assumed that only the owner of Ambient IoT device can authenticate the Ambient IoT device and initiate the disabling operation procedure to disable the Ambient IoT device’s capability to transmit RF signals.

In this solution, it is assumed that the Ambient IoT device is stateful. After the authentication is performed successfully, the Ambient IoT device enters the next state (e.g. secure state), in which it can execute the disabling operation.

For disabling the Ambient IoT device, the owner sends the Disabling request to the Ambient IoT device via the gNB/Intermediate node. If the owner is the application provider, the Disabling request is E2E protected between the AF and Ambient IoT device. If the owner is the operator, the Disabling request is protected by the AIoTF/AMF.

Once receiving the Disabling command, the Ambient IoT device determines whether to execute this operation based on authentication status and verification result. By maintaining the authentication status, the Ambient IoT device can ensure that the Disabling command is sent from the legitimate requester instead of the attacker. By maintaining the security material, the Ambient IoT device can ensure that the Disabling command is not tampered by the attacker.

NOTE 1: The states of Ambient IoT device are defined in described in solutions (e.g. #15, #16, #33, etc.) of TR 23.700-13 [4], which are maintained in the NVM of Ambient IoT device.

6.16.2	Solution details

This solution assumes that the authentication between the Ambient IoT device and its owner has been performed before disabling operation and the authentication status is stored in the Ambient IoT device. It is also assumed that the Ambient IoT device already share the security materials for communication protection with the AF or AIoTF/AMF before disabling operation, which can be dedicated for disabling operation, or be generalized for all command procedure (i.e. read, write, disable).

NOTE 2:	The authentication and security context establishment between the Ambient IoT device and its owner is out of scope of this solution.

Figure 6.16.2-1: Disabling operation procedure for Ambient IoT services

- The device owner sends the Disabling request to AIoTF/AMF. The Disabling request may include Ambient IoT device ID(s), disabling command, etc. If the owner of Ambient IoT device is application provider, the disabling command is E2E protected between the AF and Ambient IoT device.

2.	The AIoTF/AMF determines the gNB/Intermediate node to transmit this Disabling request.

3.	If the owner of Ambient IoT device is operator, the AIoTF/AMF protects the disabling command using the shared security materials. The AIoTF/AMF sends the Disabling request to the selected gNB/Intermediate node.

4.	Upon reception of the request message, the selected gNB/Intermediate node executes inventory procedure with the Ambient IoT device.

5. 	The selected gNB/Intermediate node sends the protected Disabling Command message to the Ambient IoT device.

6.	Once receiving the Disabling command, the Ambient IoT device firstly determines whether the authentication has been performed based on the authentication status. If the requester has been authenticated, the Ambient IoT device further verifies the received Disabling request by using the shared security materials. If the verification is successful, the Ambient IoT device can execute the disabling operation and return the command response message to the Reader.

NOTE 1:	Once the disabling operation is finished, the owner removes the security context of Ambient IoT device.

NOTE 2:	Alignment with conclusion from TR 23.700-13 [4] is not addressed in the present document

6.16.3	Evaluation

This solution addresses the requirement of Key Issue #1 and applies to both Topology 1 and Topology 2.

This solution ensures that the disabling operation can only be performed after checking the authentication status and verification result. If the Ambient IoT device has not been authenticated by the network or the verification of disabling request is failed, the Ambient IoT device discards the disabling request.

In this solution, the states and security materials shared with AF or AMF/AIoTF are required to be maintained by the Ambient IoT device.

This solution assumes that the Ambient IoT device is stateful. The states are maintained in NVM of Ambient IoT device.

* * * End of Changes * * * *