3GPP TSG-SA3 Meeting #122 	S3-252101

# Fukuoka, Japan, 19 - 23 May 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 2138 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removal of the terminology “AS root key” |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple, NTT DOCOMO, Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-5-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There is a terminology issue in clause 6.10 which is misleading and confusing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - the removal of the termonology “AS root key”. Clause 6.10 uses “AS root key” as the KgNB or KNG-RAN. However, there is no “AS root key” defined in TS 33.501. During handover, it may cause the misunderstanding that “AS root key” refers to the initial key derived from KAMF. Based on the context of clause 6.10.2.4.2 and 6.10.3.1, the AS root key is referring to the current KgNB or KNG-RAN. - the change of an incorrect reference. In the last paragraph of Clause 6.10.3.1, the reference to 6.7.3.3 is not correct. Clause 6.7.3.3 is only on the algorithm change, while 6.10.3.1 context is illustrating the refresh of KNG-RAN, which is captured in clause 6.9.2.3.1. In summary, this CR is to clarify the description of the procedures but not the actual details of the procedures. It will be based on SA3 decision on what is the earliest release to update. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusing description. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.10.2.4.2 and 6.10.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START OF 1st CHANGES ***

6.10.2.4.2	Security context initialization for selective SCPAC

To prevent key-stream reuse when the UE switches back and forth to the same PSCell or SN, the MN shall assign a sequence of distinct SN Counter values (maintained for dual connectivity detailed in clause 6.10.3.1 of this document) per candidate SN during the SCPAC procedure. The same SN Counter as used for DC shall be used to generate the keys also for SCPAC and the MN shall ensure that no generated SN Counter value will accidentally be used to derive a KSN more than once. Each SN Counter value is unique, and the sequences (i.e. sequences of SN Counter values of candidate SNs) are non-overlapping. These sequence s shall be provided to the UE by the MN. The UE shall store these sequences.

The MN shall derive the KSN keys corresponding to the SN Counter values from the KgNB of the UE as described in Annex A.16. The MN shall send the KSN keys associated with the SN together with their corresponding SN Counter values to that SN in the SN Addition Request. The SN shall store the received KSN keys and the SN Counter values of the UE. The MN shall maintain the largest assigned SN Counter value and monotonically increment it either for the next KSN calculation for DC as described in clause 6.10.3.1 of this document or for further assignment for the SCPAC detailed in this clause.

When a new KgNB in the associated 5G AS security context of the UE is established, and the SN Counter is set to ‘0’ as specified in clause 6.10.3.1, the MN derives a new sequence of distinct SN Counter values per candidate SN and sends these to the UE in the same RRC Reconfiguration as the one that activates the new KgNB. The UE shall delete the stored SN Counter value sequences and store the received new SN Counter values. Further, the MN derives the corresponding KSN for each target SN, and the derived KSN keys and the corresponding SN Counter values are sent to the SN from the MN. Each SN shall delete the stored KSNs and corresponding SN Counter values and store the received new KSNs and the corresponding SN Counter values.

*** END OF 1st CHANGES ***

*** START OF 2nd CHANGES ***

6.10.3	Establishing the security context between the UE and SN

6.10.3.1	SN Counter maintenance

The MN shall maintain a 16-bit counter, SN Counter, in its AS security context. The SN Counter is used when computing the KSN.

The MN maintains the value of the counter SN Counter for a duration of the current 5G AS security context between UE and MN. The UE does not need to maintain the SN Counter after it has computed the KSN since the MN provides the UE with the current SN Counter value when the UE needs to compute a new KSN.

The SN Counter is a fresh input to KSN derivation. That is, the UE assumes that the MN provides a fresh SN Counter each time and does not need to verify the freshness of the SN Counter.

NOTE: An attacker cannot, over the air modify the SN Counter and force re-use of the same SN Counter. The reason for this is that the SN Counter is delivered over the RRC connection between the MN and the UE, and this connection is both integrity protected and protected from replay.

The MN shall set the SN Counter to ‘0’ when a new KNG-RAN in the associated 5G AS security context is established. The MN shall set the SN Counter to ‘1’ after the first calculated KSN, and monotonically increment it for each additional calculated KSN. The SN Counter value '0' is used to calculate the first KSN.

If the MN decides to release the offloaded connections to the SN and later decides to re-start the offloading to the same SN, the SN Counter value shall keep increasing, thus keeping the computed KSN fresh.

The MN shall refresh the current KNG-RAN in the 5G AS security context associated with the SN Counter before the SN Counter wraps around. Refreshing the KNG-RAN and the derived keys is done using intra cell handover as described in subclause 6.7.3.3 of the present document. When the KNG-RAN is refreshed, the SN Counter is reset to '0' as defined above.

*** END OF 2nd CHANGES ***