3GPP TSG-SA3 Meeting #122	S3-252109

# Fukuoka, Japan, 19 - 23 May 2025

Source:	Huawei, HiSilicon, Nokia, Nokia Shanghai Bell

Title:	Security of Avatar Communication

Document for:	Approval

Agenda item:	4.23

Spec:	3GPP TR 33.328

Version:	18.3.0

Work Item:	NG_RTC_SEC_Ph2

Comments

Aligned with the conclusion of KI#2 in TR 33.790.

* * * First Change * * * *

Annex Y (Normative):	Securing the IMS based Avatar Communication

Y.1	General

This clause describes the security of the IMS based Avatar Communication, taking the TS 23.228[3], Annex AC.11 in account.

Y.2	Security requirements

The security requirements for the IMS based Avatar Communication are:

- The 3GPP system shall support means to ensure that stored Avatar objects and Avatar-IDs are accessed only by authenticated and authorized entities.

- The 3GPP system shall support means to ensure that the Avatar ID sent by the sending UE belongs to the UE.

- The 3GPP system shall support means for integrity and confidentiality protection for Avatar ID List downloading, avatar representation downloading and Avatar ID transferring.

Y.3	Protection of Avatar communication

The DC AS or BAR shall verify whether the Avatar ID provided by the sending UE is in the sending UE's Avatar ID list to prevent the sending UE from providing the Avatar ID belonging to other UEs.

The authorization of avatar representation downloading shall be based on the token mechanism. The token shall include Avatar ID, UE ID of the sending UE, MF ID (or DC AS ID), and BAR ID. The BAR shall verify the token as follows:

- The BAR shall verify the signature of the token.

- The BAR shall verify whether the Avatar ID in the Avatar representation downloading request matches that in the token.

- The BAR shall verify whether the UE ID of the sending UE in the Avatar representation downloading request matches that in the token.

- The BAR shall check whether MF ID or DC AS ID in the token matches the entity sending the Avatar representation downloading request.

- The BAR shall check whether the BAR ID matches itself.

- If the verification is successful, the BAR shall send sending UE's Avatar representation to the MF or DC AS.

The e2DCe or e2e media plane protection mechanism in clause 5 shall be reused for authentication, integrity and confidentiality protection between UE and MF, UE and DC AS for Avatar ID list downloading and avatar representation downloading, Avatar ID transfer via BDC/ADC. Security specification in Annex P.1 shall be reused for security of aspects of SBA in IMS media control interface, to protect SBA communication between MF/DCSF and DC AS/BAR. Security specification in TS 33.501 [11], clause 12 shall be reused for protection of the NEF – AF interface, to protect communication between DCSF and DC AS if the DC AS is deployed outside of operator’s domain.