3GPP TSG-SA3 Meeting #122	S3-252113

# Fukuoka, Japan, 19 - 23 May 2025

Source:	Huawei, HiSilicon

Title:	Addressing EN for Data Channel Exposure

Document for:	Approval

Agenda item:	4.23

Spec:	3GPP TR 33.328

Version:	18.3.0

Work Item:	NG_RTC_SEC_Ph2

Comments

In order to be aligned with the conclusion of KI#3 in TR 33.790, the following Editor's Notes in both Clause X.1 and X.3 can be removed:

Editor's Note: More content is FFS.

* * * First Change * * * *

Annex X (Normative):	Security and privacy for IMS capability exposure

X.1	General

This clause specifies the security and privacy for IMS capability exposure including two parts. One covers the security of the IMS (DC or non-DC) event exposure and the other covers the security of the IMS DC control procedures.

TS 23.228 [3] and the TS 23.502 [XX] has specified the IMS Subscribe/Notify Framework Architecture as well as the DC session control handling procedures.

X.2	Security requirements

The security requirements for the security and privacy of IMS capability exposure are:

- The 5G system shall support authentication and authorization of data channel application server during the IMS capability exposure procedures.

X.3	Procedures

Existing security specification in TS 33.501 [47], clause 12 for the NEF-AF interface shall be re-used to perform mutual authentication and authorization of NEF and AF, and the interface protection.