3GPP TSG-SA3 Meeting #122	S3-252115

Fukuoka, Japan, 19 - 23 May 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.328 | CR | 0081 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Changing DC to Data Channel |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_SEC |  |  |  |  |  | Date: |  |  | 2025-05-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In order to avoid confusion with the commonly used DC abbreviation for dual connectivity and since the abbreviation DC for Data Channel is not defined, the proposal is to spell out all occurences of "DC" in the context of NG RTC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | changing "DC" to "Data Channel" |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusing abbreviation "DC" |  |  |  |  |  |  |  |  |
| Clauses affected: |  | N.3.1, N.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S3-252115 |  |  |  |  |  |  |  |  |

**************** START of 1st CHANGE****************

N.3.1	General

This clause describes how end-to-access-edge (e2ae) and end-to-end (e2e) security is achieved for WebRTC Data Channels (see IETF RFC 8831 [50], IETF RFC 8841 [48], and IETF RFC 8864 [51]). In addition the end-to-Data Channel-edge (e2DCe) security is specified. The end-to-Data Channel-edge (e2DCe) is defined to be the path between the UE and MF.

WebRTC-compatible browsers use SCTP over DTLS as transport protocol for peer-to-peer data. A WebRTC Data Channel is defined as two unidirectional SCTP streams, one in each direction, which are managed together as a single entity (see IETF RFC 8831 [50]). The application protocol which runs on top of the WebRTC Data Channel is not specified and the JavaScript is free to implement any protocol it requires.

The application protocols that a WebRTC IMS Client may need to support are MSRP, BFCP, T.140, and T.38. A DCMTSI IMS Client [35] also supports the unspecified application protocol approach used by WebRTC Data Channel with, for example, a related web page and JavaScript handling the needed transmission and protocol format actions.

Figure N.3.1-1 shows the common protocol stack and the required protocol translation for a WebRTC IMS Client, where the WebRTC Data Channel stack is not used in IMS core network or towards the peer. The transport protocol that the IMS-AGW applies on the remote side (marked X in the figure) depends on the application protocol. For MSRP and BFCP X=TCP, for T.140 X=RTP/UDP, and for T.38 X=UDPTL/UDP. In general, the IMS-AGW will forward the application protocol messages transparently. The only exception is MSRP messages which contain IP address information and therefore needs to be re-written by the IMS-AGW. This can however be avoided if both endpoint support the MSRP CEMA extension [24].

T.140 (real-time text) and T.38 (fax) are included here for sake of completeness. These are legacy protocols and are not expected to be commonly used.

Editor’s Note: The final list of supported application protocols (e.g., MSRP, BFCP, T.140, and T.38) is to be decided by CT groups.

Figure N.3.1-1: Protocol stack for WebRTC IMS Client (WIC) Data Channels

Figures N.3.1-2 and N.3.1-3 are based on TS 23.228 [3] and show two examples of the protocol stack used by DCMTSI [35] Clients. Figure N3.1-2 shows a protocol stack for the HTTP proxy configuration mode for the Bootstrap Data Channel or Application Data Channel in case MF is anchoring DTLS.

Figure N.3.1-2: Protocol stack for IMS bootstrap data channels (TS 23.228[3])

Figure N.3.1-3 shows protocol stack for the UDP proxy configuration mode for the Application Data Channel case, providing a Person2Application/Application2Person/Person2Person Data Channel Application.

Figure N.3.1-3: Protocol stack for P2P/P2A/A2P IMS application data channels without MF (TS 23.228[3])

**************** START of 2nd CHANGE****************

N.3.4	e2e security for IMS data channels

E2e security for IMS Data Channels for DCMTSI clients is achieved in a similar way as for e2DCe security. See clause N.3.3. In both cases, certificate fingerprints need to be exchanged over SDP between the peers and certificates are exchanged through DTLS handshake in-band on the media path. The media may be anchored in IMS by inserting a gateway on the media path. To ensure the integrity of the certificate fingerprint, the signalling path is assumed to be protected.

Figure N.3.4-1: E2E protection of IMS Data Channels

NOTE:	SDP is not exchanged between the S-CSCF and the Data Channel Application Server. The DC3 and DC4 interfaces are not specified in TS 23.228[3].

**************** END of CHANGE****************