3GPP TSG-SA3 Meeting #122	S3-252121

# Fukuoka, Japan, 19 - 23 May 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 2139 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addressing the EN on the concrete modes of AES for protecting XRM Media related information when using connect-UDP forwarded mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2-Sec |  |  |  |  |  | Date: |  |  | 2025-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| 5 |  |  |  |  |  |  |  |  |  |  |
| Reason for change: |  | Some editor’s notes remain, and further detailing is needed for Stage 3 to complete the work. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Resolution of editor’s notes and additional details on the choice of crypto algorithm when using forwarded mode. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No protection of the MRI information. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 18.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**************** START of CHANGE****************

18.2.4	Security parameters to be used in the Forwarded mode

When the Forwarded mode is used, Media related information (MRI) is encrypted and integrity protected using a separate security context from that being used in the QUIC tunnel between AS and UPF.

Media related information is encrypted and integrity protected using AES in GCM mode or AES in CCM mode. These modes are preconfigured in the UPF and AS, along with the certificates or any other security credential parameters required to set up QUIC. Therefore, the choice of AES mode is determined by the configuration, except for key derivation.

AES-CCM is used for MRI protection in Forwarded Mode as it supports 64-bit authentication tags. A 64-bit counter-based nonce shall be used with replay protection implemented. For MRI protection in Forwarded Mode requiring enhanced security, AES-GCM shall be used with 96-bit authentication tags and a 96-bit nonce.

Unique keys shall be used per direction. In the present document data is only transmitted in one direction is used. The encryption key is derived as described in clause 18.2.5. Nonce is created as described in clause 18.2.6.

**************** END of CHANGE****************