3GPP TSG-SA3 Meeting #122	S3-252125

Fukuoka, Japan  19 – 23 May 2025

Source:	OPPO

Title:	PCR on Protection of information during AIoT service communication

Document for:	Approval

Agenda item:	4.26

Spec:	3GPP TS 33.369

Version:	0.1.0

Work Item:	AmbientIoT-SEC

Comments

This pCR proposes to add the information protection procedures during AIoT service communication.

Proposed Changes

* * * Start of Change * * * *

5.3	Protection of information during AIoT service communication

5.3.X	General

This clause describes the security procedures on the information protection in command message. The AIOTF acts as the network layer security termination point for information protection. The specified device security capabilities are pre-configured/known in the network side in advance.

5.3.Y	Security procedures for information protection based on the session key

Figure 5.3.2-1 describes the security procedures for information of command message protection based on the session key.

Figure 5.3.Y-1: Security procedures for information protection based on the session key

1-10, Step 1-10 of clause 6.2.3 Procedure for Command in TS 23.369 [2].

- The AIOTF retrieves the Authentication data from the ADM.

- The ADM derives the session key KAIoTF.

- The ADM sends the Authentication data response including the KAIoTF and security capabilities of the AIoT device.

- The AIOTF performs the following security processing operations:

- AIOTF encrypts the command using the session key KAIoTF and direction (e.g., 0 for uplink AIoT NAS message, 1 for downlink AIoT NAS message) as the input of the pre-configured encryption algorithm.

- AIOTF compute the MAC1 using the session key KAIoTF, direction and Command as the input of the pre-configured integrity algorithm.

- AIOTF sends the AIoT Command Request message to the AIoT device via AIoT RAN, including Command and MAC1.

- The AIoT device performs the following security processing operations:

- AIoT device derives the session keys KAIoTF using the same method as ADM.

- AIoT device verifies the MAC1 and decrypts the Command using same methods as AIOTF.

- AIoT device encrypts the AIoT data and computes the MAC2 using the same methods as AIOTF.

- AIoT device sends the AIoT Command Response message to the AIOTF via AIoT RAN, including MAC2 and AIoT data.

- The AIOTF verifies MAC2 and decrypts the AIoT data.

- Step 16-17 of clause 6.2.3 Procedure for Command in TS 23.369 [2].

* * * End of Change * * * *