3GPP TSG-SA3 Meeting #122																			S3-252159

Fukuoka, Japan, 19 – 23 May 2025

Source:	Qualcomm Incorporated

Title:	Protection of AIoT device identifier privacy

Document for:	Approval

Agenda item:	4.26

Spec:	3GPP TS 33.369

Version:	0.1.0

Work Item:	AmbientIoT-SEC

Comments

Based on the analysis described in S3-252158, this contribution proposes to specify the derived temporary ID based privacy protection mechanisms in clause 5.4.

Proposed Changes

# * * * * First change * * * *

## 5.4	Protection of AIoT device identifier privacy

### 5.4.1	General

This clause describes the mechanism to protect AIoT device identifier privacy during the inventory procedure based on a temporary ID (T-ID). The T-ID is generated based on the key (i.e., KAIoTF) shared between AIoT device and AIoTF.

### 5.4.2	Privacy protection of AIoT device ID in inventory with AIoT device ID

Figure 5.4.2-1:  Privacy protection of AIoT device ID in inventory with AIoT device ID

1.	The AF initiates Inventory as specified in clause 6.2.2 of TS 23.369 [2]. The AIoT_Inventory request contains the AIoT device ID.

2.	The AIoTF shall retrieve the KAIoTF for the AIoT device ID.

If the KAIoTF is not available locally, the AIoTF shall perform a key bootstrapping procedure to establish the KAIoTF.

Editor’s Note: One key bootstrapping mechanism is described in different contribution (S3-25aaaa-g).

The AIoTF shall compute a T-ID using the KAIoTF , the AIoT device ID and a randomly generated nonce, Noncenetwork as specified in clause B.D. Then, the AIoTF shall include the T-ID and Noncenetwork in an Inventory Request and send the Inventory Request to the NG-RAN.

3.	The NG-RAN shall send a Paging message containing the T-ID and Noncenetwork.

4.	Upon receiving the Inventory request, the AIoT device shall compute a T-ID using the KAIoTF, its AIoT device ID and Noncenetwork as specified in clause B.D. If the computed T-ID matches with the received T-ID, the AIoT device shall compute a T-ID’ using the KAIoTF, its AIoT device ID, the received Noncenetwork and a randomly generated nonce, Noncedevice as specified in clause B.E. If the computed T-ID does not match with the received T-ID, the AIoT device shall stop further processing of the Inventory Request, i.e., the rest of the steps are not performed.

5.	The AIoT device shall include the T-ID’ and Noncedevice in a D2R message and send it to the NG-RAN.

6.	The NG-RAN shall send the Inventory Report containing the T-ID’ and Noncedevice to the AIoTF.

7.	Upon receiving the Inventory response, the AIoTF shall compute a T-ID’ as specified in clause B.E. If the computed T-ID’ matches with the received T-ID’, the AIoTF shall construct an AIoT_Inventory Notify including the AIoT device ID that corresponds to the received T-ID’.

8.	The AIoTF shall send the AIoT_Inventory Notify to the AF as specified in clause 6.2.2 of TS 23.369 [2].

### 5.4.3	Privacy protection of AIoT device ID in inventory with filtering information

Figure 5.4.3-1:  Privacy protection of AIoT device ID in inventory with filtering information

1.	The AF initiates Inventory as specified in clause 6.2.2 of TS 23.369 [2]. The AIoT_Inventory request shall contain the filtering information.

2.	The AIoTF shall retrieve AIoT device IDs that match with the filtering information and the corresponding keys (i.e., KAIoTF). Then, the AIoTF shall construct the list of T-IDs for the AIoT device IDs as specified in clause B.D.

NOTE: Construction of the list can happen later, but before step 8.

3.	The AIoTF shall include the filtering information and Noncenetwork in an Inventory Request and send the Inventory Request to the NG-RAN.

4.	The NG-RAN shall send a Paging message containing the filtering information and Noncenetwork.

5.	Upon receiving the Inventory request, the AIoT device shall check if the filtering information matches with the configured filtering information. If they match, the AIoT device shall compute a T-ID using KAIoTF , its AIoT device ID and Noncenetwork as specified in clause B.D. If they do not match, the AIoT device shall stop further processing of the Inventory Request, i.e., the rest of the steps are not performed.

Editor’s Note: the format of the configured filtering information is FFS.

6.	The AIoT device shall include the T-ID in an D2R message and send it to the NG-RAN.

7.	The NG-RAN shall send an Inventory Report containing the T-ID to the AIoTF.

8.	Upon receiving the Inventory response, the AIoTF shall find a matching T-ID from the T-ID list. If there is a match, the AIoTF shall construct an AIoT_Inventory Notify including the AIoT device ID that corresponds to the received T-ID.

9.	The AIoTF shall send the AIoT_Inventory Notify to the AF as specified in clause 6.2.2 of TS 23.369 [2].

# * * * * Second change * * * *

# B.D	Temporary ID generation scheme 1

When deriving the temporary ID, the following parameters are used to form the input S to the KDF:

-	FC = 0xDD;

-	P0 = AIoT device ID;

-	L0 = length of AIoT device ID;

-	P1 = Noncenetwork;

-	L1 = length of Noncenetwork;

If the temporary ID generation is used for key bootstrapping as specified in clause 5.A, the input key shall be the long-term key associated with the AIoT device ID. Otherwise the input key shall be KAIoTF.

# * * * * Third change * * * *

# B.E	Temporary ID generation scheme 2

When deriving the temporary ID, the following parameters are used to form the input S to the KDF:

-	FC = 0xEE;

-	P0 = AIoT device ID;

-	L0 = length of AIoT device ID;

-	P1 = Noncenetwork;

-	L1 = length of Noncenetwork;

-	P2 = Noncedevice;

-	L2 = length of Noncedevice;

The input key shall be KAIoTF.

# * * * * End of change * * * *