3GPP TSG-SA3 Meeting #122																			S3-252168

Fukuoka, Japan, 19 – 23 May 2025

Source:	Qualcomm Incorporated

Title:	Confidentiality and integrity protection requirements on device

Document for:	Approval

Agenda item:	4.26

Spec:	3GPP TS 33.369

Version:	0.1.0

Work Item:	AmbientIoT-SEC

Comments

It is proposed to specify confidentiality and integrity protection requirements on AIoT device in clause 4.2.1.

This contribution uses the merged contribution S3-251705 from the last meeting as a baseline. Further changes made in this contribution are as follows:

- The AIoT device supports one of the confidentiality/integrity algorithms specified in Annex D of TS 33.501.

- Support of NIA0 is removed for mandatory integrity protection.

- The AIoT device is preconfigured with the confidentiality/integrity algorithm to use.

Proposed Changes

# * * * * First change * * * *

### 4.2.1	Requirements on the device

Editor’s Note: This clause contains the security requirement on the device, including secure storage and processing of credentials.

The AIoT device shall support confidentiality protection of AIoT NAS messages between the AIoT device and the AIoTF.

Confidentiality protection of AIoT NAS messages between the AIoT device and the AIoTF is optional to use.

The AIoT device shall support integrity protection of AIoT NAS messages between the AIoT device and the AIoTF.

One of the confidentiality algorithms specified in Annex D of TS 33.501[5] i.e., NEA0, 128-NEA1, 128-NEA2, and 128-NEA3, shall be supported by the AIoT Device.

One of the integrity algorithms specified in Annex D of TS 33.501[5] i.e., 128-NIA1, 128-NIA2 and 128-NIA3 shall be supported by the AIoT Device.

The AIoT device shall be preconfigured with the confidentiality algorithm to use.

The AIoT device shall be preconfigured with the integrity algorithm to use.

# * * * * End of change * * * *