3GPP TSG-SA3 Meeting #122																			S3-252170

Fukuoka, Japan, 19 – 23 May 2025

Source:	Qualcomm Incorporated

Title:	AIoT specific key bootstrapping procedure

Document for:	Approval

Agenda item:	4.26

Spec:	3GPP TS 33.369

Version:	0.1.0

Work Item:	AmbientIoT-SEC

Comments

This contribution proposes the key bootstrapping procedure that enables the AIoTF and the AIoT device share the same key to perform inventory and command procedures multiple times. The key bootstrapping procedure reuses the inventory and command procedure.

This key bootstrapping procedure reduces the number of interactions between the AIoTF and ADM/Credential Holder. Accordingly, it reduces the latency of the inventory and command procedures, and the processing and messaging overhead incurred at the ADM/Credential Holder.

Note that Temporary ID generation scheme 1 in the third change and Temporary ID generation scheme 2 in the fourth change are used in both the key bootstrapping procedure in this contribution and the protection of AIoT device ID privacy in S3-252159.

Proposed Changes

# * * * * First change * * * *

## 5.A	Initial key bootstrapping procedure

### 5.A.1	General

This clause describes the key bootstrapping procedure that enables the AIoTF and the AIoT device share the same key to perform inventory and command procedures multiple times. The key bootstrapping procedure reuses the inventory and command procedure described in clause 5.3.2 and clause 5.4.2.

This key bootstrapping procedure reduces the number of interactions between the AIoTF and ADM/Credential Holder (CH). Accordingly, it reduces the latency of the inventory and command procedures, and the processing and messaging overhead incurred at the ADM/CH.

### 5.A.2	Initial key bootstrapping procedure

Figure 5.A.2-1:  Initial key bootstrapping procedure

1.	The bootstrapping of KAIoTF is triggered when the AIoTF does not have the key (i.e., KAIoTF) to protect the Inventory procedure with the AIoT device, requested by the AF. Alternatively, it may be triggered by the AIoTF based on configuration.

2.	The AIoTF shall send a Key request message to ADM/Credential Holder (CH) to retrieve a new KAIoTF for the AIoT device. The Key request shall contain the AIoT device ID.

3.	Upon receiving the Key request, the ADM/CH shall generate a KAIoTF using the long-term key associated with the AIoT device ID, AIoT device ID and randomly generated nonce, Noncenetwork as specified in clause B.C. The ADM/CH shall compute a T-ID using the long-term key associated with the AIoT device ID, AIoT device ID and Noncenetwork as specified in clause B.D.

4.	The ADM/CH provides the KAIoTF, T-ID and Noncenetwork to the AIoTF along with the security configurations. The security configurations shall include the configured confidentiality and integrity algorithms for the AIoT device.

5.	The AIoTF shall construct an Inventory Request containing the T-ID, Noncenetwork and a key bootstrapping indicator. The key bootstrapping indicator indicates the AIoT device to generate a new KAIoTF. The AIoTF sends the Inventory Request to NG-RAN.

NOTE 1: The T-ID is calculated using the long-term key to reduce the unnecessary computation at the unintended AIoT devices that receive the Inventory Request. Otherwise, all AIoT devices that receive the Inventory Request need to derive KAIoTF to check the T-ID.

6.	The NG-RAN shall send a Paging message containing the T-ID, Noncenetwork and the key bootstrapping indicator.

7.	Upon receiving the Paging message, the AIoT device shall compute a T-ID using its long-term key, AIoT device ID and the received Noncenetwork as specified in clause B.D. If the computed T-ID matches with the received T-ID, the AIoT device shall generate the KAIoTF based on its long-term key, AIoT device ID and the received Noncenetwork as specified in clause B.C, and perform step 8. If the computed T-ID does not match with the received T-ID, the AIoT device shall stop further processing of the Paging message, i.e., the rest of the steps are not performed.

8.	The AIoT device shall compute T-ID’ using its AIoT device ID, KAIoTF, Noncenetwork and randomly generated nonce, Noncedevice as specified in clause B.E. The AIoT device sends a D2R message containing the T-ID’ and Noncedevice, to the NG-RAN.

9.	The NG-RAN shall send the Inventory Report containing the received T-ID’ and Noncedevice.

10.	Upon receiving the Inventory Report message, the AIoTF shall compute a T-ID’ using the AIoT device ID, KAIoTF, Noncenetwork and Noncedevice as specified in clause B.E. If the computed T-ID’ matches with the received T-ID’, the AIoTF shall continue with the Command procedure as in step 11. If the computed T-ID’ does not match with the received T-ID’, the AIoTF shall stop the bootstrapping procedure, i.e., step 11 is not performed.

11.	The AIoTF shall send the protected NAS Command Request to confirm the AIoT device has derived the same KAIoTF.

12.	The AIoT device shall verify the received NAS Command Request message as specified in clause 5.3. If the verification is successful, it shall store the KAIoTF. Then, the AIoT device sends the protected NAS Command Response to the AIoTF.

13.	Upon receiving the NAS Command Response, the AIoTF shall verify the received NAS Command Response message as specified in clause 5.3. If the verification is successful, it shall store the KAIoTF. If the AIoTF does not receive the NAS Command Response for a predefined time interval, it shall reinitiate the key bootstrapping procedure when it next wants to contact this AIoT device.

NOTE 2: There can be a key mismatch between the AIoTF and AIoT device if the NAS Command Response is not received by the AIoTF. In such case, the AIoTF initiates a new key bootstrapping procedure to resolve this mismatch.

# * * * * Second change * * * *

# Annex B (normative):
Key derivation functions

# B.C	Derivation of KAIoTF

When deriving the KAIoTF from long-term key, the following parameters are used to form the input S to the KDF:

-	FC = 0xCC;

-	P0 = "KAIOTF";

-	L0 = length of "KAIOTF";

-	P1 = AIoT device ID;

-	L1 = length of AIoT device ID;

-	P2 = Noncenetwork;

-	L2 = length of Noncenetwork;

The input key KEY is long-term key associated with the AIoT device ID.

# * * * * Third change * * * *

# B.D	Temporary ID generation scheme 1

When deriving the temporary ID, the following parameters are used to form the input S to the KDF:

-	FC = 0xDD;

-	P0 = AIoT device ID;

-	L0 = length of AIoT device ID;

-	P1 = Noncenetwork;

-	L1 = length of Noncenetwork;

If the temporary ID generation is used for key bootstrapping as specified in clause 5.A, the input key shall be the long-term key associated with the AIoT device ID. Otherwise the input key shall be KAIoTF.

# * * * * Fourth change * * * *

# B.E	Temporary ID generation scheme 2

When deriving the temporary ID, the following parameters are used to form the input S to the KDF:

-	FC = 0xEE;

-	P0 = AIoT device ID;

-	L0 = length of AIoT device ID;

-	P1 = Noncenetwork;

-	L1 = length of Noncenetwork;

-	P2 = Noncedevice;

-	L2 = length of Noncedevice;

The input key shall be KAIoTF.

# * * * * End of change * * * *