# 3GPP TSG-SA3 Meeting #122																			S3-252173

# Fukuoka, Japan, 19 – 23 May 2025

Source:	Qualcomm Incorporated

Title:	Pseudo-CR on 3GPP Cryptographic Inventory Table for TLS

Document for:	Approval

Agenda item:	5.20

Spec:	3GPP TR 33.938

Version:	0.2.0

Work Item:	FS_CryptoInv

Comments

This document provides entries for tables 4.2-1 and 4.3-1 corresponding to clause 4.4.2 TLS.

The profiles of TLS 1.2 and TLS 1.3 are presented separately in TS 33.210 and are different from each other: clause 6.2.2 provides the profile of TLS 1.3 and clause 6.2.3 provides the profile of TLS 1.2. Accordingly, we have created separate table entries of TLS 1.2 and TLS 1.3. Here is a brief analysis of how the table entries were determined.

For TLS 1.2, RFC 5246 clause 9 (pp. 65) says “In the absence of an application profile standard specifying otherwise, a TLS-compliant application MUST implement the cipher suite TLS_RSA_WITH_AES_128_CBC_SHA (see Appendix A.5 for the definition).”

However, since TS 33.210 clause 6.2.3 says “Only cipher suites with AEAD (e.g. GCM) and PFS (i.e., ECDHE) shall be supported.”, the cipher suite TLS_RSA_WITH_AES_128_CBC_SHA is not supported in 3GPP.

Other cipher suites mentioned (either mandatory or recommended) in TS 33.210 clause 6.2.3 are:

- TLS_ECDHE_ECDSA_WITH_AES_128_GCM_SHA256

- TLS_DHE_RSA_WITH_AES_128_GCM_SHA256

- TLS_ECDHE_RSA_WITH_AES_128_GCM_SHA256

- TLS_ECDHE_ECDSA_WITH_AES_256_GCM_SHA384

- TLS_ECDHE_RSA_WITH_AES_256_GCM_SHA384

However, since TS 33.210 clause 6.2.3 says “Finite field Diffie-Hellman (i.e. DHE) shall not be supported”, the cipher suite TLS_DHE_RSA_WITH_AES_128_GCM_SHA256 is also not supported in 3GPP. So, the only 4 cipher suites in bold above are supported in 3GPP. In summary, the supported cryptographic algorithms for TLS 1.2 are:

- Asymmetric:

  - Key agreement: ECDHE

  - Digital signature: ECDSA, RSA

- Symmetric:

  - Authenticated encryption: AES_128_GCM, AES_256_GCM

  - Hash: SHA256, SHA384

Similarly, for TLS 1.3 RFC 8446 clause 9.1 (pp. 102) says “A TLS-compliant application MUST implement the TLS_AES_128_GCM_SHA256  [GCM] cipher suite and SHOULD implement the TLS_AES_256_GCM_SHA384 [GCM] and TLS_CHACHA20_POLY1305_SHA256 [RFC8439] cipher suites (see Appendix B.4). A TLS-compliant application MUST support digital signatures with rsa_pkcs1_sha256 (for certificates), rsa_pss_rsae_sha256 (for CertificateVerify and certificates), and ecdsa_secp256r1_sha256. A TLS-compliant application MUST support key exchange with secp256r1 (NIST P-256) and SHOULD support key exchange with X25519 [RFC7748].”

So, the supported cryptographic algorithms for TLS 1.3 are:

- Asymmetric:

  - Key agreement: ECDHE

  - Digital signature: ECDSA, RSA

- Symmetric:

  - Authenticated encryption: AES_128_GCM, AES_256_GCM, CHACHA20_POLY1305

  - Hash: SHA256, SHA384

* * * First Change * * * *

Table 4.2-1: Protocols Used in 3GPP Employing Symmetric Cryptographic Algorithms (5G System)

| Protocol/Function | Protocol Profile, Clauses | Cryptographic Algorithm(s) | Feature(s), Usage Type |
| --- | --- | --- | --- |
| TLS 1.2 (IETF RFC 5246) | TS 33.210 [2] clauses 6.2.1, 6.2.3 | AES_128_GCM, AES_256_GCM | Authenticated encryption |
|  |  | SHA256, SHA384 | Hash |
| TLS 1.3 (IETF RFC 8446) | TS 33.210 [2] clauses 6.2.1, 6.2.2 | AES_128_GCM, AES_256_GCM, CHACHA20_POLY1305 | Authenticated encryption |
|  |  | SHA256, SHA384 | Hash |

* * * Next Change * * * *

Table 4.3-1: Protocols Used in 3GPP Employing Asymmetric Cryptographic Algorithms (5G System)

| Protocol/Function | Protocol Profile, Clauses | Cryptographic Algorithm(s) | Feature(s), Usage Type |
| --- | --- | --- | --- |
| TLS 1.2 (IETF RFC 5246) | TS 33.210 [2] clauses 6.2.1, 6.2.3 | ECDHE | Key agreement |
|  |  | ECDSA, RSA | Digital signature |
| TLS 1.3 (IETF RFC 8446) | TS 33.210 [2] clauses 6.2.1, 6.2.2 | ECDHE | Key agreement |
|  |  | ECDSA, RSA | Digital signature |

* * * End of Changes * * * *