# 3GPP TSG-SA3 Meeting #122																			S3-252175

# Fukuoka, Japan, 19 – 23 May 2025

Source:	Qualcomm Incorporated

Title:	Pseudo-CR on 3GPP Cryptographic Inventory Clause Restructuring

Document for:	Approval

Agenda item:	5.20

Spec:	3GPP TR 33.938

Version:	0.2.0

Work Item:	FS_CryptoInv

Comments

This document provides clause restructuring suggestions.

Since, both clauses 4.2 and 4.3 provide summaries of the detailed protocol list in 4.4, we propose the following changes to clause numbering, such that the details are presented before their summaries:

- Old clause 4.4  new clause 4.2

- Old clause 4.2  new clause 4.3.1

- Old clause 4.3  new clause 4.3.2

* * * First Change * * * *

# 4	3GPP Cryptographic Inventory – 5G System

## 4.1	General

This clause provides inventory of security protocols that use cryptography in 3GPP specifications for 5G systems (limited to the standalone mode). The clause 4.2 presents inventory in detailed lists, and the clause 4.3 summarizes them in tables.

## 4.2	3GPP Symmetric Cryptographic Algorithms

Editor’s Note: The current table is for example and placeholder purposes. It would be revised/refined once the detailed protocol list description has been agreed.

The following table summarizes the security related protocols used in 3GPP employing symmetric cryptographic algorithms including hash functions (5G System).

Table 4.2-1: Protocols Used in 3GPP Employing Symmetric Cryptographic Algorithms (5G System)

## 4.3	3GPP Asymmetric Cryptographic Algorithms

Editor’s Note: The current table is for example and placeholder purposes. It would be revised/refined once the detailed protocol list description has been agreed.

The following table summarizes the security related protocols used in 3GPP employing asymmetric cryptographic algorithms (5G System).

Table 4.3-1: Protocols Used in 3GPP Employing Asymmetric Cryptographic Algorithms (5G System)

## 4.2	Detailed Protocol List

Editor’s Note: This detailed protocol list is expected to finalize first.

### 4.2.1	DTLS

DTLS is used in 5G system in standalone mode to protect the following:

- N2 interface (see clause 9.2 of TS 33.501 [4]).

- Xn interface (see clause 9.4 of TS 33.501 [4]).

- DIAMETER or GTP-based interfaces (see clause 9.5 of TS 33.501 [4]).

- gNB internal interfaces (see clause 9.8 of TS 33.501 [4]).

Security profiles for DTLS implementation and usage in 3GPP are given in clause 6.2 of TS 33.210 [2] and the certificate profile is given in clause 6.1.3a of TS 33.310 [3].

DTLS employs symmetric cryptography for confidentiality and integrity protection.

DTLS employs asymmetric cryptography for digital signature and key agreement.

### 4.2.2	TLS

TLS is used in 5G system in standalone mode to protect the following:

- NIDD interfaces (see clause 6.16.3 of TS 33.501 [4]).

- DIAMETER or GTP-based interfaces (see clause 9.5 of TS 33.501 [4]).

- NEF – AF interface (see clauses 12.2 and 12.3 of TS 33.501 [4]).

- Interfaces between network functions (see clauses 13.1, 13.2, 13.5 of TS 33.501 [4]).

- N32 interface (see clause 13.2 of TS 33.501 [4]).

- Network slice management interfaces (see clauses 15.2 and 15.3 of TS 33.501 [4]).

- Message Service interfaces for MIoT over the 5G System (see clauses Y.2 – Y.4 of TS 33.501 [4]).

Security profiles for TLS implementation and usage in 3GPP are given in clause 6.2 of TS 33.210 [2] and the certificate profile is given in clause 6.1.3a of TS 33.310 [3].

TLS employs symmetric cryptography for confidentiality and integrity protection.

TLS employs asymmetric cryptography for digital signature and key agreement.

### 4.2.3	EAP-TLS

EAP-TLS [5][6] is used in 5G system in standalone mode to realise the following:

- Mutual authentication between UE and AUSF (see Annex B of TS 33.501 [4])

- Mutual authentication between UE and AAA (see Annex I of TS 33.501 [4])

- Mutual authentication between UE and AUSF (see Annex I of TS 33.501 [4])

- Mutual authentication between N5GC and AUSF (see Annex O of TS 33.501 [4])

The 3GPP TLS protocol profile related to supported TLS versions and supported TLS cipher suites in 3GPP networks is specified in clause 6.2 of TS 33.210 [2]. The 3GPP profile of TLS certificates is specified in clause 6.1.3a of TS 33.310 [3].

EAP-TLS employs asymmetric cryptography for authentication and key agreement.

EAP-TLS employs symmetric cryptography for authentication and key agreement.

EAP-TLS employs hash function for session key derivation.

### 4.2.4	ECIES

ECIES is used in 5G system in standalone mode for the following:

- Confidentiality and Integrity Protection of the SUPI (see Annex C.3 of TS 33.501 [4]).

The ECIES profiles follow the terminology and processing specified in SECG version 2 [7] and [8]. The security profiles for the ECIES implementation and usage in 3GPP is given in clause C.3.4 of TS 33.501 [4].

ECIES employs asymmetric cryptography for the key agreement of the symmetric keys.

ECIES employs symmetric cryptography for the confidentiality and integrity protection of the SUPI.

### 4.2.5	PKI

PKI is used in 5G system in standalone mode for the following:

- Issuing of X.509 certificates (see Clause 4 of TS 33.310 [3]).

- PKI architecture for NDS/AF (see Clause 5.1 of TS 33.310 [3]).

PKI employs asymmetric cryptography for certificate signing and verification.

PKI employs hash function for computation of digests.

### 4.2.6	Online Certificate Status Protocol (OCSP)

Online Certificate Status Protocol (OCSP) is used in 5G system in standalone mode for the following:

- OCSP and the related profiles (see Clause 6.1b of TS 33.310 [3]).

- TLS profiles for TLS certificate status request extension, i.e., OCSP stapling (see Clause 6 of TS 33.210 [2]).

- Introduction to Revocation of subscriber certificates (see Clause B.2.2 of TS 33.501 [4]).

OCSP employs asymmetric cryptography for digital signing and signature verification.

OCSP employs hash algorithms for computation of digests.

### 4.2.7	QUIC and MPQUIC

The QUIC and MPQUIC are used in 5G system in standalone mode for the following:

- Security of the QUIC connection between UPF and AS proxy (see Clause 18 of TS 33.501 [4]).

- Multipath QUIC (MPQUIC) steering functionality is used for ATSSS (see Clause AA of TS 33.501 [4]).

For the QUIC establishment, the RFC 9001 [9] mandates the use of TLS as described in the TLS clause 4.4.2.

### 4.2.8	CBOR Object Signing and Encryption (COSE)

The COSE [10] is used in 5G system in standalone mode for the following:

- OSCORE [12] for cryptographic algorithm selection between UE and BSE (reference point Ua) (see Clause P.3.3 of TS 33.220 [11]).

COSE employs asymmetric cryptography for digital signature and key agreement.

COSE employs symmetric cryptography for confidentiality and integrity protection.

COSE employs hash functions for session key derivation.

### 4.2.9	MIKEY-SAKKE

MIKEY-SAKKE is used in the 5G system to securely transport cryptographic keys for Mission Critical Services. It is used in the following scenarios:

- Group Master Keys (GMKs) from a Group Management Server to a Group Management Client on a MC UE (see Annex E clause E.2 TS 33.180 [13])

- Private Call Keys (PCKs) between MC UEs (see Annex E clause E.3 TS 33.180 [13])

- Client-Server keys (CSKs) between MCX Server and MC client (see Annex E clause E.3 TS 33.180 [13])

- Multicast Signalling Keys (MuSiK) from MCX Servers to MC clients (see Annex E clause E.3 TS 33.180 [13])

Security profiles for MIKEY-SAKKE are left for implementation.

MIKEY-SAKKE is specified in IETF RFC 6509 [14].

MIKEY-SAKKE employs asymmetric cryptography for key distribution.

### 4.2.10	IKEv2

IKEv2 protocol is used to perform authentication and setup Security Associations (SA) for IPsec tunnels. The IPsec ESP protocol is described in clause 4.4.15.

IKEv2 is used in 5G system to provide security for the following:

-	Untrusted non-3GPP access to the 5G core network (see clause 7 of TS 33.501 [4]) and trusted non-3GPP access to the 5G core network (see clause 7 of TS 33.501 [4])

-	IP based interfaces for 5GC and 5G-AN according to NDS/IP (see clause 9 of TS 33.501 [4])

-	N2 interface between the AMF and the 5G-AN (see clause 9.2 of TS 33.501 [4])

-	N3 interface between the UPF and 5G-AN (see clause 9.3 of TS 33.501 [4])

-	Xn interface between 5G-AN (see clause 9.4 of TS 33.501 [4])

-	F1 and E1 of the gNB internal interfaces (see clause 9.8 of TS 33.501 [4])

-	Non-SBA interfaces internal to 5GC and between PLMNs (see clause 9.9 of TS 33.501 [4])

-	F1 interface between the IAB-node (gNB-DU) and the IAB-donor-CU (see clause M3.3 and M5 of TS 33.501 [4])

Security profiles for IKEv2 implementation and usage in 3GPP are given in clauses 5.2, 5.4, and 5.6 of TS 33.210 [2] and clauses 5, 6.2, and 7.5 of TS 33.310 [3].

IKEv2 employs symmetric cryptography for confidentiality and integrity protection.

IKEv2 employs asymmetric cryptography for digital signature and key agreement.

IKEv2 employs both symmetric cryptography and asymmetric cryptography for authentication.

### 4.2.11	PDCP security

The PDCP security protocol between the UE and the NG-RAN is responsible for the security protection of the following scenarios in 5G system:

- RRC integrity and confidentiality protection between UE and gNB (see clause 6.5.1 and 6.5.2 of TS 33.501 [4]).

- User plane data integrity and confidentiality protection between UE and gNB (see clause 6.6.3 and 6.6.4 of TS 33.501 [4]).

PDCP security protocol employs symmetric cryptography for confidentiality and integrity protection.

### 4.2.12	NAS security

The NAS security mechanisms is to protect NAS signaling and data between the UE and the AMF over the N1 reference point in 5G system:

- NAS signaling integrity and confidentiality protection between UE and AMF (see clause 6.4.3 and 6.4.4 of TS 33.501 [4]).

- User plane data (SMS over NAS) integrity and confidentiality protection between UE and AMF (see clause 6.4.7 and 6.16 of TS 33.501 [4]).

NAS security protocol employs symmetric cryptography for confidentiality and integrity protection.

### 4.2.13	EAP-AKA’

EAP-AKA’ enables mutual authentication between the UE and AUSF and provides keying material that can be used between the UE and the serving network in subsequent security procedures.

The long term key K and the SUPI are preconfigured in the USIM (in the UE) and in the UDM/ARPF.

EAP-AKA' is specified in RFC 5448 [15].

The 3GPP 5G profile for EAP-AKA' is specified in the normative Annex F of TS 33.501 [4].

KDF for key generation is HMAC-SHA-256 as per Annex B.2.0 of TS 33.220 [11].

EAP-AKA’ requires functions as described for 128 Bit MILENAGE in TS 35.205 [16], 128 Bit or 256 Bit TUAK in TS 35.231 [17] and in TS 35.234 [18] for 256 Bit MILENAGE.

EAP-AKA’ employs symmetric cryptography for authentication and key agreement.

EAP-AKA’ employs hash function for session key derivation.

### 4.2.14	5G-AKA

5G-AKA enables mutual authentication between the UE and AUSF with proof of successful authentication of the UE from the visited network. 5G-AKA provides keying material that can be used between the UE and the serving network in subsequent security procedures.

The long term key K and the SUPI are preconfigured in the USIM (in the UE) and in the UDM/ARPF.

5G-AKA is specified in TS 33.501 [4].

KDF for key generation is HMAC-SHA-256 as per Annex B.2.0 of TS 33.220 [11].

5G-AKA requires functions as described for 128 Bit MILENAGE in TS 35.205 [16], 128 Bit or 256 Bit TUAK in TS 35.231 [17] and in TS 35.234 [18] for 256 Bit MILENAGE.

5G-AKA employs symmetric cryptography for authentication and key agreement.

5G-AKA employs hash function for session key derivation.

### 4.2.15	IPsec ESP

IPsec ESP is used in 5G system to provide security for the following:

- Untrusted non-3GPP access to the 5G core network (see clause 7 of TS 33.501 [4]) and trusted non-3GPP access to the 5G core network (see clause 7 of TS 33.501 [4])

- IP based interfaces for 5GC and 5G-AN according to NDS/IP (see clause 9 of TS 33.501 [4])

- N2 interface between the AMF and the 5G-AN (see clause 9.2 of TS 33.501 [4])

- N3 interface between the UPF and 5G-AN (see clause 9.3 of TS 33.501 [4])

- Xn interface between 5G-AN (see clause 9.4 of TS 33.501 [4])

- F1 and E1 of the gNB internal interfaces (see clause 9.8 of TS 33.501 [4])

- Non-SBA interfaces internal to 5GC and between PLMNs (see clause 9.9 of TS 33.501 [4])

- F1 interface between the IAB-node (gNB-DU) and the IAB-donor-CU (see Annex M3.3 and M5 of TS 33.501 [4])

- Policy discrimination of GTP-C, GTP-U and protection of GTP-C transport protocol (see Annex B of TS 33.210 [2])

-	Protection of IMS protocols and interfaces for all SIP signalling traversing inter-security domain boundaries. (see Annex C of TS 33.210 [2])

-	Protection of UTRAN/GERAN IP transport protocols and interfaces for all RANAP and RNSAP messages traversing inter-security domain boundaries. (see Annex D of TS 33.210 [2])

Security profile for IPsec ESP implementation in 3GPP are given in clause 5.3 of TS 33.210 [2].

IPSec ESP employs symmetric cryptography for confidentiality, integrity and replay protection.

Keying happens using IKEv2 (Internet Key Exchange Protocol Version 2 (IKEv2) as mentioned in clause 4.4.10.

### 4.2.16	Key Derivation Function (KDF)

The KDF is used in 5G system in standalone mode and is defined in the normative Annex A of TS 33.501 [4].

- The generic KDF for the purpose of a cryptographic key computation is specified in the normative Annex B.2 of TS 33.220 [11].

The KDF employs hash function for key derivation.

### 4.2.17	JWE and JWS

JSON Web Encryption (JWE) and/or JSON Web Signature (JWS) are used in 5G system in standalone mode to protect the following:

- N32 interface (see clause 13.2 of TS 33.501 [4]).

- NF service access (see clause 13.4 of TS 33.501 [4]).

Profiles for JWE/JWS implementation and usage in 3GPP are given in clause 6.3 of TS 33.210 [2].

JWE/JWS employ symmetric cryptography for confidentiality and integrity protection.

JWE/JWS employ asymmetric cryptography for digital signature and key agreement.

## 4.3 	Summary Tables

### 4.3.1	3GPP Symmetric Cryptographic Algorithms

Editor’s Note: The current table is for example and placeholder purposes. It would be revised/refined once the detailed protocol list description has been agreed.

The following table summarizes the security related protocols used in 3GPP employing symmetric cryptographic algorithms including hash functions (5G System).

Table 4.2-1: Protocols Used in 3GPP Employing Symmetric Cryptographic Algorithms (5G System)

| Protocol/Function | Protocol Profile, Clauses | Cryptographic Algorithm(s) | Feature(s), Usage Type |
| --- | --- | --- | --- |
| e.g., PDCP (TS 38.323[]) | TS 33.501 [4] | 128-NxA1 | Confidentiality and Integrity Protection |
|  |  | 128-NxA2 | Confidentiality and Integrity Protection |

### 4.3.2	3GPP Asymmetric Cryptographic Algorithms

Editor’s Note: The current table is for example and placeholder purposes. It would be revised/refined once the detailed protocol list description has been agreed.

The following table summarizes the security related protocols used in 3GPP employing asymmetric cryptographic algorithms (5G System).

Table 4.3-1: Protocols Used in 3GPP Employing Asymmetric Cryptographic Algorithms (5G System)

| Protocol/Function | Protocol Profile, Clauses | Cryptographic Algorithm(s) | Feature(s), Usage Type |
| --- | --- | --- | --- |
| e.g., TLS (IETF RFC 8446) | TS 33.210 [2] | ECDHE (IETF RFC 8996) | Key Agreement |
|  |  | RSA (IETF RFC 8017) | Digital Signature and Authentication |

* * * End of Changes * * * *