# 3GPP TSG-SA3 Meeting #122																			S3-252193

# Fukuoka, Japan, 19 – 23 May 2025

Source:	Ericsson

Title:	Conclusion update for KI#2: Security of IMS based Avatar Communication

Document for:	Approval

Agenda item:	5.2

Spec:	3GPP TR 33.790

Version:	1.3.0

Work Item:	FS_NG_RTC_SEC_Ph2

Comments

The conclusions for KI#2: Security of IMS based Avatar Communication, are incomplete.

This document proposes a way forward for concluding KI#2.

The following Editor's Note can be simply removed since it was taken care of by the NOTE after the EN.

Editor's Note: Protection of communication between MF and DC AS/BAR if the DC AS/BAR is deployed outside of operator’s domain is FFS.

A proposal for removing the following Editor's Note is to state that avatar representation needs to be removed after the receiving UE centric procedure.

Editor's Note: How to prevent UE2 from using UE1's avatar representation is FFS, if sending UE centric procedure is used after UE2 obtains UE1's avatar representation from receiving UE centric procedure.

* * * First Change * * * *

## 7.2		Conclusions for Key Issue #2: Security of IMS based Avatar Communication

The following principles are concluded for normative work:

1.	Any security normative work for this key issue will take the TS 23.228[7], Annex AC.11 in account.

2.	The Avatar ID sent by the UE1 should be verified by the BAR about whether it belongs to the UE1.

Editor's Note: Whether Avatar ID sent by the UE1 should be verified by DC AS is FFS.

3.	The authorization of avatar representation downloading is based on a token mechanism.

Editor's Note: Further details on the token-based authorization mechanism are FFS.

4. 	The e2DCe or e2e media plane protection mechanism specified in TS 33.328 [17] are reused for authentication, integrity and confidentiality protection between UE and MF, UE and DC AS for Avatar ID List downloading and avatar representation downloading, Avatar ID transfer via BDC/ADC. As a result there is no need for new normative work for authentication, integrity and confidentiality protection when bootstrap and application data channels are used between two communicating entities

5.	Reuse existing security specification in TS 33.328 [17], Annex P.1, for security of aspects of SBA in IMS media control interface, to protect SBA communication between MF/DCSF and DC AS/BAR.

6.	Reuse existing security specification in TS 33.501 [11], clause 12 for Protection of the NEF – AF interface, to protect communication between DCSF and DC AS if the DC AS is deployed outside of operator’s domain.

Editor's Note: Protection of communication between MF and DC AS/BAR if the DC AS/BAR is deployed outside of operator’s domain is FFS.

NOTE 1:	Protection of communication between MF and DC AS/BAR is out of scope.

A UE which receive a UE avatar representation from another UE for avatar rendering in a receiving UE centric rendering mode deletes the avatar representation after the avatar session is over.

NOTE 2:	This is to prevent UE2 from using the avatar representation of UE1 in a sending UE centric procedure right after a receiving UE centric avatar session in which UE2 receives the avatar representation of UE1 for rendering.

* * * End of Changes * * * *