3GPP TSG-SA3 Meeting #122																			S3-252202

Fukuoka, Japan, 19 – 23 May 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.328 | CR | draftCR | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Living document for NG_RTC_SEC_Ph2: draftCR to TS 33.328, Security and privacy of IMS capability exposure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Huawei, HiSilicon, China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_SEC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As concluded in TR 33.790, a new clause in TS 33.328 is needed, which provides a summary of the security of IMS capability exposure, a reference to the related clauses in TS 23.228 and TS 23.502, a list of security requirements among other changes. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new clause. which provides a summary of the security of IMS capability exposure, a reference to the related clauses in TS 23.228 and TS 23.502, a list of security requirements among other changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Security of IMS capability exposure is not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, Annex X (new clause), Annex X.1 (new), Annex X.2 (new), Annex X.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | SA3#121: S3-251752, version after incorporating changes from S3-251596, S3-251336, S3-251538 |  |  |  |  |  |  |  |  |

*** 1st CHANGE ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

…

[49]	IETF RFC 8842: "Session Description Protocol (SDP) Offer/Answer Considerations for Datagram Transport Layer Security (DTLS) and Transport Layer Security (TLS)".

[50]	IETF RFC 8831: "WebRTC Data Channels".

[51]	IETF RFC 8864 : "Negotiation Data Channels Using the Session Description Protocol (SDP)".

[XX]	3GPP TS 23.502: " Procedures for the 5G System (5GS); Stage 2".

*** 2nd CHANGE ***

# Annex X (Normative):	Security and privacy for IMS capability exposure

## X.1	General

This clause specifies the security and privacy for IMS capability exposure including two parts. One covers the security of the IMS (DC or non-DC) event exposure and the other covers the security of the IMS DC control procedures.

TS 23.228 [3] and the TS 23.502 [XX] has specified the IMS Subscribe/Notify Framework Architecture as well as the DC session control handling procedures.

Editor's Note: More content is FFS.

## X.2	Security requirements

The security requirements for the security and privacy of IMS capability exposure are:

- The 5G system shall support authentication and authorization of data channel application server during the IMS capability exposure procedures.

## X.3	Procedures

Existing security specification in TS 33.501 [47], clause 12 for the NEF-AF interface shall be re-used to perform mutual authentication and authorization of NEF and AF, and the interface protection.

Editor's Note: More content is FFS.

*** END OF CHANGES ***