3GPP TSG-SA3 Meeting #122																			    S3-252249

Fukuoka, Japan, 19 – 23 May 2025

Source:	Nokia

Title:	Update to S3-251848 on Nested API

Document for:	Approval

Agenda item:	4.22

Spec:	3GPP TS/TR 33.122

Version:	19.0.0

Work Item:	CAPIF_Ph3_sec

Comments

pCR to S3-251868 Living CR on Nested API

changes shown in blue

* * * Start of the Changes * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[3]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs".

[4]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[5]	IETF RFC 6750: "The OAuth 2.0 Authorization Framework: Bearer Token Usage".

[6]	IETF RFC 7519: "JSON Web Token (JWT)".

[7]	IETF RFC 7515: "JSON Web Signature (JWS)".

[8]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA)".

[9]	Void

[10]	3GPP TS 33.210: "3G security; Network Domain Security (NDS); IP network layer security".

[11]	IETF RFC 7636: " Proof Key for Code Exchange by OAuth Public Clients".

[xx]	IETF RFC 8693: "OAuth 2.0 Token Exchange".

* * * Next Change * * * *

## 6.X	Authorization procedure in a nested API invocation

Editor’s Note:	This clause will include authorization procedure for nested API invocation cases.

The nested API invocation scenario is a scenario where an API invocation towards a first API exposing function (AEF-1) triggers that this API exposing function (AEF-1) requests an API invocation towards a second API exposing function (AEF-2). I.e. AEF-1 becomes API Invoker towards AEF-2. Both belong to the same API provider domain.

The authorization of API invocation triggered towards the second API exposing function shall use token exchange procedure as specified in IETF RFC 8693 [xx], where the access token of the API invoker to be used towards AEF-1 is used as the subject token as per the IETF RFC 8693 [xx]. AEF-1 before triggering API invocation towards AEF-2, invokes the token exchange request towards the CCF by sending the subject token to receive a new access token. AEF-1 uses the received new access token towards AEF-2 for nested API invocation. This is depicted in figure 6.X-1.

Figure 6.X-1: Authorization for nested API invocation

1.	API invoker - 1 invokes the AEF-1 service by using the access token obtained from the CCF/Authorization Function.

2.	Based on the service API invocation request, API exposing function 1 (AEF-1) verifies the access token and decides to invoke another service API exposed by API exposing function 2 (AEF-2).

3.	AEF-1 sends a token exchange request message to CCF, to get a token to invoke the service API in AEF-2. The CCF validates whether the requesting AEF-1 is allowed for accessing the requested service API of AEF-2. Also, the CCF validates the access token in the request message that is provided by the API invoker to AEF-1.  After successful validation, the CCF generates a new access token to allow AEF-1 to invoke the service API on AEF-2. CCF responds to AEF-1 with the token exchange response message including the new access token for service API invocation on AEF-2.

Editor’s Note:	The content of the newly issued token is FFS.

Editor’s Note:	The parameters in the token exchange request and response is FFS.

Editor’s Note:	How the CCF validates whether the requesting API exposing function 1 is allowed for delegated authorization to access service API of API exposing function 2 is FFS.

4.	AEF-1 shall send a service API invocation request to AEF-2 with the authorization information i.e., access token received in step 3.

5.	AEF-2 checks whether the API invoker is authorized to invoke the requested service API based on the received token and if allowed, AEF-2 sends a service API invocation response.

6.	The API invoker receives from AEF-1 the service API invocation response after the service API invocation from AEF-2.

* * * End of the Changes * * * *