3GPP TSG-SA3 Meeting #122	S3-252251

Fukuoka, Japan, 19-23 May 2025

Source:	Nokia

Title:	Update to S3-251890 on CAPIF-interconnection

Document for:	Approval

Agenda item:	4.22

Spec:	3GPP TS/TR 33.122

Version:	19.0.0

Work Item:	CAPIF_Ph3_sec

Comments

pCR to S3-251890 Living CR on CAPIF-interconnection

Changes on top of joint pre-work are marked blue.

* * * First Change * * * *

## 6.X	Security procedures for CAPIF interconnection

### 6.X.1	General

The CAPIF provider A and CAPIF provider B host the CAPIF in their trust domains as specified in clause 6.2.2 in TS 23.222 [3]. The designated CAPIF core function of the CAPIF provider A interconnects with the designated CAPIF core function of the CAPIF provider B over CAPIF-6/6e interface.

The following clauses 6.X.2 and 6.X.3 detail security aspects of the scenario where the API invoker is onboarded to CCF-B of the CAPIF provider B and the target AEF is registered to CCF-A of CAPIF provider A.

### 6.X.2	Security method negotiation

For security method negotiation procedure in CAPIF interconnection, clause 6.3.1.2 shall be followed with the following enhancement:

- The API invoker shall send the security method request to the CCF-B.

- In case where CCF-B is in possession of the security method(s) as specified in clause 8.25.3.1 of TS 23.222 [3], CCF-B shall select a security method to be used over CAPIF-2/2e reference point for each AEF based on the access scenarios and AEF capabilities.

- In case where CCF-B is not in possession of the security method(s), based on the AEF details received from the API invoker, CCF-B identifies the CCF-A where the AEF is registered and sends the request to CCF-A to either get the supported list of security method(s) of AEF or to get a selected security method. The request to CCF-A shall include AEF details and may include the API invoker ID and security method supported by API invoker (e.g., to enable CCF-A to select the security method). The CCF-A shall provide to CCF-B either the list of supported security methods of AEF or the selected security method. If the list of supported security methods of AEF is received, the CCF-B shall select a security method to be used over CAPIF-2/2e reference point for each AEF based on the access scenarios and AEF capabilities.

- The CCF-B shall send Security Method Response message to the API invoker indicating the selected security method for each AEF.

### 6.X.3	Authentication and authorization procedure

For the mutual authentication between the API invoker onboarded to CCF-B in CAPIF provider B and AEF registered to the CCF-A in CAPIF provider A, the procedures as defined in clause 6.5.2 shall be followed with the enhancements as specified in in this clause.

#### 6.X.3.1	Method 1: Using TLS-PSK

The authentication procedure in clause 6.5.2.1 shall be followed with the following enhancements for CAPIF interconnect:

- The API invoker in CAPIF provider domain B shall include the API invoker ID in the authentication initiation request message sent to the target AEF-A in CAPIF provider domain A for CAPIF interconnection.

- Based on the AEF-A details available at the API invoker which indicates the AEF belongs to CCF-A, the API invoker shall include the CCF-B ID (CAPIF core function the API invoker is onboarded to) in the authentication initiation request message.

NOTE: CCF-B ID reveals that the API Invoker is accessing API services from a different CAPIF provider domain.

- The AEF in CAPIF provider domain A shall request for security information (AEFPSK) from CCF-A to perform authentication and secure connection establishment with the API invoker, if the AEF does not have a security information. The request shall include the API invoker ID and the CCF-B ID.

- When CCF-A receives the request message for security information from AEF-A, CCF-A fetches the security information related to the chosen security method (TLS-PSK: AEFPSK), based on API invoker ID and CCF-B ID.

- If the CCF-A does not have security information of the API invoker locally available, CCF-A shall request the security information from CCF-B over CAPIF-6/6e reference point based on the received API invoker ID, and the available AEF details. The CCF-B shall provide the TLS-PSK related security information (AEFPSK) to CCF-A.

NOTE: The CCF-A can check the received API Invoker ID and CCF-B ID based on the information locally available to contact the right CCF-B to fetch the security information.

Editor’s Note: For AEFPSK identification, if AEF details are sufficient or service API information is needed is FFS.

- The CCF-B shall provide the security information related to the chosen security method to the CCF-A.

- The CCF-A shall provide the received security information to the AEF.

- After sending the authentication initiation response, API invoker in CAPIF provider domain B and AEF in CAPIF provider domain A establish a TLS connection using the security information obtained.

After successful authentication of API invoker and AEF, the  AEF sends the authorization information request to CCF-A. Based on CCF-A's response to the authorization information request, AEF performs authorization based on the obtained authorization information.

Editor’s Note: Whether CCF-A will have sufficient information to authorize the API invoker or it needs additional information from CCF-B is FFS.

#### 6.X.3.2	Method 2: Using PKI

The authentication procedure in clause 6.5.2.2 shall be followed with the following enhancements for CAPIF interconnect to establish a dedicated secure session over CAPIF-2e using mTLS:

- For fetching the security information related to the chosen security method (TLS-PKI) the CCF-B includes only the API invoker ID.

- The CCF-B provides the security information (API invoker's root CA certificate) to the AEF via CCF-A, for allowing the AEF to validate the API invoker's certificate.

After successful establishment of TLS between API invoker and AEF, AEF is able to authorize the API invoker’s service API invocation request from the other API provider domain.

#### 6.X.3.3		Method 3: TLS with OAuth Token

The authentication procedure in clause 6.5.2.3 shall be followed to establish a dedicated secure session between API invoker and CCF-B of the API provider domain B over CAPIF-2e using mTLS.

The authorization procedure in clause 6.5.2.3 shall be followed with the following enhancements:

- The API invoker shall send the access token request message to the onboarded CCF-B. Since the discovered service API is provided by a different CAPIF provider domain, the API invoker shall provide a client credential assertion token to allow for authentication of the API Invoker ID by CCF and AEF of the other CAPIF provider domain.

- CCF-B determines that the service API requested is provided by an AEF in CAPIF provider domain A and sends the access token request message to CCF-A in CAPIF provider domain A. If the Onboard_secret is included in the access token request to CCF_B, it shall be removed before forwarding the request to CCF-A.

- CCF-A authenticates the API invoker. If the CCA is included, CCF-A may verify the API invoker identity. If successfully authenticated, CCF-A generates an access token specific to the API invoker and provides it in an Access Token Response message to the API invoker via CCF-B.

- On CAPIF-2e, the API invoker authenticates to the AEF by establishing a TLS session .

- If successfully authenticated to the AEF, the API invoker shall initiate the invocation of a 3GPP northbound API request with the AEF including the OAuth 2.0 access token in the request. .

- AEF verifies the integrity of the access token by verifying CCF-A's signature to validate the access permission for the requested service API as specified in clause 6.5.2.3.

* * * End of Change * * * *