3GPP TSG-SA3 Meeting #122	S3-252256

Fukuoka, Japan, 19 – 23 May 2025

# Source:	GSMA

# Title:	LS  to 3GPP about the external data channel content access requirements

# Document for:	Information, Discussion

# Agenda Item:3.1

# Liaison Statement

| Liaison Statement Title: | LS to 3GPP about the external data channel content access requirements |
| --- | --- |

| Source Meeting Information |  |  |
| --- | --- | --- |
| Meeting Number | Meeting Date | Meeting Location |
| UPG#14 | April 3,2025 | Tokyo, Japan |

| Document Details |  |  |
| --- | --- | --- |
| Document Number: | Creation Date: | Document Author: |
| UPG14_109r3 | April 4,2025 | Walter Zielinski/Huawei |
| Originating GSMA Source: | Deadline for response: | Liaison Statement Contact |
| NG UPG | See below | GSMALiaisons@gsma.com |
| Security classification: | Non-confidential |  |

| Recipients: |  |
| --- | --- |
| Internal recipients: |  |
| External recipients: | To : SA1, SA2,SA3, SA4 ,SA6, CT |

# Background

GSMA NG UPG#13 responded to 3GPP SA6 question in SP-241404 concerning the external server API gaps and GSMA members have been asked to review and comment on the content of Annex C and Annex D of GSMA PRD NG.134 describing the external access scenario.

The external content access was judged important when GSMA PRD NG.134 was developed as it allows the data channel applications to have an access to all the www content available when interacting with the user. That can be used in gaming, translation, or fintech applications, e.g. stock price quote. Even the access to so popular LLM models would require such external access.

The following is the technical input to 3GPP on the technical implementation aspects and potential gaps related to the external content access. Specifically, it is requested that 3GPP examine the following:

- Under-specification of 3GPP Release 18 TS 26.114 defined IMS data channel applications where under-specification refers to the state, that a certain aspects of data channel application definition leave space for misinterpretation. The content compositional pattern is left underspecified specifically with respect to the use, and interactions with the external content, i.e. when IMS content is mixed with third-party content giving raise to mashups. That is 3GPP TS 26.114 defined data channel applications are a monolithic and homogenous entities. Their content is self-contained through static definition or computed upon the interaction with the users. But in all cases the content is limited to the content hosted by IMS network, as no references are made to content originating from other networks. There is no concept of mashups where applications use content from multiple sources to create its final output, therefore potentially limiting Communication Service Provider (CSP) access to external 3rd party content, e.g. specialized LLM or other content. Essentially, it appears that the mashups are not allowed, under some interpretations, unless mashup content is hosted internally within the CSP administrative domain. Therefore, implementing applications where the front end components, e.g. branded User Interface/Icon, would be provided by the operator and the actual application would be sourced from the external provider network does not seem to be supported by 3GPP TS 26.114.

- The need for further description of IMS data channel application capabilities since 3GPP Release 18 TS 26.114 defines data channel application as JavaScript application but the application so defined requires an external access otherwise it is misaligned with the general ways how JavaScript API and JavaScript API ecosystem works. The term “JavaScript” application implies support for standardized and commonly used JavaScript functionality therefore W3C JavaScript  API should be allowed to be invoked successfully, i.e. as there is no prohibition on the API usage within 3GPP definition. Therefore, the data channel application should be allowed to use the fetch API, i.e. https://fetch.spec.whatwg.org/#fetch-method, towards any URL including the external URL. Fetch can be used for architectural reasons, e.g. single page application design, or for content access and injection. That being the case it is not clear how the fetch should work when an arbitrary URL is provided and whether fetch to external destination should be allowed and if not then how should it be blocked. Specifically, 3GPP specifications give no explicit guidance on what web APIs are permitted or how network requests are handled, i.e. no clarity on using the Fetch API to access external URLs from within an IMS Data Channel application, and no specification of whether such requests should go via the IMS PDN connection or the general Internet PDN (multi-APN issue). IMS traffic is served  on a dedicated APN (for IMS signaling/media), separate from the Internet APN and 3GPP Release-18 does not state if a IMS Data Channel applications can open sockets or HTTP requests outside the IMS channel.

- The HTML usage and HTML flavour supported by 3GPP Release 18 standards due to the potential lack of support for external URL tags/references and/or the third party JavaScript code. That is a code executed on a publishers page but served from a remote address, often hosted by Content Delivery Networks, e.g. to access jQuery or other JavaScript libraries.

- JavaScript applications use 3rd party JavaScript code in its business logic, e.g. JQuery, to simplify the code base, to measure user engagement or do analytics. Currently, no clause in 3GPP TS 26.114 or other documents covers external media or script tags, representing an incomplete definition of data channel runtime. This leaves questions on whether and how such referenced content can be included and how they should be fetched. Without the external access enabled standard media or scripts tags might fail.

- 3GPP to non-3GPP content interworking and specifically in what concerns

  - the interface allowing to break out from IP Multimedia Subsystem (IMS) to Internet or other data networks

  - the procedures allowing interworking between DCMTSI client in terminal and  the external HTTP server hosting JavaScript/HTML/CSS content

- 3GPP Release 18 lacks both the architecture and the interface specification for such interworking and there is no reference point defined for an IMS Application Server allowing to interact with the external web content servers in the context of an ongoing IMS session. Similarly, there is no “breakout” HTTP gateway functionality standardized implying that IMS Data Channel applications can only incorporate external content using the proprietary solutions.

- The requirements for architectural and operational concept allowing to securely on-board in the real-time or otherwise the external content. Such mechanisms to convey that a content has been verified are supported by other solutions, e.g. chatbot. The current architecture does not provide the controls for vetting of any content regardless whether it is internal or external. That is a method classifying which content is authorized and is to be trusted by the data channel subsystem. That concerns the content screening itself but also the external server screening and validation that it can be allowed into the content mashup and API execution loop, e.g. black list or white list type mechanism. For instance, if an IMS Data Channel applications fetches a script from an unknown host, 3GPP specifications do not say whether the IMS core should inspect or block it, or if the UE should apply any policy. In the same manner the trust establishment between the operator’s IMS platform and third-party application servers is not standardized in 3GPP Release 18, e.g. exchange of certificates or tokens. In summary, operators are left without 3GPP guidance on runtime policies to apply, e.g. allowed domains, content filtering, and certificates, for external data used in IMS sessions.

- The regulatory requirements to classify 3GPP TS 26.114 defined IMS data channel applications and specifically whether IMS data channel application constitute an operator service per excellence. And whether it retains the same status when the external content is used partially or totally to construct the data channel application logic. That concerns the obligations that apply when mashups are used within the context of MMTEL call. This lack of clarity is a gap as it might be necessary to classify whether IMS Data Channel applications constitute Telco value-added services or rather internet sessions. Additionally other kind of issues need to be addressed. For instance who and how ensures compliance with accessibility or other service rules, e.g. Lawful Intercept, if part of the user experience is external to the network that initiated the call, e.g. if external content/ service influences the call does it fall under the operator’s service responsibilities?

# Requests to 3GPP SA1, SA2, SA3, SA4, SA6 and CT

GSMA NG UPG kindly requests 3GPP to provide the following clarifications:

- Question 1 to SA4:

Does 3GPP SA4 plan to enhance 3GPP TS 26.114 to support IMS Data Channel content mixing, i.e. mashups with the third-party content? A normative guidance on content composition, content embedding or referencing external content within an IMS-delivered application is needed in a fashion that ensures interoperability. For example, defining content policies allowing for mixing operator provided HTML with third-party JavaScript logic.

- Question 2 to SA2 and SA4:

Does 3GPP SA2/SA4 plan to enhance its standards so the respective documents would answer how, and if, IMS and Internet connectivity can be supported by the UE and the network. This question needs to be answered in order to establish how the invocation of W3C standardized APIs should be handled within IMS context.

- Question 3 to SA4 and CT:

Does 3GPP SA4/CT plan to standardize a method allowing IMS Data Channel Applications to support external content references, e.g.  <script src> or <img src> tags since 3GPP Release 18 provides no standards or profiles for handling such external resource loading. This gap might impact interoperability since implementations might evolve and become proprietary as some suppliers might chose to disallow external scripts, while others might permit it using the Internet connection.

- Question 4 to SA2 and SA6:

Does 3GPP SA2/SA6 plan to define procedures and architecture for interworking between the IMS session and external (non-IMS) content sources allowing for example pulling information from a public API in a secure manner?

- Question 5 to SA3:

Does 3GPP SA3 plan to standardize security architecture and measures for content and source validation including the method to compute trust towards external hosts and content?

- Question 6 to SA1:

Given the fact that IMS telephony services are subject to telecom regulation while HTML and JavaScript content is not a subject to such regulation what is 3GPP SA1 view on if and what regulatory exceptions apply to 3GPP TS 26.114 defined IMS Data Channel applications in general and specifically when those applications are mixed with the external content.

# Contact

In the case of further questions and/or feedback, please respond to GSMALiaisons@gsma.com

# NEXT UPG MEETINGS:

- UPG#15 : July 1st 2025

- UPG#16: October 2025