3GPP TSG-SA3 Meeting #122	S3-252264

Fukuoka, Japan, 19 – 23 May 2025

Title:	LS on PLMN ID checks in interconnect scenarios when NFc supports multiple PLMN IDs

Response to:

Release:	Release-16 onwards

Work Item:

Source:	SA3

To:	CT4

Cc:	CT3

Contact person:	Mohsin Khan

mohsin.a.khan@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

SA3 would like to inform CT4 about an issue with PLMN ID checks that may lead to service interruption, when the requesting NF consumer supports multiple PLMN IDs in interconnect scenarios.

For security purposes, the NF producer needs to verify the PLMN ID of the NF consumer in the service request. Towards that goal, in Rel-15, after SA3’s request, CT4 added NF consumer's PLMN ID in the Access Token Claims (CR C4-191528) to enable the checks on NF consumer’s PLMN ID by pSEPP and the NF producer.

In Rel-16, to consistently define authorization parameters for different services/procedures in the NRF, in the Access Token Request message, CT4 included a list of PLMN IDs instead of a single PLMN ID when NF supports multiple PLMN IDs (CR C4-203256).

During issuing an access token, it is unclear how the NRF chooses which of the NF consumer’s PLMN ID to include in the Access Token Claims, if the access token request by the NF consumer includes multiple PLMN IDs. Additionally, access tokens can include only one PLMN ID of the NF consumer and NRF is not aware which PLMN ID does the NF consumer intend to use in the service request API message towards the target NF producer.

Consequently, it may happen that the NF consumer obtains an access token from the NRF with a PLMN ID of the NF consumer that does not match with the PLMN ID that the NF consumer uses in the service request API message towards the target NF producer. Similarly, the PLMN ID in the access token might not match with the PLMN ID that the NF consumer uses in the service request body. As a result, for both cases access token verification fails, and the target NF producer rejects the service request.

# 2	Actions

To CT4

ACTION:

- SA3 kindly asks CT4 to share their perspective with regards to the described issue.

- SA3 kindly asks CT4 to consider solutions under the CT4 scope.

# 3	Dates of next TSG SA WG 3 meetings

SA3#123	25 – 29 August 2025		Goteborg, Sweden

SA3#124	13 – 17 October 2025		China