3GPP TSG-SA3 Meeting #122	S3-252272

Fukuoka, Japan, 19 - 23 May 2025   (merger of S3-251930, S3-251939, S3-251988, S3-251989, S3-251990, S3-252017, S3-252071, S3-252104. S3-252187)

Source:	Samsung, Intel, Huawei, HiSilicon, LG Electronics, Xiaomi, OPPO, ZTE Corporation, Apple, Vivo, Qualcomm Incorporated

Title:	Security procedure for inter-CU LTM

Document for:	Approval

Agenda item:	4.12

Spec:	3GPP TS 33.501

Version:	19.1.0

Work Item:	NR_Mob_Ph4_Sec

Comments

This pCR proposes normative text to the draftCR for security procedure for inter-CU LTM, based on the progress made in RAN WGs [R2-2502981 and S3-251824].

* * * First Change * * * *

#### 6.X.Y	Security mechanism and procedures for L1/L2 Triggered Mobility

##### 6.X.Y.1	When DC is not configured

For the case where CU is acting as MN and DC is not configured as specified in TS 38.300 [52], during LTM preparation phase, the serving gNB shall send the {KNG-RAN*, NCC} pair (per candidate cell), UE's 5G security capabilities, ciphering and integrity algorithms used in the serving cell and the UE's UP security policy along with the UE security context to the candidate gNB(s). The candidate gNB(s) shall send the LTM configuration(s) (containing the selected AS security algorithms and the UE’s UP security activation status) to the serving gNB for the accepted candidate cell(s). The serving gNB shall send the RRCReconfiguration message to the UE including the LTM candidate configurations containing the selected AS security algorithms and the UE’s UP security activation status.

The {KNG-RAN*, NCC} pair at the candidate cells require updating when a new key at the serving gNB, e.g. due to an inter-CU handover/cell switch or an intra-CU handover/cell switch with a change of key. To update the candidate cells, the serving gNB shall send the generated {KNG-RAN*, NCC} pair (per candidate cell) to the candidate gNB(s) using LTM Configuration Update message.

During LTM execution phase, the serving gNB includes the NCC used for the derivation of KNG-RAN* in the Cell Switch Command MAC CE to the UE. Upon receiving the cell switch command, the UE derives the KNG-RAN* as described in clause 6.9.2.3.4 of the present document and switches to the target gNB.

NOTE: Key changes that use the keySetChangeIndicator field to be set to true use the normal L3 handover.

If the target gNB receives UE's 5G security capabilities from the AMF (as detailed in clause 6.7.3.1 of the present document) and/or UE's UP security policy from the SMF (as detailed in clause 6.6.1 of the present document) in the Path-Switch Acknowledge message, then the target gNB may initiate an intra-gNB-CU handover to refresh KNG-RAN* or activate or de-activate the UP integrity/confidentiality as per the received policy from SMF or to indicate the selected algorithms appropriately. Further, the target gNB either releases or updates the LTM configuration (UE's 5G security capabilities and/or UE's UP security policy) of the candidate cells. If necessary, the serving gNB then updates the UE with the configuration aligned with LTM configuration in the network.

##### 6.X.Y.2	When DC is configured

In case of inter-SN SCG LTM as specified in TS 37.340 [51], the security mechanism and procedures as specified in clause 6.10.2.4 of this specification for SCPAC shall be applied.

* * * End of Changes * * * *