# 3GPP TSG-SA3 Meeting #122											                          draft_S3-252292

# Fukuoka, Japan, 19 – 23 May 2025				  merger of S3-251974, S3-252248, S3-252213, S3-252232

Source:	Huawei, HiSilicon, Nokia, Ericsson, Lenovo

Title:	Pseudo-CR on updating text for finer level authorization

Document for:	Approval

Agenda item:	4.22

Spec:	3GPP TS 33.122/ Living CR S3-251721

Version:	19.0.0

Work Item:	CAPIF_Ph3_Sec

Comments

This pCR is to provide revisions to the finer level authorization procedures.

* * * First Change * * * *

## 6.Y	Authorization for finer level service API access

The authorization function shall support finer level authorization as specified in TS 23.222 [3]. In both RNAA and non-RNAA scenarios, finer level service API access authorization can be used to limit access to specific services, specific service operations and/or resources.

To enable finer level service API access, the authorization request or access token request (from API invoker to the CCF/Authorization Server) includes the requested service, service operations, and/or resource(s) at the respective granularity. For RNAA, the request may also include the resource owner ID.

If the API invoker is authorized, the CCF responds to the API invoker with an access token including finer level authorization information, or an authorization code.

The following procedures are adapted to support finer level authorization:

Clause 6.5.3.2 on Authorization using OAuth client credential flow:

- The access token request message may also include details on finer level authorization.

Clause 6.5.3.3 on Authorization using authorization code (optional PKCE) flow:

- The authorization request may also include finer level authorization information.

- The CCF shall also check the finer level authorization if included in the request.

Clause 6.5.3.4 on	Revocation:

- The additional information in the Authorization Revocation Request message may also include information for finer level authorization.

Clause C.2.2 on Token claims:

- The scope can also include a list of service operation(s) and/or resource(s).

Clause C.3.2 on Access token request

- The scope can also include a list of service operation(s) and/or resource(s).

* * * End of Changes * * * *