3GPP TSG-SA3 Meeting #122	S3-252298

# Fukuoka, Japan, 19 - 23 May 2025               merger of S3-252044, S3-252045, S3-252111, S3-252193

Source:	Huawei, HiSilicon, Ericsson, Samsung, Nokia, Nokia Shanghai Bell, Philips International B.V.

Title:	Pseudo-CR on Updating conclusion of KI#2

Document for:	Approval

Agenda item:	5.2

Spec:	3GPP TR 33.790

Version:	1.3.0

Work Item:	FS_NG_RTC_SEC_Ph2

Comments

The following Editor's Notes are solved:

Editor's Note: Whether Avatar ID sent by the UE1 should be verified by DC AS is FFS.

Editor's Note: Further details on the token-based authorization mechanism are FFS.

For the first EN, according to clause AC.11.3.1 of TS 23.228, the UE can only update its Avatar ID list from DC AS through BDC or ADC, which means that the following scenario will not exist: the UE updates the Avatar ID while the DC AS is not updated. Therefore, the Avatar ID sent by the UE1 can be verified by DC AS.

For the second EN, it is solved by adding 1. the token should include Avatar ID and UE1 ID; 2. the BAR should verify whether the Avatar ID in the Avatar representation downloading request matches that in the token.

* * * First Change * * * *

7.X		Conclusions for Key Issue #2: Security of IMS based Avatar Communication

The following principles are concluded for normative work:

1.	Any security normative work for this key issue will take the TS 23.228[7], Annex AC.11 in account.

2.	The Avatar ID sent by the UE1 should be verified by the BAR about whether it belongs to the UE1.

NOTE: Whether the DC AS should verify the Avatar ID will be determined during the normative work.

3.	The authorization of avatar representation downloading is based on a token mechanism. The token includes Avatar ID, UE1 (sending UE) ID, the UE2 (receiving UE) ID for the receiving UE centric rendering case, one of MF ID or DC AS ID, BAR ID, and expiration time. The BAR verifies the token as follows:

- The BAR verifies the signature of the token.

- The BAR verifies whether the Avatar ID in the Avatar representation downloading request matches that in the token.

- The BAR verifies whether the UE1 ID in the Avatar representation downloading request matches that in the token.

- In the receiving UE centric rendering mode, the UE2 ID is included in the token, and the BAR also verifies whether the UE2 ID in the Avatar representation downloading request matches that in the token.

- The BAR checks whether MF ID or DC AS ID matches the entity sending the Avatar representation downloading request.

- The BAR checks whether the BAR ID matches itself.

- The BAR checks, based on the token expiration time, whether the token is valid.

- If the token verification is successful, the BAR provides the Avatar representation of UE1 to the requesting NF (MF or DC AS).

4. 	The e2DCe or e2e media plane protection mechanism specified in TS 33.328 [17] are reused for authentication, integrity and confidentiality protection between UE and MF, UE and DC AS for Avatar ID List downloading and avatar representation downloading, Avatar ID transfer via BDC/ADC. As a result there is no need for new normative work for authentication, integrity and confidentiality protection when bootstrap and application data channels are used between two communicating entities

5.	Reuse existing security specification in TS 33.328 [17], Annex P.1, for security of aspects of SBA in IMS media control interface, to protect SBA communication between MF/DCSF and DC AS/BAR.

6.	Reuse existing security specification in TS 33.501 [11], clause 12 for Protection of the NEF – AF interface, to protect communication between DCSF and DC AS if the DC AS is deployed outside of operator’s domain.

NOTE:	Protection of communication between MF and DC AS/BAR is out of scope.

NOTE: If sending UE centric procedure is used after UE2 obtains UE1's avatar representation from receiving UE centric procedure, how to prevent UE2 from using UE1's avatar representation is not addressed in the present document.

The UE2 is recommended to delete the UE1 avatar representation after the avatar session is over.