3GPP TSG-SA3 Meeting #122	S3-252372

# Fukuoka, Japan,  19 - 23 May 2025

Source:	Xidian University, OPPO

Title:	Pseudo-CR on Update AIOT sol#37

Document for:	Approval

Agenda item:	5.9

Spec:	3GPP TR 33.713

Version:	0.8.0

Work Item:	FS_Ambient_IoT_Sec

Comments

It is proposed to remove the following Editor’s Notes from the sol#37, revise the solution detail and provide evaluation. According to the conclusion reached in SA2, as stated in the latest version of 23700-13 that “When the AIoT device profile data is managed by the 5GC, it is stored in the ADM (Ambient IoT Data Management), possibly together with UDR, which exclusively supports AIoT devices.” Correspondingly, the security credential could be stored in ADM. This solution would not impact UDM, therefore the following EN can be removed.

Editor’s Note: Impact of using UDM on the network for credential storage and key generation is FFS.

Proposed Changes

* * * First Change * * * *

## 6.37	Solution #37:  Mutual Authentication Using AEAD for Inventory and Command case

### 6.37.1	Introduction

The solution addresses KI#5. It proposes a mutual authentication using AEAD between AIoT devices and the network. According to the conclusions in TR 23.700-13, the solution makes the following assumptions:

- An AIoT device shares a permanent identity (i.e., AIoT Device ID) and root key (i.e., K) with the network side.

### 6.37.2	Solution details

The message flow of this solution is described below:

Figure 6.37.2-1 Mutual authentication for security credentials stored in the ADM

1-2. Third-party AF sends inventory request to the AIoT function through NEF. The request can include an AIoT Device ID to page the device.

3-4. AIoT function generates a fresh nonce and sends the nonce to the AIoT devices through a Reader.

5. AIoT device calculate KAF and authentication parameters.

5.1 AIoT device computes KAF using key derived function. The input parameters include the K, the Device ID and the nonce.

5.2 The device uses f2 or AES algorithm to compute RES. The input parameters include the K, the identity and the nonce.

5.3 Alternatively, the devices can use AEAD algorithm to compute a message authentication code to protect the authenticity of uplink data (i.e. Device ID). The input parameters include the KAF and uplink data (i.e. Device ID).

6. The device sends inventory response (authentication request) including the identity, RES or message authentication code to the Reader.

7. The Reader forwards the response to the AIoT function, including authentication request.

8. The AIoT function sends the authentication request to the ADM, including nonce.

9. The ADM computes KAF and verifies the RES or message authentication code.

9.1 computes KAF using key derived function. The input parameters include the K, the Device ID and the nonce.

9.2 retrieves the SQN and the K according to the ID.

9.3 computes the anonymity key (AK) using f5 or AES algorithm. The input parameters include the K and the fresh nonce.

9.4 computes MAC using f1 or AES function. The input parameters include the K, the SQN fresh nonce and the command message.

10. The ADM sends KAF, AK⊕SQN and MAC to the AIoT function and increases the SQN by one.

11. The AIoT function stores the KAF corresponding to the identity and sends the Command request to the Reader, the message includes AK⊕SQN and MAC.

12. The Reader forwards the Command request to the AIoT device.

13. The AIoT device computes the AK in the same way as the ADM and decrypts the concealed SQN. Then the AIoT device executes the validation procedures of the SQN in the same way as the 5G-AKA (including the re-synchronisation procedures). If the SQN is valid, then it verifies MAC.

14 – 17.  Optionally AIoT device sends command response message.

Editor’s Note: Impact of using UDM on the network for credential storage and key generation is FFS.

### 6.37.3	Evaluation

TBD.

This solution addresses the requirement of Key Issue #5. It proposes a mutual authentication using AEAD between AIoT devices and the network for Inventory and Command case. The AIoT device profile data is stored in the ADM (Ambient IoT Data Management) if it is managed by the 5GC.

NOTE: This solution is not evaluated.

* * * End of Changes * * * *