3GPP TSG-SA3 Meeting #122																			S3-251834

Fukuoka, Japan, 19 – 23 May 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Draft_CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR |  | rev |  | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Living document for TEI19: Token-based authorization for indirect communication scenarios when NF is selected at target PLMN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-04-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The CR 4808 to TS 23.502 (S2-2407356) describes a scenario where NF Service Consumer and Producer are located in different domains, e.g. different different PLMN or NPN, and are using Indirect Communication. Steps 7-9 in Firgure 4.17.10a-1 introduce the option that the SCP in the target network performs the selection of the NF Service Producer. The procedures for service access authorization in interconnect and roaming scenarios, in clauses 13.4.1.1A and 13.4.1.2 of the present specification, do not cover indirect communication scenarios. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add procedure for token-based authorization for the scenario with indirect communication where NF Service Consumer and Producer are in different domains and the NF is selected at the target network. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not clear how token-based authorization is performed with indirect communication when Service Consumer and Producer are in different domains and the NF is selected at the target network. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13.4.1.X.Y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The X in the affected clause 13.4.1.X.Y is the same as the X in 13.4.1.X in CR S3-251676. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Merger S3-250351, S3-250813, and S3-250814 SA3#121: S3-251633, S3-251675 |  |  |  |  |  |  |  |  |

*** BEGIN CHANGES ***

#### 13.4.1.X.Y	Authorization with NF selection at target network

###### 13.4.1.X.Y.0	General

In scenarios where NF Service Consumer and NF Service Producer are in different PLMNs, when indirect communication is deployed in both source and target network, it is possible to delegate NF selection to the target network as described in clause 4.17.10a of TS 23.502 [8].

The present clause describes token-based authorization for the scenario where NF Service Consumer and NF Service Producer are in different PLMNs with indirect communication deployed (with or without delegated discovery), and NF selection is performed at the target PLMN.

###### 13.4.1.X.Y.1 Authorization for indirect communication without delegated discovery procedure

Figure 13.4.1.X.Y.1-1: Service access authorization when NF Service Consumer and NF Service Producer are in different PLMNs with indirect communication without delegated discovery and NF selection is at target PLMN

Steps 1 to 10 of Figure 13.4.1.X.Y.1-1 for discovery and service access authorization correspond to one of the following:

-	Steps 0 to 8 in clause 13.4.1.3.1.1 (Authorization for indirect communication scenarios without delegated discovery, with mutual authentication between NF Service Consumer and NRF at the transport layer), or

-	Steps 0 to 10 in clause 13.4.1.3.1.2 (Authorization for indirect communication scenarios without delegated discovery, without mutual authentication between NF Service Consumer and NRF at the transport layer),

with the following differences in Step 2 of figure 13.4.1.X.Y.1-1:

-	The NRF in the source PLMN (NRF1) does not perform the authorization of the NF Service Consumer but forwards the access token request to an NRF in the target PLMN (NRF2), via the SEPPs of the source PLMN and the target PLMN.

-	The NRF in the target PLMN (NRF2) authorizes the NF Service Consumer as described for the hNRF in clause 13.4.1.2.2 (Service access authorization in roaming scenarios, service request process).

NOTE: The NRF in the target PLMN and NF Service Producer cannot authenticate the NF Service Consumer unless the NRF in the target PLMN and the NF Service Producer are provisioned with required certificate to authenticate the NF Service Consumer.

###### 13.4.1.X.Y.2 Authorization for indirect communication with delegated discovery procedure

Figure 13.4.1.X.Y.2-1: Service access authorization when NF Service Consumer and NF Service Producer are in different PLMNs with indirect communication with delegated discovery and NF selection is at target PLMN

Steps 1 to 9 of figure 13.4.1.X.Y.2-1 for service access authorization correspond to:

-	Steps 1 to 9 in clause 13.4.1.3.2 (Authorization for indirect communication with delegated discovery procedure)

with the following differences:

-	The Service Request (Step 1 in figure 13.4.1.X.Y.2-1) and Service Response (Step 9 in figure 13.4.1.X.Y.2-1) are sent between the NF Service Consumer in the source PLMN and SCP2 in the target PLMN, via SCP1 in the source PLMN and via the SEPPs in the source PLMN and target PLMN.

NOTE: The NRF in the target PLMN and NF Service Producer cannot authenticate the NF Service Consumer unless the NRF in the target PLMN and the NF Service Producer is provisioned with required certificates to authenticate the NF Service Consumer.

*** END CHANGES ***