3GPP TSG-SA3 Meeting #122	S3-251846

Fukuoka, Japan, 19 - 23 May 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.529 | CR | 0002 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Minor editorial corrections and clean-up. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Indian Institute of Technology Bombay |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G_Maint |  |  |  |  |  | Date: |  |  |  |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The threat references have been updated with correct clauses from the threats specific to SMSF as described in TR 33.926. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated TBA fields with correct references and minor corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Without correct threat references, understanding the testcases would be challenging. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************** Start of 1st Change ****************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 33.117: "Catalogue of general security assurance requirements".

[3]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[4]	3GPP TR 33.926: "Security Assurance Specification (SCAS) threats and critical assets in 3GPP network product classes".

[5]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)".

[6]	3GPP TS 23.501: "System Architecture for the 5G System (5GS)".

[7]	3GPP TS 29.540: "5G System; SMS Services".

[8]	3GPP TS 29.338: "Diameter based protocols to support Short Message Service (SMS) capable Mobile Management Entities (MMEs)".

[9]	3GPP TS 29.002: "Mobile Application Part (MAP) specification".

[10]	3GPP TS 33.210: "Network Domain Security (NDS): IP network layer security".

[11]	3GPP TS 33.310: "Network Domain Security (NDS): Authentication Framework".

[12]	IETF RFC 6733: "Diameter Base Protocol".

[13]	3GPP TS 33.204: "3G Security; Network Domain Security (NDS): Transaction Capabilities Application Part (TCAP) user security".

*************** End of 1st Change ****************

*************** Start of 2nd Change ****************

The clause 4.2.6 of TS 33.117 [2] applies to SMSF, except for the clauses 4.2.6.2.3 and 4.2.6.2.4, which are not applicable. There are no SMSF-specific additions to clause 4.2.6 of TS 33.117 [2].

*************** End of 2nd Change ****************

*************** Start of 3rd Change ****************

#### 4.2.7.2	Protection of Diameter Session on SGd Interface

Requirement Name: Diameter session on SGd interface

Requirement Reference: TS 29.338 [8], clause 4.5; RFC 6733 [12], clause 8.8

Requirement Description:

SMSF supports implicit termination of SGd Diameter application sessions. The client (server) includes in its requests (responses) the Auth-Session-State AVP set to the value NO_STATE_MAINTAINED (1), as described in IETF RFC 6733 [12]. The server sets the Auth-Session-State AVP value to NO_STATE_MAINTAINED (1), irrespective of what value the client sets. Neither the Authorization-Lifetime AVP nor the Session-Timeout AVP be present in requests or responses [8].

To protect Diameter sessions, SMSF supports the following requirements:

1.	Diameter session ID AVP be unique, i.e. uniquely identify the user session and distinguish the session from all other active sessions.

2. The session ID be generated by the Diameter application node that initiates the session.

## Threat References: TR 33.926 [4], Annex Y.2.2.2, Diameter session information disclosure

*************** End of 3rd Change ****************

*************** Start of 4th Change ****************

#### 4.2.7.3	Protecting availability and integrity on Diameter-based SGd interface

Requirement Name: Diameter filtering on the SGd interface

Requirement Reference: TS 29.338 [8], clause 6.3.2.2

Requirement Description:

TS 29.338 [8] defines the following commands and their command codes for the SGd application: MO-Forward-Short-Message-Request (OFR) - 8388645, MO-Forward-Short-Message-Answer (OFA) - 8388645, MT-Forward-Short-Message-Request (TFR) - 8388646, MT-Forward-Short-Message-Answer (TFA) - 8388646, Alert-Service-Centre-Request (ALR) - 8388648 and Alert-Service-Centre-Answer (ALA) - 8388648. It also mentions that the Application ID field for OFR, OFA, TFR, TFA commands allocated by IANA is 16777313 and that for ALR, ALA commands is 16777312. 

SMSF provides a mechanism, or rely on the other network function, to filter incoming Diameter messages on the SGd interface based on the Application IDs and command codes.

In particular, SMSF that supports filtering provides a mechanism:

1)	To filter incoming Diameter messages on the SGd interface at the Application layer of the stack ISO/OSI.

2)	To allow specified actions to be taken when a filter rule matches. In particular at least the following actions should be supported:

-	Discard: the matching message is discarded; no subsequent rules are applied and no answer is sent back.

-	Accept: the matching message is accepted.

-	Account: the matching message is accounted for i.e. a counter for the rule is incremented. This action can be combined with the previous ones. This feature is useful to monitor traffic before its blocking.

3)	To enable/disable the logging for each rule for troubleshooting.

4)	To filter on the basis of the value(s) of any portion of the protocol header.

5)	To reset the accounting.

6)	To disable/enable each defined rule.

Threat References: TR 33.926 [4], Annex Y.2.2.1, Diameter filtering

*************** End of 4th Change ****************

*************** Start of 5th Change ****************

#### 4.2.7.4	Protecting from unknown peers on Diameter-based SGd interface

Requirement Name: Disabling peer discovery on the SGd interface

Requirement Reference: RFC 6733 [12], clause 5.2;

Requirement Description:

Capability Exchange Request (CER) and Capability Exchange Answer (CEA) are Diameter messages used on SGd interface to discover diameter nodes and know their capability. If peer discovery is enabled, an attacker can connect to the SMSF and learn about its capabilities by sending CER messages. Network products with the SGd interface should be configured to accept only known peers.

To protect SMSF from attacks by unknown peers (attackers), it supports the following requirements:

1)	It maintains list of all trusted nodes with static IP addresses in a peer table.

2)	It is configured only to respond to the CER messages from trusted nodes listed in the peer table, and it ignores CERs from all other nodes.

Threat References:  TR 33.926 [4], Annex Y.2.2.1, Diameter filtering

*************** End of 5th Change ****************

*************** Start of 6th Change ****************

#### 4.2.7.5	Protecting availability and integrity on Map-based SS7 interface

Requirement Name: Map filtering on the SS7 interface

Requirement Reference: TS 33.204 [13], clause 4.1; TS 29.002 [9], clause 12

Requirement Description:

TS 33.204 [13], clause 4.1 mentions "TCAPsec does not validate the TCAP user payload content. Message screening functions for particular message types may be needed on top of TCAPsec." TS 29.002 [9] defines the Short Message Service (SMS) management services in clause 12. 

Hence, message filtering mechanism shall be used on Map-based SS7 interface of SMSF to filter and allow only SMS management services related messages which are applicable for SMSF, for example, MAP-MT-FORWARD-SHORT-MESSAGE service, MAP-MO-FORWARD-SHORT-MESSAGE service, and MAP-ALERT-SERVICE-CENTRE service.

In particular, SMSF that supports filtering provides a mechanism:

1)	To filter incoming Map messages on the SS7 interface at the Application layer of the stack ISO/OSI.

2)	To allow specified actions to be taken when a filter rule matches. In particular at least the following actions should be supported:

-	Discard: the matching message is discarded; no subsequent rules are applied and no answer is sent back.

-	Accept: the matching message is accepted.

Threat References: TR 33.926 [4], Annex Y.2.3.1, Map filtering

*************** End of 6th Change ****************