3GPP TSG-SA3 Meeting #122	S3-251886

Fukuoka, Japan, 19 – 23 May 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.122 | CR | draft | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Onboarding API Invoker Residing in the UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3_Sec |  |  |  |  |  | Date: |  |  | 2024-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As per TS 23.222 Clause 8.1.1, ‘The CAPIF enables one time onboarding process that enrolls an API invoker as recognized user of the CAPIF, which is triggered by the API invoker via CAPIF-1 or CAPIF-1e, based on provisioning’. Further TS 23.122 Annex F clarifies, the API invoker can be Application servers or BSS/OSS system which invokes CAPIF APIs as per its business logic/business relationship. TS 23.122 Annex G confirms that an API Invoker can either be an application on a server or an application in the UE. Existing Onboarding procedure specifed in TS 33.122 Clause 6.1 depends on the pecondition that there exists an Onboarding enrollment information. But allows only server side authentication and further relies upon the Onboarding credential (e.g., message digest /Oauth 2.0 token) to verify and if succesful provides the API Invoker profile. There is no standard means to verify the identifier associated to the API Invoker. Additionally when the API invoker resides as part of the UE, the only business relationship that exists between the UE and the Operator network is the subscription and there is no check that happens during the Onboarding to verify if the API invoker really belongs to a UE or not leading to any remote entity familiar with CCF address and root certificate can further share with ‘n’ number of malicious entities who can easily generate pubic private key pairs and gets successfully onboarded with CCF with mere message digest verification leading to targeted attacks over the CCF. Threats listed in TR 33.700-22 Clause 5.6 is also applicable and similar issues described in TS 23.222 Annex G is applicable while Onboarding the API invoker of the UE. Further RFC 6749 Clause 10 Security Consideration, for client authentication states that, ‘When client authentication is not possible, the authorization server SHOULD employ other means to validate the client's identity’. |  |  |  |  |  |  |  |  |
| Summary of change: |  | CCF verifies if the API invoker is part of the UE before onboarding the API invoker of the UE to the CAPIF, so that CAPIF can consider the onboarded API invoker as the recognozed user of the CAPIF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Any API invoker without being part of a UE will onboard to the CAPIF and can falsely claim as the API invoker residing in the UE causing the theats |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*****Start of Change 1*****

## 6.1	Security procedures for API invoker onboarding

The API invoker and the CAPIF core function shall follow the procedure in this subclause to secure and authenticate the onboarding of the API invoker to the CAPIF core function. The API invoker and the CAPIF core function shall establish a secure session using TLS. Security profiles for TLS implementation and usage shall follow the provisions given in TS 33.310 [2], Annex E .

With a secure session established, the API Invoker sends an Onboard API Invoker Request message to the CAPIF core function. The Onboard API Invoker Request message carries an onboard credential obtained during pre-provisioning of the onboard enrolment information, which may be an OAuth 2.0 [4] access token. When the OAuth 2.0 token based mechanism is used as the onboarding credential, the access token shall be encoded as JSON web token as specified in IETF RFC 7519 [6], shall include the JSON web signature as specified in IETF RFC 7515 [7], and shall be validated per OAuth 2.0 [4], IETF RFC 7519 [6] and IETF RFC 7515 [7]. Other credentials may also be used (e.g. message digest).

Figure 6.1-1 details the security information flow for the API invoker onboarding procedure. The OAuth 2.0 token based authentication credential is shown in this example.

Figure 6.1-1: Security procedure for API invoker onboarding

1.	As a prerequisite to the onboarding procedure, the API invoker obtains onboarding enrolment information from the API provider domain. The onboarding enrolment information is used to authenticate and establish a secure TLS communication with the CAPIF core function during the onboarding process. The enrolment information includes details of the CAPIF core function (Address, and Root CA certificate) and includes an onboarding credential (the OAuth 2.0 [4] access token).

NOTE 1a: For the API Invoker residing as part of the UE, if the CCF does not know to whom the API Invoker belongs to, the CAPIF service provider or operator will not have sufficient information to determine who is accountable for the services being offered or when CAPIF services are abused, there will be no accountability.

If the API invoker resides as part of the UE, the onboarding credential is bound to the UE ID, i.e, the GPSI.

NOTE 1b: Each API invoker on UE is onboarded separately.

NOTE 1:	The procedure used to obtain the enrolment information by the API invoker is out of scope of the present document.

2.	The API invoker and CAPIF core function shall establish a secure session based on TLS (Server side certificate authentication). The API invoker shall use the enrolment information obtained in step 1 to establish the TLS session with the CAPIF core function.

3.	After successful establishment of the TLS session, the API invoker shall send an Onboard API invoker request message to the CAPIF core function along with the enrolment credential (OAuth 2.0 [4] access token). The API invoker generates the key pair {Private Key, Public key} and provides the public key along with the Onboard API invoker request.

If the API invoker resides as part of the UE, the Onboarding API invoker request additionally includes the UE ID i.e, GPSI, where the Onboarding credential includes either an OAuth 2.0 access token or message digest to be used as an authentication code to verify if the API invoker is part of the UE.

4.	The CAPIF core function shall validate the enrolment credential (OAuth 2.0 [4] access token). If validation of the credential (the OAuth 2.0 [4] access token in this example) is successful, the CAPIF core function shall generate an API invoker's profile as specified in TS 23.222 [3] which may contain the selected method for AEF authentication and authorization between the API Invoker and the AEF (see subclause 6.5.2). The CAPIF core function may generate API invoker's certificate on its own, for the assigned API invoker identity and public key. This certificate shall be used by the API invoker for subsequent authentication procedures with the CAPIF core function and may be used for establishing a secure connection and authentication with the API Exposing Function. The CAPIF core function may optionally generate an Onboard_Secret if the subscribed Service API uses Method 3 (as specified in clause 6.5.2.3 of the present document) for CAPIF-2e security. The Onboard_Secret value remains the same during the lifetime of the onboarding, and shall be bound to the CAPIF core function specific API Invoker ID.

NOTE 2:	When API invoker's client certificate is issued by the third party, then in Step 3 the API invoker can additionally include the certificate in Onboard API Invoker request message. If the CAPIF core function trusts the issuer of the API invoker's client certificate, then the CAPIF Core Function includes the provided certificate in the API invoker's profile, in step 4. It is up to the CAPIF domain policy to accept the client certificates issued by third party.

5.	The CAPIF core function shall respond with an Onboard API invoker response message. The response shall include the CAPIF core function assigned API invoker ID, AEF Authentication and authorization information (if generated in step 4), API invoker's certificate and the API invoker Onboard_Secret (if generated by the CAPIF core function).

*****End of Change 1*****