3GPP TSG-SA3 Meeting #122																			S3-251907

Fukuoka, Japan, 19 – 23 May 2025

# Source:	KPN

# Title:	AIoT authentication procedure based on stored nonces

# Document for:	Approval

Agenda Item:	4.26

# 1	Decision/action requested

It is requested to approve the following pCR.

# 2	References

[1]	3GPP TS 33.369, version 0.1.0, Security aspects of Ambient IoT service

[2]		3GPP TS 23.369, version 0.3.0, Architecture support for Ambient power-enabled Internet of Things;Stage 2

[3]	3GPP TS 38.300, NR; NR and NG-RAN Overall Description; Stage 2

[4]	3GPP TS 38.391, Ambient IoT Medium Access Control (MAC) Protocol specification

[5]	R2-2504398, draftCR for 38.300 v18.5.0, 38.300 running CR for Ambient IoT

[6]	R2-2503997, draft TS, TS 38.391 v0.0.2, Ambient IoT Medium Access Control Protocol specification

# 3	Rationale

This pCR contains an authentication procedure for AIoT devices that adheres to the baselines of SA2 as specified in 3GPP TS 23.369 [2] and to the baseline and agreements of RAN2 as specified in 3GPP TS 38.300 [3] updated with the endorsed draftCR R2-2504398 [5] and 3GPP TS 38.391 [4] as captured in the draft TS R2-2503997 [6].

The authentication procedure in this proposal assumes that the paging message from NG-RAN to AIoT device does not contain IEs visible to the upper layers other than Paging ID. It does not assume that nonces/randoms or other IEs visible to the upper layers are included in the paging message.

The authentication procedure in this proposal is applicable in case no privacy protection of the AIoT Device ID is necessary.

The procedure in this proposal is also suitable to send data from AIoT device to NG-RAN in protected form. Hence it is also suitable as procedure in clause 5.3 of 3GPP TS 33.369 [1].

# 4	Detailed proposal

It is proposed to make the following changes to 3GPP TS 33.369, version 0.1.0 [1]

FIRST CHANGE

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]		3GPP TS 23.369: "Architecture support for Ambient power-enabled Internet of Things".

[3]	3GPP TS 38.300: "NR; NR and NG-RAN Overall description; Stage-2".

[4]	3GPP TS 22.369: "Service requirements for Ambient power-enabled IoT".

[5]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[x]	3GPP TS 38.391: "Ambient IoT Medium Access Control Protocol specification".

NEXT CHANGE

## 5.2	Authentication procedure

Editor’s Note: This clause contains the security procedures on the authentication.

For the case where the paging message only contains a Paging ID (or no Paging ID at all), and no privacy is needed for the AIoT Device ID, the authentication procedure is as described below.

The solution makes use of nonces and associated nonce-IDs. The nonce is used to create unique encryption keys which are used to encrypt the device ID of an AIoT device, and the encrypted device ID is used to authentication a message. This prevents that an attacker can forge a valid message. By marking used nonces as used, replay attacks are prevented.

The use of nonce-IDs in addition to nonces facilitates the lookup of nonces in the network and thus eases the computational burden on the network, especially when the number of stored nonces has grown during time.

Figure 5.2-1: Authentication procedure based on stored nonces

0,	The AIoT device is provisioned with a Device ID, shared symmetric key KAIoT, an initial set of nonces and a set of associated nonce-IDs. For each device, the ADM stores its Device ID and provisions it with the associated key KAIoT, the initial set of nonces and the associated set of nonce-IDs. The initial set of nonces and associated nonce-IDs are the same for the device and for the ADM. 

The provisioning of the shared key, the initial set of nonces, and the initial set of nonce-IDs is performed in an environment where the AIoT device has sufficient energy.

1.	AF to NEF: Nnef_AIoT_Inventory Request(AF ID, [External Target Area information], information about the target AIoT Device(s), [Approximate number of AIoT Devices]).

The meaning of the parameters of this message are specified in 3GPP TS 23.369 [2], clause 6.2.2, step 1.

2.	The NEF selects one or multiple AIOTF(s) to handle the request as specified in 3GPP TS 23.369 [2], clause 6.2.2, step 2.

3.	NEF to AIOTF: Naiotf_AIoT_Inventory Request (AF ID, [External Target Area information], [information about the target AIoT Device(s)], [Approximate number of AIoT Devices]) as specified in 3GPP TS 23.369 [2], clause 6.2.2, step 3.

4.	After checking the parameters receivd in the request, the AIOTF generates the Inventory Request as specified in 3GPP TS 23.369 [2], clause 6.2.2, step 4.

5.	AIOTF to NEF: Naiotf_AIoT_Inventory Response as specified in 3GPP TS 23.369 [2], clause 6.2.2, step 5.

6.	NEF to AF: Nnef_AIoT_Inventory Response as specified in 3GPP TS 23.369 [2], clause 6.2.2, step 6.

7.	AIOTF to NG-RAN: Inventory Request as specified in 3GPP TS 23.369 [2], clause 6.2.2, step 7.

8.	NG-RAN to AIOTF: Inventory Response as specified in 3GPP TS 23.369 [2], clause 6.2.2, step 8.

9.	NG-RAN to AIoT Device(s): Paging message as specified in 3GPP TS 38.391 [x].

10.	The AIoT Device prepares a D2R message by performing the following steps:

10a.	The AIoT Device selects the next nonce from the set of stored nonces (Nonce1, with associated Nonce1_ID) and derives a symmetric key Kd using KAIoT and Nonce1 as inputs. If the set of stored nonces is empty, the AIoT device prepares and stores the next set of nonces with associated nonce_IDs as described in step 18.

10b.	The AIoT device marks Nonce1 as used and removes it from the set of stored nonces.

10.c	The AIoT device encrypts data that needs to be sent to the AF along with Device ID using Kd as the encryption key or compute a keyed hash of the Device ID using Kd, resulting in Enc_Kd(data) and Enc_Kd(Device ID), respectively.

10d.	The AIoT device generates a hash of data, resulting in Hash(data).

11.	AIoT Device to NG-RAN: D2R message as specified in 3GPP TS 38.391 [x] containing a Data SDU IE.

Data SDU contains the AIOT NAS message containing Device ID, Enc_Kd(Device ID), Nonce1, Nonce1_ID, and optionally Enc_Kd(data) and Hash(data).

NOTE 1:	Hash(data) is present in the message if Enc_Kd(data) is present in the message.

12.	NG-RAN to AIOTF: Inventory Report as specified in 3GPP TS 23.369 [2], clause 6.2.2, step 10.

The AIOT NAS message in the Inventory Report contains Device ID, Enc_Kd(Device ID), Nonce1, Nonce1_ID, and optionally Enc_Kd(data) and Hash(data).

13.	AIOTF to ADM: Nadm_AIoT_Authenticate Request(Device ID, Enc_Kd(Device ID), Nonce1, Nonce1_ID, [Enc_Kd(data)], [Hash(data)]).

14.	The ADM verifies the received request and (if valid) decrypts the included data by performing the following steps:

14a.	The ADM looks up the nonce associated with the received Nonce1_ID in its set of stored nonces.

-	If found and matching with the received Nonce1, the ADM verifies whether the nonce is marked as used.

-	If the nonce is marked as used, the received message is discarded (as being a replay), Authentication_Result is set to Failed, and the rest of the steps in 14 are skipped.

-	If the nonce is not marked as used, continue with the rest of the steps in 14.

-	If found and not matching, the received message is discarded (as being invalid), Authentication_Result is set to Failed, and the rest of the steps in 14 are skipped.

-	If not found, continue with the rest of the steps in 14.

14b.	The ADM obtains KAIoT based on received Device ID.

14c.	The ADM derives Kd using KAIoT and Nonce1 as inputs.

NOTE 2:	The algorithm and inputs used by the ADM to derive Kd are the same as the one used by the AIoT device to derive Kd. Hence the two keys are identical.

14d.	The ADM decrypts Enc_Kd(Device ID), using Kd, and checks if the decrypted Device ID matches the unencrypted Device ID that was received.

-If the decrypted Device ID matches the received unencrypted Device ID, the AIoT device is considered to be authenticated, and the received message is considered to be valid. Authentication_Result is set to Successful.

-If the decrypted Device ID doesn’t match the received unencrypted Device ID, the AIoT device is considered to be not authenticated, and the received message is discarded (as being invalid). Authentication_Result is set to Failed, and the rest of the steps in 14 are skipped.

14e.	If the received nonce has not been found in the set of stored nonces, the ADM prepares the next set of nonces with associated nonce_IDs as described in step 19 and stores them along with the Device ID.

14f.	The ADM marks Nonce1 as used.

14g.	The ADM decrypts Enc_Kd(data), using Kd, resulting in (unencrypted) data.

NOTE 3:	Step 14g only occurs if Enc_Kd(data) is present in the message received from the AIoT device.

14h.	The ADM computes Hash(data) and checks if it matches the Hash(data) received in the message. If the hashes do not match, the received data is discarded.

NOTE 4:	Step 14h only occurs if Hash(data) and hence Enc_Kd(data) is present in the message received from the AIoT device.

15.	ADM to AIOTF: Nadm_AIoT_Authenticate Response(Authentication_Result, [data]).

Authentication_Result has obtained a value of Failed or Successful in step 14.

- if Authentication_Result  = Failed and no aggregation is performed, steps 16 and 17 are skipped.

- if Authentication_Result = Failed and aggregation is performed, the result from the message received from the AIoT devices is not included in the aggregated message to the NEF.

- if Authentication_Result = Successful, steps 16 and 17 are performed; if aggregation is performed, the result from the message received from the AIoT device is included in the aggregated message to the NEF.

NOTE 5:	data is only included in the message if Enc_Kd(data) was present in the message received from the AIoT device and if data was not discarded in step 14h.

16.	AIOTF to NEF: Naiotf_AIoT_Inventory Notify as specified in 3GPP TS 23.369 [2], clause 6.2.2, step 13.

The message includes data if present in the message received from the ADM in step 15.

17.	NEF to AF: Nnef_AIoT_Inventory Notify as specified in 3GPP TS 23.369 [2], clause 6.2.2, step 14.

The message includes data if present in the message received from the AIOTF in step 16.

If required, the AIoT device and/or the ADM can prepare a next set of nonces based on the existing set of nonces. The generation of the next set of nonces produces the same nonces in AIoT Device and in the ADM.

18.	The AIoT Device may prepare a next set of nonces provided it has enough energy for this.

NOTE 6:	The preparation of a next set of nonces can occur at the beginning of a future inventory procedure.

19.	The ADM may prepare a next set of nonces.

NOTE 7:	The preparation of a next set of nonces can occur at the beginning of a future inventory procedure.

END OF CHANGES