3GPP TSG-SA3 Meeting #122	S3-251910

Fukuoka, Japan, 19 – 23 May 2025

# Source:	CableLabs

# Title:	New WID on PRINS Refinement

# Document for:	Approval

# Agenda Item:	6.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: New WID on PRINS Refinement

Acronym: RefinePRINS

Unique identifier:

{A number to be provided by MCC at the plenary}

Potential target Release:	Rel-20 (5GA)

# 1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  |  |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
| X | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A |  |  |  |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| N/A |  |  |

# 3	Justification

GSMA is preparing an LS to 3GPP SA3 on further refinement of PRINS to make it easier for roaming intermediaries (RIs) to support PRINS and its end-to-end application layer security property. The following aspects of PRINS are under discussion for further refinement:

1. The requirement for RIs to support HTTP CONNECT to establish end-to-end N32-c between vSEPP and hSEPP can introduce security risks to RIs, since unauthorized parties may use HTTP CONNECT to establish encrypted TLS tunnels through RIs (e.g., to transmit unauthorized signalling traffic or malicious traffic). Currently, RIs use the information elements (e.g., p-seep-fqdn, allowed-n32-purpose) in the 3gpp-Connect-Req-Infor header to decide on whether to accept an HTTP CONNECT request. However, those information elements are in plain text and can be easily forged. IP address-based access control is helpful but also adds management overhead. Integrity protection of HTTP CONNECT requests/responses may be needed to mitigate security risks for RIs.

2. N32-c procedures are currently end-to-end protected between vSEPP and hSEPP with both confidentiality and integrity when RIs are present. However, RIs may require visibility into some N32-c handshaking procedures to be able to both support PRINS and to ensure adherence to contractual agreements. For example, an RI needs to see the modification policy that is being exchanged between vSEPP and hSEPP in order to ensure that it complies with the business contract the RIs have with the two PLMNs.  Knowing the modification policy exchanged between vSEPP and hSEPP can also improve RI’s confidence that its modification of N32-f information elements using PRINS will not be rejected by the vSEPP and/or the hSEPP.

3. The current PRINS specifications consider an RI as a single entity with mixed business logic and security capability. In practice, an RI may deploy an HTTP proxy for security capabilities and some application functions for message modifications. To better support PRINS, the behaviours of RI proxy need to be clearly specified, taking into consideration that modifications to N32-f messages may be performed by an RI application function outside of RI proxy. It can be assumed that both RI proxy and RI application are under the administration of the same RI.

4. Currently, 3GPP TS 33.501 (clause 13.2.1 of TS 33.501) specifies two options for N32-f transport layer protection: NDS/IP and TLS VPN. Note TLS VPN is a TLS tunnel, similar to IPsec tunnel, which often carries IP packets that can be further routed after exiting the tunnel. Thus, TLS VPN differs from HTTP over TLS (HTTPS) in that the latter is limited to carrying HTTP payload inside the TLS tunnel. In practice, HTTPS is often used and is implicitly used in GSMA guidelines (e.g., NG.113) for 5G roaming. Thus, it may be beneficial to explicitly specify HTTPS as an option for N32-f transport layer protection.

# 4	Objective

Based on the above justifications, the following objectives are anticipated:

1. Protecting HTTP CONNECT requests and response by adding integrity protection of 3gpp-Connect-Req-info and 3gpp-Connect-Resp-Info headers. This allows an RI to cryptographically authenticate HTTP CONNECT requests and responses.

2. Allowing some parameters exchanged in N32-c procedures (e.g., protection policy) that are relevant to RIs to be sent over N32-f. Note that clause 13.2.2.3 of TS 33.501 already allows error messages to be sent over N32-f if they are relevant to RIs.

3. Separating RI into RI proxy and RI application, where RI application performs business logic by modifying HTTP messages and RI proxy performs security functions (e.g., generating and digitally signing JSON patches).

4. Specifying HTTPS as an option for protecting the transport of N32-f.

Note: the anticipated objectives above will be adjusted according to GSMA input.

## TU estimates

Total TU estimates: 2 TUs (4 meeting cycles)

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 33.501 | Security architecture and procedures for 5G system | TSG#111 (Mar 2026) |  |

# 6	Work item Rapporteur(s)

# 7	Work item leadership

SA3

# 8	Aspects that involve other WGs

None

# 9	Supporting Individual Members

{At least 4 supporting Individual Members are needed. There is an expectation that these companies will provide resources to progress the work. Note that having 4 supporting companies is a necessary but not sufficient condition: the usual TSG approval process by consensus is needed for the WID approval}

Supporting IM name

CableLabs

China Telecom