3GPP TSG-SA3 Meeting #122	S3-251913

# Fukuoka, Japan, 19 – 23 May 2025

Source:	Vodafone

Title:	Pseudo CR on Avatar Identification and mapping within BAR

Document for:	Approval

Agenda item:	5.2

Spec:	TR 33.790

Version:	1.3.0

Work Item:	FS_NG_RTC_SEC_Ph2

Comments

This document outlines a solution for identifying and mapping avatars to terminal or user identifiers within the Base Avatar Repository (BAR).

* * * First Change * * * *

# 6	Solutions

## 6.0	Mapping between key issues and solutions

Table 6.0-1: Mapping of solutions to key issues

| Solutions | KI#1 | KI#2 | KI#3 |
| --- | --- | --- | --- |
| Solution #1: Signing and verification of third party ID information | X |  |  |
| Solution #2: Security of 3rd party specific identities | X |  |  |
| Solution #3: Support of Third Party specific User Identities in IMS using STIR/SHAKEN | X |  |  |
| Solution #4: SHAKEN based third-party specific user identities | X |  |  |
| Solution #5: Securing the IMS based avatar communication |  | X |  |
| Solution #6: Solution for secure IMS based avatar communication |  | X |  |
| Solution #7: Protect IMS DC based Avatar Communication |  | X |  |
| Solution #8: Security for IMS based Avatar Communication |  | X |  |
| Solution #9: Secure IMS DC capability exposure |  |  | X |
| Solution #10: User aware IMS DC capability exposure |  |  | X |
| Solution #11: IMS (DC) capability exposure security based on existing specification |  |  | X |
| Solution #12: Solution for secure IMS based avatar communication using STIR/SHAKEN |  | X |  |
| Solution #13: IMS avatar communication security based on existing specification |  | X |  |
| Solution #14: Identification of Avatar and Mapping to terminal/user identities |  | X |  |

* * * Next Change * * * *

## 6.14	Solution #14: Identification of Avatar and mapping to terminal/user identities

### 6.14.1	Introduction

In Release 19, avatar communication is supported through the data channel architecture. This can be over IMS Data channel, with mapping to IMS and Non-IMS based services.

The creation and rendering of an Avatar Representative can occur within the UE or the network, either partially or completely, depending on the capabilities of the UE and the network. When using network-based rendering, the DC AS sends the Avatar communication service control instruction to the MF. The MF supports media transcoding, including different media formats (e.g. ASR, TTS, SLR, and SSL).

SA2 (23700-77 NG-RTC phase2) introduced a new Base Avatar Repository (BAR) which stores Avatar representation and related avatar ID to be used in identification and authentication. Avatar IDs would be assigned to registered Avatars by BAR and mapped to SUPI or GPSI, Avatar ID should be stored for Avatar representative (UE) within UDM/UDR and DC AS as part of UE subscription data.

When a user logs in to create an Avatar, authentication occurs using the Avatar application identification and SUPI/GPSI/PEI in the Avatar creation application or client towards the network.

NOTE 4: User application is an AF to the network and authentication would take place as 3rd party authentication to the NEF, using e.g. TLS authentication.

NOTE 5: This application may belong to an MNO network, a third party, or an AR/XR supporting device application, which must support authentication and registration with the BAR/UDM/UDR repository.

The Avatar representation can be animated using audio/video or metadata (such as action and facial expression data) by UE or network. The metadata can be generated by UE or the network.

### 6.14.2	Solution details

When generated avatar is being submitted to network, during authentication with BAR/UDM/UDR, BAR assigns Avatar ID to the requested avatar and stores it in the Avatar profile which is associated to the identity of the subscriber or of the terminal. Upon completing the storage phase, BAR updates subscriber info in UDM/UDR and IMS AS.

Figure 2. Identification of Avatar

- The Avatar Registration request is sent from UE and application repository to the IMS AS

- The IMS AS sends the User Info request to DCSF, which queries the subscription info from UDM/UDR. The response includes UE’s SUPI/GPSI/PEI and avatar ID list.

- The DCSF, after receiving information from UDR/UDM, sends the Avatar Registration request to BAR through DC AS, containing the UE subscription information and Avatar ID list to get a new ID from BAR.

- BAR checks the UE profile for the Avatar ID list, generates the new ID, and sends it back to DCSF.

- DCSF provides the Avatar ID in response to the IMS AS registration request.

- The IMS AS updates the Avatar ID list in UDR/UDM and XR (DC/AR) AS.

The communication towards XR (DC/AR) AS is executed via DC1 and DC4 interfaces. DC4 shall contain GPSI information to identify the UE. IMS AS confirms the ID to BAR and sends the new Avatar ID to the UE. The UE stores the new avatar in BAR with the corresponding Avatar ID.

### 6.14.3	Evaluation

This solution addresses the Key Issue #2 by preventing malicious users from impersonating others through Avatar representations. It also ensures that Avatar objects are secured and cannot be tampered with, by establishing a unique mapping between Avatar ID and terminal/user identities.

* * * End of Changes * * * *